3GPP TSG-SA WG6 Meeting #59	S6-240168

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

Source:	China Mobile

Title:	pCR on eMMTel high level architecture aligned with GSMA NG.134

Spec:	3GPP TR 23.700-92 v0.3.0

Agenda item:	8.2

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

Some architectures related to eMMTel service have been specified in both 3GPP and GSMA, including architecture specified in 3GPP TS 23.228, GSMA TGY.02, GSMA NG.134 and GSMA TS.66. All these architectures have not covered the enabler layer of eMMTel service. When the architecture of eMMTel Enabler is proposed, it is needed to be aligned with the existing architectures. This pCR proposes solution of eMMTel high level architecture aligned with the GSMA NG.134. This solution is a subset of a big solution of architecture alignment which will be described in clause 8.X.

2. Reason for Change

Contribution with new text.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.3.0.

* * * First Change * * * *

8.X.y	eMMTel Enabler architecture aligned with GSMA NG.134

A profile for IMS data channel services over IMS is specified in GSMA NG.134 [X1]. A use case listed in GSMA NG.134 [X1] is that a IMS data channel application service logic and media content MAY be either provided by the Data Channel Server inside the operator’s administrative domain, or by an External Server which is deployed outside the operator’s administrative domain. This use case is list in Annex C and Annex D of GSMA NG.134 [X1] for information. When the GSMA NG.134 [X1] is released, the Rel-18 work on NG_RTC has not been finished, the architecture of Data Channel Capable PLMN in GSMA NG.134 [X1] is not detailed profiled. Since GSMA NG.134 [X1] will only define profiles based on 3GPP specifications but will not develop technical solutions, the architecture specified in Annex AC of 3GPP TS 23.228 [3] can be considered as the IMS Data Channel Capable PLMN in GSMA NG.134 [X1].

There are two Communication Model for Accessing External Server listed in Annex D of GSMA NG.134 [X1]. The Model A is HTTP Server without IETF RFC 8831 Data Channel Support and shown in Figure C.1-1 of GSMA NG.134 [X1]. This figure is shown in Figure 8.x.y-1 of this clause.

Figure 8.x.y-1: Figure C.1-1 of GSMA NG.134 [X1]: Model A – IMS data channel access to external HTTP Server

The Model B is HTTP Server with IETF RFC 8831 Data Channel Support and shown in Figure C.2-1 of GSMA NG.134 [X1]. This figure is shown in Figure 8.x.y-2 of this clause.

Figure 8.x.y-2: Figure C.2-1: Model B, IMS data channel access to external HTTP Server with Data Channel Support

In the Annex C and Annex D of GSMA NG.134 [X1], external content access within SIP session and communication Model for Accessing External Server are described, but how to provide service enabler layer capabilities exposure of eMMTel services.

When Mode A is used, the architecture when the eMMTel Enabler is introduced is listed in figure 8.x.y-3.

Figure 8.x.y-3: high level architecture of eMMTel Enabler aligned with Figure C.1-1 of GSMA NG.134

When Mode B is used, the architecture when the eMMTel Enabler is introduced is listed in figure 8.x.y-4.

Figure 8.x.y-4: high level architecture of eMMTel Enabler aligned with Figure C.2-1 of GSMA NG.134

When eMMTel Enabler is introduced, the two different models listed in GSMA NG.134 [X1] may not impact the architecture of eMMTel Enabler. Therefore the high level architecture of eMMTel Enabler aligned with GSMA NG.134 [X1] can be simplified and is shown in figure 8.x.y-5.

Figure 8.x.y-5: high level architecture of eMMTel Enabler aligned with GSMA NG.134

In the figure Figure 8.x.y-5, the north-bound APIs provided by the IMS Data Channel Capable PLMN, including:

a)	the APIs provided by DCSF via DC3/N33 reference point;

b)	the APIs provided by DCSF via DC4 reference point;

c)	the APIs provided by MF/MRF via MDC3/MDC4 reference points; and

d)	the capabilities provided by the DC Application Server via eMMTel-3 reference point,

are discovered by the SCEF+NEF node via the eMMTel Enabler Layer which acts as the CAPIF function specified in TS 23.222 [9]. The DCSF, MF/MRF and DC Application Server act as CAPIF's API exposing function as specified in 3GPP TS 23.222 [9].

The External Server which provides IMS data channel application service logic and media content may also provide capabilities, e.g. AR Communication specified in Annex AC.9 of 3GPP TS 23.228 [3], to the eMMTel Capabilities users. The capabilities provided by the External Server are also discovered by the SCEF+NEF  node via the eMMTel Enabler Layer which acts as the CAPIF function specified in TS 23.222 [9]. The External Server acts as CAPIF's API exposing function as specified in 3GPP TS 23.222 [9].

To ensure the security access of the IMS Data Channel Capable PLMN, the CAPIF-1(e) and CAPIF-2(e) reference points between the eMMTel Capabilities users and the IMS Data Channel Capable PLMN are all provided by the eMMTel Enabler via eMMTel-2 reference point. The DC3/N33, DC4, MDC3/MDC4 and eMMTel-3 reference points act as both CAPIF-2(e) and CAPIF-3(e) reference points as specified in 3GPP TS 23.222 [9].

The External Server may also communicate with the IMS Data Channel Capable PLMN via the eMMTel Enabler Layer by communicating with the eMMTel-2 reference point.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[4]	OMA-AD-NGSI-V1_0-20120529-A: "Next Generation Service Interfaces Architecture".

[5]	OMA-TS-REST_NetAPI_ThirdPartyCall-V1_0-20130212-C: "RESTful Network API for Third Party Call".

[6]	OMA-TS-REST_NetAPI_CallNotification-V1_0-20200226-C: "RESTful Network API for Call Notification".

[7]	OMA-TS-REST_NetAPI_AudioCall-V1_0-20200226-C: "RESTful Network API for Audio Call".

[8]	3GPP TS 23.198: "Open Service Access (OSA)".

[9]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[10]	3GPP TR 22.873: "Study on evolution of the IP Multimedia Subsystem (IMS) multimedia telephony service".

[11]	GSMA NG.129: "IMS Data Channel White Paper v0.1".

[12]	GSMA TGY.02: "Business Voice Calling".

[13]	3GPP TR 23.700-87: "Study on system architecture enhancement for next generation real time communication".

[14]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction"

[X1]	GSMA NG.134: "IMS Data Channel, Version 1.0".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".