3GPP TSG-SA WG6 Meeting #59	S6-240196

Athens, Greece, 26th February – 1st March 2024	(was S6-234054)

Source:	Ericsson

Title:	Solution to KI#7 on AI/ML member selection policies provisioning and management

Spec:	3GPP TS 23.700-82 V0.2.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

It is essential to clarify the terminology:

- the AIML member discovery (when the VAL server wants to one-time get the list of available AIML clients without any further action); and

- the AIML member selection (when the VAL server selects and allocates the AIML member for the AIML operation, and the AIML server will monitor/configure/manage the selected member condition in order to support the AIML operation). Manage includes re-selection (including/excluding/replacing) of the selected AIML members in real-time.

This functionality is related to member selection and re-selection and is needed in the following scenarios:

- VAL server requests the AIML server to select X AIML members for the operation and provide the proper monitoring/configuration;

- The AIML server provides monitors the state of AIML members and performs re-selection actions based on the provisioned policies.

Examples of the use-cases on AIML policies application:

- Given: The critical vulnerability is found in OS X version of AIML Enablement clients. Action: The VAL server can add or update the selection policies to exclude all AIML members with the given OS X version. Result: the AIML Enablement Server initiates re-selection procedure and excluding/replacing all AIML members with the given OS X version.

- Given: The AIML Enablement Client was selected as a member for participate in the AIML operation(s) and experiences congestion in the network. Action: The AIML Enablement Server observes the congestion state of the AIML client. The AIML Enablement Server configures the nework QoS for the AIML client (based on provided member participation policies by the VAL server) in order to keep the AIML Enablement Client as selected member for the AI/ML operation(s). If failed, the AIML Enablement Server initiates the re-selection procedure. Result: AIML Enablement Server maintains the continuity of the ALML process via generating or updating 5G network policies based on the provided AIML member selection policies.

- Given: The AIML Enablement Server is assisting the AIML member selection. Action: The AIML Enablement Server may utilize ADAE and NWDAF services to predict the parameters that are provided in the AIML member selection policies in the given time horizon. The AIML Enablement Server can utilize the received predictions to select the members for AIML operation(s). Result: The AIML Enablement Server provides a reliable and continues AIML experience for the VAL server without switching the AIML members.

Thus, AIML policies for member selection and re-selection enables continues and reliable AIML experience for the VAL application.

2. Reason for Change

The provisioning and management of the The AI/ML member selection policies are essential for the AI/ML Services in regular, distributed, and federated learning scenarios.

3. Conclusions

This pCR proposes the provisioning and management of the AI/ML member selection policies in the AIMLAPP layer.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.2.0.

# *** First Change ***

## 8.0	Mapping of solutions to key issues

Table 8.1-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |  |
| Sol #4 |  | X |  |  |  |  |  |
| Sol #5 |  | X |  |  |  |  |  |
| Sol #6 |  |  | X |  |  |  |  |
| Sol #7 |  |  | X |  |  |  |  |
| Sol #8 |  |  | X |  |  |  |  |
| Sol #9 |  |  | X |  |  |  |  |
| Sol #10 |  |  |  |  |  |  | X |
| Sol #A |  |  |  |  |  |  | X |

* * * Next Change * * * *

## 8.A	Solution #A: AI/ML member selection policies provisioning and management

### 8.A.1	General

The following clauses specify procedures, information flows and APIs for Key issue# A to provision and manage the AI/ML member selection policies for the AI/ML Enablement services.

### 8.A.2	Procedures

#### 8.A.2.1	AI/ML member selection policies provisioning and management

Figure 8.A.2-1: AI/ML member selection policies provisioning and management procedure

1.	The VAL server sends an AI/ML member selection policies provisioning and management request as defined in clause 8.A.3.1.

2.	Upon receiving the request, the AI/ML Enablement Server performs an authorization check of the VAL server.

3.	If the VAL server is authorised, the AI/ML Enablement Server stores the AI/ML member selection policies and applies the provisioned policies to ongoing and further the AI/ML operations (e.g., ML model training).

4.	AI/ML Enablement Server provides the response to the VAL server with a status via AI/ML member selection policies provisioning and management response defined in clause 8.A.3.2.

NOTE:	The service operation defined in clause 8.A.2.1 can be utilized for AI/ML member selection policies management (i.e., update/delete) functionality.

### 8.A.3	Information flows

#### 8.A.3.1	AI/ML member selection policies provisioning and management request

Table 8.A.3.1-1 describes the information flow from the VAL server to the AI/ML Enablement Server as a request for the AI/ML member selection policies provisioning and management.

Table 8.A.3.1-1: AI/ML member selection policies provisioning and management request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M (NOTE 1) | The identity of the VAL server performing the request. |
| VAL service ID | M | Identify the VAL service ID. |
| List of AI/ML traffic QoS policies | O | Provides the list of AI/ML traffic QoS policies, e.g., AI/ML traffic QoS parameter(s) (e.g., bitrate, jitter, latency, PLR, connection sustainability) with the corresponding threshold(s) and threshold matching direction(s) (e.g., above, below or crossed). (NOTE 3) |
| List of AI/ML compute capability policies | O | Provides the list of the AI/ML compute capability policies, e.g., the OS, OS version, hardware capabilities, software capabilities. (NOTE 3) |
| List of AI/ML member mobility policies | O | Provides the list of mobility policies for the AI/ML member, e.g., low mobility, high mobility, stationary. (NOTE 3) |
| List of AI/ML member participation configurations | O | Provides the list of the AI/ML member participation configurations as defined in table 8.10.3.1-1. (NOTE 2) |
| NOTE 1: This information element shall not be updated in the AI/ML member participation configurations management request. NOTE 2: The member participation configurations represent the requestor conditions for the member selection based on the configurations provided in table 8.10.3.1-1 to be selected to participate into the AI/ML operations. NOTE 3 The policy format is provided in table 8.A.3.1-2. |  |  |

Editor’s note: Whether additional AI/ML capabilities are needed is FFS.

Table 8.A.3.1-2: Policy format

| Information element | Status | Description |
| --- | --- | --- |
| Policy ID | M | The identity of the policy. |
| Action | M | Represents the policy action, e.g., include member, exclude member. |
| List of the conditions | M | Provides the list of conditions, e.g., the threshold(s) with the associated threshold matching direction(s) or the set of value(s). |

#### 8.A.3.2	AI/ML member selection policies provisioning and management response

Table 8.A.3.2-1 describes the information flow from the AI/ML Enablement Server to the VAL server as a response for the AI/ML member selection policies provisioning and management.

Table 8.A.3.2-1: AI/ML member selection policies provisioning and management response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Indicates success or failure of the request |

# *** End of Changes ***