3GPP TSG-SA WG6 Meeting #59	S6-240200

Athens, Greece, 26th February – 1st March 2024

Source:	Ericsson

Title:	Support multi-modal QoS measurement and exposure in SEALDD

Spec:	3GPP TS 23.700-23 V0.1.0

Agenda item:	8.5

Document for:	Approval

1. Introduction

The XR traffic has Multi-Modal nature that requires the improvement in the existing SEAL Data Delivery service to support the multi-modal handling for the SEALDD enabled data transmission quality measurement.

2. Reason for Change

KI#1 has the following open issues:

-	identify what KPI the application enabling layer could provide that impact the QoS of the XR services.

-	how the KPI could be measured at the application enabling layer, and the enhancement of the SEALDD services.

The XR scenario requires :

- more granular QoS measurement and exposure, .e.g., Uplink/Downlink/End-to-End for individual and set of multi-modal flows;

- mapping between measured QoS and XR specific QoE metrics, e.g., Motion-to-Photon for edge deployed XR application.

  - The Motion-to-Photon latency for edge deployed XR application example, the camera on the XR device captures the motion and utilize SEALDD connection to send the video stream to the VAL server deployed in Edge Data Network. The VAL server processes the stream and sends back the processed data steam to the XR device. The device renders data stream on the display. The E2E latency can represent the lower bound for the Motion-to-Photon latency for edge deployed XR application (excluding delay for rendering of the perocessed data steram on the display).

- configurable aggregation accross multiple Multi-Modal Flows.

3. Conclusions

This pCR proposes a solution to address multi-modal service for XR traffic, which enables QoS measurement in XR scenario.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-23 V0.1.0.

* * * First Change * * * *

## 7.1	Mapping of solutions to key issues

Table 7.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #X | x |  |  |  |

* * * Next Change (all texts are new) * * * *

## 7.x	Solution #X: Support of QoS measurement for Multi-Modal traffic in SEALDD layer

### 7.x.1	Architecture Impacts

This solution is based on architecture of SEALDD as described in 3GPP TS 23.433 [23433].

### 7.x.2	Solution description

The solution improves the SEALDD layer defined in 3GPP TS 23.433, i.e.:

1.	The Multi-Modal Service Info is included into the SEALDD enabled data transmission quality measurement subscription request in order to signal the multi-modal service QoS request in SEALDD layer;

2.	The handling of the Multi-Modal Service Info is specified in Data transmission quality measurement; and

3.	Finer granularity of QoS measurement identifiers, i.e., UL/DL/E2E. Example: UL jitter.

4.	New multi-modal crossflows identifiers are proposed including Multi-Modal flows aggregation method. Examples: maximum crossflow average uplink PLR for X multi-modal flows.

5.	Mapping between measured QoS and XR specific QoE metrics, e.g., Motion-to-Photon for edge deployed XR application.

6.	Improvement of the SEALDD Client for the crossflow RTT measurement for multiple devices in XR scenario, e.g., wireless controllers and headset interaction.

#### 7.x.2.2	Impact to existing SEALDD procedures

The SEALDD procedures and information flows in 3GPP TS 23.433 [23433] can be enhanced (highlighted in bold italics) as follows.

NOTE:	SEALDD data transmission procedures with single SEALDD connection are described as typical example below.

#### 9.7.2.1 Data transmission quality measurement Figure 9.7.2-1 illustrate the procedure for SEALDD enabled data transmission quality measurement. The SEALDD client and SEALDD server is enhanced to carry out the data transmission quality measurement. Pre-conditions: 1. The SEALDD server and SEALDD client are synchronized to the time source provided by 5GS as specified in 3GPP TS 23.501 [5]. 2. The VAL server discovers and selects the SEALDD server by CAPIF functions. Figure 9.7.2.1-1: SEALDD enabled data transmission quality measurement procedure 1. An on-going regular data transmission connection is established according to clause 9.2.2.2. 2. The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), requirement of transmission quality measurement (e.g. latency, jitter, bitrate, packet loss rate) and measurement target UE (e.g. a single UE, a group of UEs or all UEs) or Multi-Modal Service Info, and may also include reporting criteria, reporting frequency, spatial condition and temporal condition. NOTE: The spatial and/or temporal condition can be used by SEALDD server to apply when and where the measurement is performed. For instance, the measurement is expected to be done for a group of VAL UEs with a scheduled route (from city A to city B via highway A2 and A3), from 9:00 a.m. to 11:00 a.m. on Tuesday and from 1:00 p.m. to 5:00 p.m. on Thursday. 3. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server sends a response to the VAL server with the subscription ID, expiration time. 4. If the transmission quality measurement requirement list provided by VAL server in step 2, indicates that the latency is needed to be measured, the SEALDD server initiates the DL packet delay measurement. The SEALDD server encapsulates the DL monitoring packet (i.e. DL SEALDD packet with SEALDD DL monitoring header and VAL traffic as payload, or dummy DL SEALDD packet generated for data transmission quality monitoring) with local time T1 when the SEALDD server sends out the DL monitoring packets. The SEALDD server considers the spatial and/or temporal conditions when starting/resuming the transmission quality measurement. If the conditions are not satisfied, the SEALDD server stops/suspends the transmission quality measurement. NOTE: For other metrics in transmission quality measurement requirement list (e.g. bitrate), the transmission quality result can be obtained by performance detection on the SEALDD server within a period of time. 5. The SEALDD client receives the DL monitoring packet, and records the local time T2. 6. Similarly, the SEALDD client encapsulates the UL monitoring packet (i.e. UL SEALDD packet with SEALDD UL monitoring header and VAL traffic as payload, or dummy UL SEALDD packet generated for data transmission quality monitoring) with local time T2 recorded in step 5 and local time T3 when the SEALDD client sends out the UL monitoring packet. 7. The SEALDD server records the local time T4 when the SEALDD server receives the UL monitoring packet and calculates the latency with T1, T2, T3, T4. The SEALDD server can also calculate the bitrate, jitter and packet loss rate over a certain period over a specific SEALDD connection by recording the status of the SEALDD packets carrying VAL traffic or dummy SEALDD packets generated for transmission quality measurement reports. The SEAL DD server also evaluates the reporting criteria if present in the SEALDD transmission quality measurement subscription request in order to generate the transmission quality measurement report. 8. The SEALDD server reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, packet loss rate) to the VAL server via the notification message. When a VAL group ID or a list of VAL UE IDs or all VAL UEs indication is received in step 2, step 4 to step 7 is repeated for VAL UEs in the group/list or for all VAL UEs. The SEALDD server maps the VAL UE group ID to a list of VAL UE IDs if a VAL group ID is received. The SEALDD server identifies SEALDD connections corresponding to the desired VAL UE(s) to trigger measurement. If the Multi-Modal Service Info is provided in step 2, step 4 to step 7 is repeated for VAL UEs for each multi-modal flow identified in the Modal Service Info. And depending on the reporting requirement for multiple UEs or SEALDD Multi-Modal flows, the SEALDD server calculates the needed report for the VAL server. When the VAL server decides to update or unsubscribe transmission quality measurement subscription after performing step 2 and step 3, the VAL server can send data transmission quality measurement subscription update request and data transmission quality measurement unsubscribe request to SEALDD server, as specified in Table 9.7.3.9-1 and Table 9.7.3.11-1, respectively. #### 9.7.2.2 Data transmission quality query Figure 9.7.2.2-1 illustrate the procedure for SEALDD enabled data transmission quality query. This procedure is used to obtain the historical transmission quality result already measured as described in clause 9.7.2.1. Pre-conditions: 1. The SEALDD server performs the data transmission quality measurement procedure, as described in clause 9.7.2.1. Figure 9.7.2.2-1: SEALDD enabled data transmission quality query procedure 1. The consumers (e.g. VAL server, SEALDD server, NSCE server, ADAE server) can send a SEALDD transmission quality query request to the SEALDD server to obtain the transmission quality measurement result. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), VAL UE ID or VAL UE group ID, or Multi-Modal Service Info. 2. The SEALDD server responds with the transmission quality measurement result (e.g. packet delay, bitrate, packet loss rate). #### 9.7.2.3 Data transmission quality measurement reported by SEALDD client Figure 9.7.2.3-1 illustrate the procedure for SEALDD enabled data transmission quality measurement for VAL traffic. The SEALDD client receives transmission quality measurement requirement, decides to start VAL data transmission monitoring and generates measurement reports. Figure 9.7.2.3-1: VAL data transmission quality measurement reported by SEALDD client 1. An on-going regular data transmission connection is established according to clause 9.2.2.2. The transmission quality measurement can be triggered by VAL server or VAL client, which is described in step 2 to step 5 and step 6, correspondingly. The request may include two VAL UE IDs and corresponding flow information (e.g. [UE1/UL flow, UE2/DL flow]) associated in the same multi-modal service for crossflow RTT measurement. 2. The VAL server sends a SEALDD transmission quality measurement subscription request to the SEALDD server. The request includes the identifiers of the application traffic (e.g. VAL service ID, VAL server ID), requirement of transmission quality measurement (e.g. latency, jitter, bitrate) and measurement target UE (e.g. a single UE, a group of UEs or all UEs), and may also include reporting criteria, reporting frequency, spatial condition and temporal condition. NOTE 1: The spatial and/or temporal condition can be used by SEALDD client to apply when and where the measurement is performed. For instance, the measurement is expected to be done for a group of VAL UEs with a scheduled route (from city A to city B via highway A2 and A3), from 9:00 a.m. to 11:00 a.m. on Tuesday and from 1:00 p.m. to 5:00 p.m. on Thursday. 3. Upon receiving the request, the SEALDD server performs an authorization check. If authorization is successful, the SEALDD server responds to the VAL server. 4-5. The SEALDD server sends a SEALDD transmission quality measurement subscription request to the SEALDD client and the SEALDD client responds to the SEALDD server. The SEALDD client, based on the received service quality guarantee policy including thresholds and action, can take corrective action as described in clause 9.7.2.3. If crossflow RTT measurement requirement is received from the VAL server, the SEALDD server identifies the SEALDD connections for the involved VAL UEs and sends to the SEALDD client(s) with crossflow RTT measurement requirement including corresponding UL/DL flow information. 6. The VAL client triggers the SEALDD transmission quality measurement procedure to the SEALDD client, in order to collect the measurement report information. 7. After SEALDD client determines to start measurement process, upon UL packet arrival, the SEALDD client initiates the UL packet delay measurement. The SEALDD client encapsulates the UL monitoring packet (i.e. UL SEALDD packet with SEALDD UL monitoring header and VAL traffic as payload for VAL data transmission quality monitoring) with local time T1 when the SEALDD client sends out the UL monitoring packet. The SEALDD client considers the spatial and/or temporal conditions when starting/resuming the transmission quality measurement. If the conditions are not satisfied, the SEALDD client stops/suspends the transmission quality measurement. For crossflow RTT measurement, the SEALDD client received UL flow information starts recording local time T1 when sending out the 1st UL packet (or the 1st UL packet in a PDU set) matching the UL flow information to the SEALDD server. The T1 is sent in the encapsulated UL monitoring packet to the SEALDD server. 8. The SEALDD server receives the UL monitoring packet, and records the local time T2. 9. Similarly, the SEALDD server encapsulates the DL monitoring packet (i.e. DL SEALDD packet with SEALDD DL monitoring header and VAL traffic as payload, or dummy UL SEALDD packet generated for data transmission quality monitoring in case there is no DL VAL traffic for DL packet delay monitoring) with local time T2 recorded in step 8 and local time T3 when the SEALDD server sends out the DL monitoring packet. NOTE 2: When the SEALDD server sends the dummy UL packet as monitoring response to the SEALDD client depends on SEALDD server implementation. For crossflow RTT measurement, when the 1st DL packet (or the last DL packet in a PDU set) matching the received DL flow information is to be sent, the SEALDD server encapsulates the DL monitoring packet with previously received T1 and sends the DL monitoring packet to the SEALDD client. 10. The SEALDD client records the local time T4 when the SEALDD client receives the DL monitoring packet and calculates the latency with T1, T2, T3, T4. The SEALDD client can also calculate the bitrate and jitter over a certain period over a specific SEALDD connection by recording the status of the SEALDD monitoring packets. The SEALDD client also evaluates the reporting criteria if present in the SEALDD transmission quality measurement subscription request in order to generate the transmission quality measurement report. For crossflow RTT measurement, if T1 is received in the DL packet, the SEALDD client records local time T2 and calculates RTT based on T1 and T2. Depending on which entity triggers the data transmission quality measurement, step 11 and step 12 corresponds to step 2 to step 5, step 13 corresponds to step 6. 11-12. The SEALDD client reports the data transmission quality measurement results (e.g. latency, jitter, bitrate, crossflow RTT) to the VAL server via the SEALDD server. 13. The SEALDD client reports the data transmission quality measurement results to the VAL client. NOTE 3: The End-to-End latency calculated by the SEALDD client in step 10 can be used as an estimation of the lower bound for the Motion-to-Photon latency in the XR scenarios. When a VAL group ID or a list of VAL UE IDs or all VAL UEs indication is received in step 2, step 4 to step 11 is repeated for VAL UEs in the group/list or for all VAL UEs. The SEALDD server maps the VAL UE group ID to a list of VAL UE IDs if a VAL group ID is received. The SEALDD server identifies SEALDD connections corresponding to the desired VAL UE(s) to trigger measurement. And depending on the reporting requirement for multiple UEs, the SEALDD server collects and aggregates the needed report for the VAL server. #### 9.7.3.1 SEALDD enabled data transmission quality measurement subscription request Table 9.7.3.1-1 describes the information flow from the VAL server to the SEALDD server for subscribing to the data transmission measurement service. Table 9.7.3.1-1: SEALDD transmission quality measurement subscription request Information element; Status; Description. Application traffic identifiers; M; Identify of the application traffic (e.g. VAL server ID, VAL service ID). VAL UE identity; O (See NOTE 1, NOTE 2); Identifier of the VAL UE for which measurements need to be provided, e.g. UE ID. VAL UE group ID; O (See NOTE 1, NOTE 2); Identifier of a specific VAL UE group, as defined in clause 7.5 of 3GPP TS 23.434 [4].. VAL UE identity list; O (See NOTE 1, NOTE 2); Identifies a list of VAL UEs, e.g. the list of UE ID.. All VAL UEs Indication; O (See NOTE 1, NOTE 2); Indicates all VAL UEs of the application identified by application traffic identifiers.. Multi-Modal Service Info; O; Represents the requested Multi-Modal Service Info.. > Multi-Modal Service ID; M; Represents the Application layer Multi-Modal Service ID.. > Multi-Modal Flows; O; Identify the list of Multi-Modal Flows within the Multi-Modal Service ID. If omitted, the request affects all flows.. Measurement conditions; O; Indicates the temporal and/or spatial conditions.. Transmission quality measurement requirements list; M; The measurement requirement information. > Measurement ID; M; Measurement identifiers, e.g. latency, bitrate, packet loss rate, jitter. > Reporting frequency; O; The reporting frequency of measurement results (e.g. periodic reporting). If not present, it implies periodic reporting.. > Reporting periodicity; O; If the reporting frequency is periodic, the reporting periodicity shall be provided. For multiple UEs, it is recommended to give sufficient time to allow report aggregation.. > Reporting granularity; O; The reporting granularity indicates whether the measurement report is for individual VAL UE or for VAL UE group or for all VAL UEs, if VAL UE group or all VAL UEs is the measurement target.. > Measurement period window; O; Indicates the measurement period window for transmission quality measurements. > Measurement expiration time; O; Indicates the measurement expiration time. > Reporting criteria; O; Indicates the criteria for reporting measurement results, e.g. if the latency or bitrate reaches below or above a certain value. It also includes a unique identifier for each criteria of more than one criteria is specified.. NOTE 1: One of them shall be present as the measurement target UE. NOTE 2: These IEs and Multi-Modal Service Info IE are mutually exclusive.. #### 9.7.3.3 SEALDD enabled data transmission quality measurement notification Table 9.7.3.3-1 describes the information flow from the SEALDD server to the VAL server for notifying the transmission quality measurement reports. Table 9.7.3.3-1: SEALDD transmission quality measurement notification Information element; Status; Description. Subscription ID; M; Subscription identifier corresponding to the subscription.. Transmission quality measurement reports list; M; The generated transmission quality results in SEALDD server, as specified in Table 9.7.3.3-2.. Table 9.7.3.3-2 describes the information elements for the transmission quality measurement reports list, provided by the SEALDD server after performing transmission quality measurement. Table 9.7.3.3-2: SEALDD transmission quality measurement reports list Information element; Status; Description. > Measurement ID; M; Measurement identifiers (e.g. UL/DL/E2E latency, UL/DL bitrate, UL/DL/E2E packet loss rate, UL/DL/E2E jitter) and multi-modal crossflows identifiers (e.g., UL/DL/E2E crossflows latency, UL/DL crossflows bitrate, UL/DL/E2E crossflows packet loss rate, UL/DL/E2E crossflows jitter). > VAL UE ID(s); O (see NOTE); It indicates the VAL UE(s) under SEALDD measurement. For a single VAL UE, it can be omitted and the associated measurement values are for the single VAL UE. For multiple VAL UEs with reporting granularity set to individual UE, the associated measurement values are for individual VAL UE as indicated in this IE. For multiple VAL UEs with reporting granularity set to VAL UE group/list or all VAL UEs, the associated measurement values are aggregation for all VAL UEs or the VAL UE group/list and this IE includes the measured VAL UEs.. > Multi-Modal Service Info; O (see NOTE); Represents the requested Multi-Modal Service Info.. >> Multi-Modal Service ID; M; Represents the application layer Multi-Modal Service ID.. >> Multi-Modal Flows; O; Identify the list of Multi-Modal Flows within the Multi-Modal Service ID. If omitted, the request affects all flows.. >> Multi-Modal flows aggregation; O; Identify the Multi-Modal flows aggregation method (e.g., average, minimum, maximum, kPercentile).. > Average measurement value; O; The average measurement value of measurement results. > Minimum measurement value; O; The minimum measurement value of measurement results. > maximum measurement value; O; The maximum measurement value of measurement results. > Standard deviation measurement value; O; Standard deviation measurement value of measurement results. > kPercentile measurement value; O; Indicates the kpercentile measurement value of measurement results. > Measurement period; O; Indicates the measurement period. > Timestamp; O; Indicates the timestamp of measurement results. NOTE: These IEs are mutually exclusive..

### 7.x.3	Solution evaluation

This solution addresses KI#1 to support:

-	what KPI the application enabling layer could provide that impact the QoS of the XR services.

-	how the KPI could be measured at the application enabling layer, and the enhancement of the SEALDD services.

# *** End of Changes ***