3GPP TSG-SA WG6 Meeting #59	S6-240229

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.289 | CR | 0116 | rev |  | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MBS service area update failure cause handling |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2023-10-30 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In SA2#160, the CR addressing the MBS service area update issue was agreed in S2-2311901(R17 CR) and S2-2311276 (R18 mirror CR) In the CR, SA2 concluded that: - Case #1- TMGI allocation: NEF reject the request if cannot satisfy the service area request. - Case #2 - MBS Session Creation without TMGI: NEF reject the request if cannot satisfy the service area request. - Case #3 - MBS Session Creation with TMGI: MB-SMF partial accept the request, and provide non-supported MBS service area and indication for this condition; - Case #4 - MBS Session Update: MB-SMF partial accept the request, and provide non-supported MBS service area and indication for this condition. Further in CT4 and CT3, the following respone were defined accordingly: - During TMGI allocation or MBS session creation: "MBS_SERVICE_AREA_NOT_SUPPORTED", HTTP "403 Forbidden" – MBS service area is not supported by any MB-SMF. "MBS_SERVICE_AREA_TOO_LARGE"+ ReducedMbsServArea, HTTP "403 Forbidden"- cannot be covered by the MB-SMF Service Area of a single MB-SMF. "ReducedMbsServArea", HTTP "201 Created" - partially accept. - Duing the MBS session update: HTTP "200 OK" with reducedMbsServArea - partially accept. In SA6, how the MC service server handles the above response from the network needs to be defined in order to efficiently make use of the MBS feature: - HTTP "403 Forbidden"+"MBS_SERVICE_AREA_NOT_SUPPORTED", the MC service server shall directly use the unicast delivery. - HTTP "403 Forbidden"+"MBS_SERVICE_AREA_TOO_LARGE"+ ReducedMbsServArea, the MC service server can retry with the ReducedMbsServArea or use the unicast delivery. - HTTP "200 OK" with reducedMbsServArea, the MC service server can retry with the reducedMbsServArea or use the unicast delivery. For the service area which is not supported by the network (i.e., not covered by the reducedMbsServArea) the MC service server can retry with a new MBS session or use the unicast delivery. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the clause to describe how the MC service server behave when the requesting MBS service area is not supported or fully supported by the network. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MC service server can not handle the MBS service area update failure correctly. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 7.3.3.4a (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS); Stage 2".

[3]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[4]	3GPP TS 23.281: "Functional architecture and information flows to support Mission Critical Video (MCVideo); Stage 2".

[5]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[6]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[7]	3GPP TS 23.501: "System architecture for the 5G System (5GS)".

[8]	3GPP TS 23.002: "Network Architecture".

[9]	3GPP TS 23.503: "Policy and Charging Control Framework for the 5G System (5GS); Stage 2".

[10]	3GPP TS 23.502: "Procedures for the 5G System (5GS)".

[11]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[12]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[13]	3GPP TS 22.281: "Mission Critical (MC) Video".

[14]	3GPP TS 22.282: "Mission Critical (MC) Data".

[15]	3GPP TS 23.247: "Architectural enhancements for 5G multicast-broadcast services; Stage 2".

[16]	3GPP TS 23.468: "Group Communication System Enablers for LTE (GCSE_LTE); Stage 2".

[17]	3GPP TS 23.304: "Proximity based Service (ProSe) in the 5G System (5GS); Stage 2".

[18]	3GPP TS 23.237: "IP Multimedia Subsystem (IMS) Service Continuity; Stage 2".

[19]	3GPP TS 38.331: "NR; Radio Resource Control (RRC) protocol specification".

[20]	3GPP TS 23.479: "UE MBMS APIs for Mission Critical Services".

[21]	3GPP TS 26.502: "5G Multicast-Broadcast User Service Architecture".

[29.532]	3GPP TS 29.532: "5G Multicast-Broadcast Session Management Services; Stage 3".

[29.522]	3GPP TS 26.502: "Network Exposure Function Northbound APIs; Stage 3".

# /******************** First Changes ********************/

7.3.3.4a	MBS service area update failure handling

The MBS service area requested by the MC service server during the TMGI allocation or MBS session update procedure towards the 5G core network may be rejected or partially accepted, the MC service server may take different actions as follows:

-	If the MBS service area requested in the TMGI allocation or MBS Session Creation without TMGI procedure as specified in 3GPP TS 23.247 [15] is rejected with the error "MBS_SERVICE_AREA_NOT_SUPPORTED" as specifined in 3GPP TS 29.532 [29.532] and 3GPP TS 29.522 [29.522], the MC service server uses the unicast delivery.

-	If the MBS service area requested in the TMGI allocation or MBS Session Creation with TMGI procedure was rejected with the error "MBS_SERVICE_AREA_TOO_LARGE" and the ReducedMbsServArea indicating the MBS Service Area that can be supported by the network as specifined in 3GPP TS 29.532 [29.532] and 3GPP TS 29.522 [29.522], the MC service server may retry with the returned ReducedMbsServArea. Alternatively, the MC service server may also determine to use the unicast delivery for the group communication in that area.

-	If the MBS service area requested in MBS Session Creation with TMGI or MBS Session Update procedure as specified 3GPP TS 23.247 [15] is partially accepted with the ReducedMbsServArea as specifined in 3GPP TS 29.532 [29.532] and 3GPP TS 29.522 [29.522], the MC service server may try another new MBS session with the MBS service area which is not covered by the MbsSessionUpdateResp, or alternatively use the unicast delivery for that area.