3GPP TSG-SA WG6 Meeting #59	S6-240240

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0575 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Federation procedure for EES and EAS triggers |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NTT DOCOMO |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-2-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | There are only two federation patterns specified in the Edge Application of 3GPP_SA6: EEC trigger pattern and EES trigger pattern, and the steps are only shown for the EEC trigger pattern, but the steps for the EES trigger pattern and the EAS trigger pattern are not mentioned. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Federation procedures for EES and EAS triggers are described. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Federation procedures for EES and EAS triggers are not supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

8.18.2.3.4	EAS discovery via Partner ECSP, EES triggered

The S-EES can use the Retrieve T-EES procedure specified in clause 8.8.3.3 to obtain the T-EES information of the partner ECSP from leading ECS, the leading ECS identifies the T-EES based on the UE serving PLMN ID and EES allowed MNO information.

Editor' Note: Whether and how to enhance the Service provisioning information retrieval procedure for the edge node sharing case considering multiple federations by an operator is FFS.

When service continuity is required, the EES can trigger the T-EAS discovery procedures as defined in clause 8.8.3.2 with the EES of the partner ECSP to find an EAS with respect to ENS. The EAS discovery request includes UE'’s serving MNO information (e.g. MNO name, PLMN ID). Upon receiving the request from the S-EES, the T-EES of the partner ECSP identifies the EAS(s) considering the allowed MNO information of the registered EASs and UE's serving MNO information.

Figure 8.18.2.3.4-1: EAS discovery, EES triggered

1．EES (ECSP-B) analyze the user's service usage, and if the usage patterns match, move to 2. (Communicate with the core NW side such as NWDAF, analyze the frequency of using different services after a specific operation, and create conditions, etc.)

2.  EES (ECSP-B) requests permission from EEC (user) to execute federation.

3.  EES(ECSP-B) send a response to the federation.

4.	The Retrieve EES request may include an ENS indication so that the ECS (ECSP-B) can skip checking T-EES(s) locally registered in the ECS (ECSP-B).

5. Once EES information of the partner ECSP is available, the EES (ECSP-B) sends the EAS discovery request to the EES (ECSP-A) as described in step 3 of clause 8.8.3.2. The EAS discovery request includes the information   of the MNO serving the UE (e.g. MNO name, PLMN ID).

6.	EES (ECSP-B) provides the discovered information to the EES as part of the EAS discovery response as specified in clause 8.5.

* * * Next Change * * * *

##### 8.18.2.3.x	EAS discovery via Partner ECSP, EAS triggered

The S-EES can use the Retrieve T-EES procedure specified in clause 8.8.3.3 to obtain the T-EES information of the partner ECSP from leading ECS, the leading ECS identifies the T-EES based on the UE serving PLMN ID and EES allowed MNO information.

Editor' Note: Whether and how to enhance the Service provisioning information retrieval procedure for the edge node sharing case considering multiple federations by an operator is FFS.

When service continuity is required, the EAS can trigger the T-EAS discovery procedures as defined in clause 8.8.3.2 with the EES of the partner ECSP to find an EAS with respect to ENS. The EAS discovery request includes UE'’s serving MNO information (e.g. MNO name, PLMN ID). Upon receiving the request from the S-EAS, the T-EES of the partner ECSP identifies the EAS(s) considering the allowed MNO information of the registered EASs and UE's serving MNO information.

Figure 8.18.2.3.x-1: EAS discovery for ENS, EAS triggered

1．EAS (ECSP-B) analyze the user's service usage, and if the usage patterns match, move to 2. (Communicate with the core NW side such as NWDAF, analyze the frequency of using different services after a specific operation, and create conditions, etc.)

2. The EAS(ECSP-B) sends EAS discovery request to EES (ECSP-B) as specified in clause 8.5 and may include the endpoint of EES (ECSP-A) in the request, if provided by the ECS as specified in clause 8.18.2.2.1.

3.  EES (ECSP-B) requests permission from EEC (user) to execute federation.

4.  EES(ECSP-B) send a response to the federation.

5.	The Retrieve EES request may include an ENS indication so that the ECS (ECSP-B) can skip checking T-EES(s) locally registered in the ECS (ECSP-B).

6.  Once EES information of the partner ECSP is available, the EES (ECSP-B) sends the EAS discovery request to the EES (ECSP-A) as described in step 3 of clause 8.8.3.2. The EAS discovery request includes the information   of the MNO serving the UE (e.g. MNO name, PLMN ID).

7.	EES (ECSP-B) provides the discovered information to the EES as part of the EAS discovery response as specified in clause 8.5.

* * * End of Change * * * *