3GPP TSG-SA WG6 Meeting #59	S6-240243

Athens, Greece, EU, 26th of February – 1st of March 2024	(revision of S6-233580)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.379 | CR | 0400 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Floor control priority when using multi-talker control |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Stage 1 TS 22.179 clause 6.2.3.7.4 require an override mechanism based on priority hierarchy per participant for multi-talker control. In clause 10.9.1.2.2 of TS 23.379 the Floor request message allows to set a priority level, but the configuration data which configures the priotity level to be allowed in groups capable to support multi-talker floor control is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding floor request priority parameter per group when for groups using multi-talker floor control. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Stage 1 requires are not completely fulfilled. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Annex A.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * 1st change * * * *

# A.4	MCPTT related group configuration data

The general aspects of group configuration are specified in 3GPP TS 23.280 [16].

Parameters specified in table A.4-1 are child parameters of the "MCPTT configuration" parameter specified in table A.4-1 in 3GPP TS 23.280 [16]. Parameters specified in table A.4-2 are child parameters of the "MCPTT configuration" parameter specified in table A.4-2 in 3GPP TS 23.280 [16]. Parameters specified in table A.4-3 are child parameters of the "MCPTT configuration" parameter specified in table A.4-3 in 3GPP TS 23.280 [16].

Table A.4-1: Group configuration data (on and off network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-6.4.9-006] of 3GPP TS 22.280 [17] | >> Preferred voice codecs for MCPTT group | Y | Y | Y |
| [R-5.7.2.1.1-013], [R-5.7.2.4.1-008] of 3GPP TS 22.280 [17] | >> Indication whether emergency group call is permitted on the MCPTT group | Y | Y | Y |
| [R-5.7.2.2.1-009] of 3GPP TS 22.280 [17] | >> Indication whether imminent peril group call is permitted on the MCPTT group | Y | Y | Y |
| [R-5.7.2.4.1-012] of 3GPP TS 22.280 [17] | >> Indication whether emergency alert is possible on the MCPTT group | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | >> Media confidentiality and integrity protection (see NOTE) | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | >> Floor control confidentiality and integrity protection (see NOTE) | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | >> Group media protection security material (see NOTE) | Y | N | Y |
| NOTE: Security mechanisms are specified in 3GPP TS 33.180 [19]. |  |  |  |  |

Table A.4-2: Group configuration data (on-network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-5.7.2.1.2-002] of 3GPP TS 22.280 [17] | >> Timeout value for the cancellation of an in-progress emergency for an on-network group call | N | Y | Y |
| [R-5.7.2.2.2-004] of 3GPP TS 22.280 [17] | >> Timeout value for the cancellation of an in progress imminent-peril group call for an on-network group call | N | Y | Y |
| [R-6.2.4-003], [R-6.4.9-002] of 3GPP TS 22.179 [2] | >> Group call hang timer | N | Y | Y |
| [R-6.4.9-003] of 3GPP TS 22.280 [17] | >> Maximum duration of group calls | N | Y | Y |
| Subclause 10.6.2 | >> Group call model used for group (chat/ pre-arranged) | Y | Y | Y |
| [R-6.2.1-004], [R-6.2.1-007] of 3GPP TS 22.179 [2] | >> Minimum number of affiliated group members acknowledging before start of audio transmission | N | Y | Y |
| [R-6.2.1-007] of 3GPP TS 22.179 [2] | >> List of group members which, if affiliated, have to acknowledge before start of audio transmission |  |  |  |
|  | >>>MCPTT IDs | N | Y | Y |
| [R-6.2.1-006], [R-6.2.1-007] of 3GPP TS 22.179 [2] | >> Geographical area where acknowledgement of all affiliated group members is required before start of audio transmission | N | Y | Y |
| [R-6.2.1-008], [R-6.2.1-009] of 3GPP TS 22.179 [2] | >> Timeout for acknowledgement of required group members | N | Y | Y |
| [R-6.2.1-008], [R-6.2.1-009], [R-6.2.1-012] of 3GPP TS 22.179 [2] | >> Action upon failure to receive acknowledgement from required group members before call timeout (proceed or abandon) | N | Y | Y |
| [R-6.2.1-001a] of 3GPP TS 22.179 [2] | >> Minimum number of group members which must be affiliated to allow the start of the group call | N | Y | Y |
| [R-6.2.1-001b] of 3GPP TS 22.179 [2] | >> List of group members which must be affiliated to allow the start of the group call |  |  |  |
|  | >>>MCPTT IDs | N | Y | Y |
| [R-6.4.9-004] of 3GPP TS 22.280 [17] | >> List of group members with receive-only capability (see NOTE 1) |  |  |  |
|  | >>>MCPTT IDs | N | Y | Y |
| [R-6.4.5-001], [R-6.4.5-003] of 3GPP TS 22.280 [17] | >> Authorisation of a user to request a list of affiliated members of a group | Y | Y | Y |
| [R-5.1.7-002], [R-6.2.2-001], [R-6.6.2.2-006,] [R-6.8.6.2-003] of 3GPP TS 22.280 [17] | >> Priority of the group | N | Y | Y |
| [R-6.8.6.2-003] of 3GPP TS 22.280 [17] | >> Pre-emption capability | Y | Y | Y |
| [R-6.8.6.2-003] of 3GPP TS 22.280 [17] | >> Pre-empted capability | Y | Y | Y |
| [R-5.13-001] of 3GPP TS 22.280 [17] | >> Group floor control protection security material for multicast use (see NOTE 2) | Y | Y | Y |
| [R-6.2.3.6.2-001] of 3GPP TS 22.179 [2] | >> Audio cut-in policy (enabled/disabled) | N | Y | Y |
| [R-6.2.3.7.2-001] of 3GPP TS 22.179 [2] | >> Support of multi-talker control | Y | Y | Y |
| [R-6.2.3.7.2-005] of 3GPP TS 22.179 [2] | >> Maximum number of simultaneous talkers applicable for multi-talker control | N | Y | Y |
| Subclause 7.4.2.3.5 and subclause 7.4.2.3.6 | >> Audio mixing is performed in the UE or in the network to support multi-talker control | Y | Y | Y |
| [R-6.2.3.7.2-003] of 3GPP TS 22.179 [2], [R-5.2.1-001] of 3GPP TS 22.280 [17] | >> List of group members which are allowed to talk (see NOTE 1, NOTE 4) |  |  |  |
|  | >>> MCPTT IDs | N | Y | Y |
| Subclause 6.2.3.7.4 of 3GPP TS 22.179 [2] | >>> Floor request priority | Y | Y | Y |
| 3GPP TS 23.283 [20] | >> Indication whether use of LMR E2EE is permitted on the MCPTT group | Y | N | Y |
| 3GPP TS 23.283 [20] | >> LMR specific identity for MCPTT group (see NOTE 3) | Y | N | Y |
| 3GPP TS 23.283 [20] | >> Group to key binding (see NOTE 3) | Y | N | Y |
| NOTE 1: A group member shall not be listed in both the list of group members with receive only capability and the list of group members which are allowed to talk. NOTE 2: The details of security related elements are specified in 3GPP TS 33.180 [19]. NOTE 3: This is an LMR specific parameter with no meaning within MC services. NOTE 4: This parameter is applicable for MCPTT groups with multi-talker control / sets the authorization to talk in pre-arranged groups used in broadcast calls. |  |  |  |  |

Table A.4-3: Group configuration data (off-network)

| Reference | Parameter description | MCPTT UE | MCPTT Server | Group management server |
| --- | --- | --- | --- | --- |
| [R-5.7.2.1.2-002] of 3GPP TS 22.280 [17] | >> Timeout value for the cancellation of an in-progress emergency for an off-network group call | Y | N | Y |
| [R-5.7.2.2.2-004] of 3GPP TS 22.280 [17] | >> Timeout value for the cancellation of an in progress imminent-peril group call for an off-network group call | Y | N | Y |
| [R-7.4-002] of 3GPP TS 22.179 [2] | >> Group call hang timer | Y | N | Y |
| [R-7.4-003] of 3GPP TS 22.280 [17] | >> Max duration of group calls | Y | N | Y |
| Subclause 10.6.3.9.1 | >> Default ProSe Per-Packet priority (as specified in 3GPP TS 23.303 [7]) values |  |  |  |
|  | >>> MCPTT group call signalling | Y | N | Y |
|  | >>> MCPTT group call media | Y | N | Y |
|  | >>> MCPTT emergency group call signalling | Y | N | Y |
|  | >>> MCPTT emergency group call media | Y | N | Y |
|  | >>> MCPTT imminent peril group call signalling | Y | N | Y |
|  | >>> MCPTT imminent peril group call media | Y | N | Y |

# * * * * End of changes * * * *