3GPP TSG-SA WG6 Meeting #59	S6-240264

Athens, Greece 26th February– 1st March 2024	(revision of S6-24xxxx)

Source:	Huawei, Hisilicon

Title:	EAS instantiation enhancement to satisfy E2E KPI requirements for XR application

Spec:	3GPP TR 23.700-23 v0.1.0

Agenda item:	8.5

Document for:	Approval

Contact:	Jian Zhang (zhangjian369@huawei.com)

1. Introduction

This paper proposes a solution for new key issue in S6-240262, to satifsy E2E KPI requirements for XR application by the EAS instantiation enhancement mechanism.

2. Reason for Change

In the new introduced key issue in S6-24xxxx, the following open issues are identified:

- How to support the E2E KPI guarantee to meet real-time XR service requirements, e.g. considersing the potiential enhancements to EDGEAPP in XR application server discovery or instantiation.

- What information is useful to interact betwwen 5G network and enablement layer to guarantee the real-time XR service requirements.

To meet the E2E KPI requirements to provide immersive experience for XR application, the EAS instantiation mechanism in EDGEAPP should be enhanced due to the following observations.

Observation#1: In Rel-17 and Rel-18, the EAS instatiation mechanism has been studied and specified in EDGEAPP topic of 3GPP TS 23.558, the EES may trigger the ECSP management to instatiate EAS as following described :

Upon receiving the EAS discovery request with EAS discovery filter from the EEC or the S-EES during the procedures for EAS discovery or ACR, the EES may fail to discover and select the EAS that matches the UE location and the requesting application characteristics specified in table 8.5.3.2-2 due to no EAS is available or instantiated. The EES may trigger the ECSP management system (which is specified in TS 28.538 [22]) to instantiate the EAS serving the AC in the EDN corresponding to the EAS that can be instantiable before returning the EAS information to the EEC or S-EES, based on the information about instantiable EASs which can be dynamically instantiated at the associated EDN. If EAS selection is performed by the EES, the selected EAS is dynamically instantiated if applicable.

However, there is no detailed information about what the size of the computing resource to the management system should be provided by EES. For the real-time XR applications with E2E KPI requirements, with the appropriate EAS with the required size of computing resource is critical to impact the processing latency of the application (e.g. media rendering in XR application server).

Proposal#1: Therefore, the EAS instantiation mechnism should be enhancement to consider how the appropriate EAS with the required size of computing resource can be determined and instantiated to satisfy the E2E KPI requirements for XR application.

Observation#2: In 5G network, the AF can request the required QoS by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502, as following:

The AF sends a request to reserve resources for an AF session using Nnef_AFsessionWithQoS_Create request message (UE address, AF Identifier, Flow description information or External Application Identifier, QoS Reference or individual QoS parameters, Alternative Service Requirements (as described in clause 6.1.3.22 of TS 23.503 [20]), DNN, S-NSSAI) to the NEF.

The above procedure focuses on the transmission quality of the 5G network. However, even with lower transmission latency provided by 5G network, if EAS cannot provide the appropriate lower application processing latency, the E2E KPI application requirements of XR application still cannot be satisfied.

Proposal#2: Therefore, only transmission optimization in the 5G network is far from sufficient to meet the E2E KPI application requirements of XR application. As the enablement layer, it can be the bridge between the 5G network and application server to determine how the E2E KPI application requirements of XR application can be achieved.

This contribution proposes that the enablement layer (i.e. EES) can dertermines the required EAS instantiation resource and/or the required QoS information. By interacting with management system and 5GC, the E2E KPI application requirements of XR application can be guaranteed.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-23 v0.1.0.

* * * First Change * * * *

7.1	Mapping of solutions to key issues

Table 7.1-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI#Y |
| --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |
| Sol #x |  |  |  |  | X |

* * * Next Change * * * *

7.x	Solution #x: EAS instantiation enhancement to satisfy E2E KPI requirements for XR application

7.x.1	Architecture Impacts

This solution is presented based on the existing EDGEAPP architecture in TS 23.558 [x].

7.x.2	Solution description

7.x.2.1	General

This solution proposes that the enablement layer (i.e. EES) can dertermines the required EAS instantiation resource and/or the required QoS information. By interacting with management system and 5GC, the E2E KPI application requirements of XR application can be guaranteed.

7.x.2.2	Procedure

The following figure provides the procedure of EAS instantiation enhancement to satisfy E2E KPI requirement (e.g. low latency) for XR application. The EES determines the required computing ability and/or the required QoS information based on the AC service KPI, the available computing capability and the available transmission capability for the requestd EAS.

Pre-conditions:

- EAS registers to EES according to EAS Registration procedure in clause 8.4.3 of TS 23.558 [x].

- EES registers to ECS according to EES Registration procedure in clause 8.4.4 of TS 23.558 [x].

- The ECSP management system provides the information about instantiable EASs (e.g. instantiable EAS ID, the available computing capability) to the EES.

Figure 7.x.2.2-1: EAS instantiation enhancement procedure

- The EEC performs service provisioning procedure, as described in clasue 8.3.3 of TS 23.558 [x].

- The EEC sends the EAS discovery request to the EES, the request includes the AC profile with the required EAS ID, and the AC service KPI (i.e. the E2E KPI requirements for XR application).

- The EAS discovery request triggers the EES to verify whether the instantiation of the EAS indicated in the request is required, as specified in clause 8.12.1 of TS 23.558 [x]. If EAS instantiation is required, the EES can dertermine the required computing ability and/or the required QoS information based on the AC service KPI, the available computing capability and the available transmission capability for the requestd EAS.

NOTE 1: The available computing capability is provided to EES by the ECSP management system as described in pre-condition, which can be used to as the maximum requested computing ability.

NOTE 2: The EES can obtain the available transmission capability from NWDAF, by utilizing the DN performance analytic in clause 6.14 of TS 23.288 [y], which can be used to as the maximum requested QoS information (i.e. transmission quality).

- If the EES determines that EAS instantiation is needed, the EES can request the ECSP management system to perform the EAS instantiation with the required EAS ID, and the required computing ability dertermined in step 3. Upon reception of the EAS instantiation request, the ECSP management system verifies EAS instantiation requirements and performs EAS instantiation as specified in 3GPP TS 28.538 [z]. After the EAS instantiation is completed, the EES can be informed by ECSP management with the result of EAS instantiation including the EAS endpoint information.

- The EES can send the AF request to 5GC via N33/N5 with the application descriptor (i.e. the EAS endpoint information after instantiated by ECSP management system), and the required QoS information dertermined in step 3, by utilizing the AF session with required QoS procedure in clause 4.15.6.6 of TS 23.502 [4].

- The EES send the EAS discovery response to EEC with the EAS endpoint information.

NOTE 3:  If the EAS instatiation is failed in step 4, the step 5 can be skipped, and the EES responds in EAS discovery response with an appropriate failure cause.

The XR application client/AC can access the instantiated EAS with the required computing ability and the required QoS information, the E2E KPI application requirements of XR application can be guaranteed.

7.x.3	Solution evaluation

Editor's note:	Solution evaluation is FFS.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TR 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[4]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[5]	3GPP TS 23.503: "Policy and charging control framework for the 5G System (5GS); Stage 2".

[x]	3GPP TS 23.558: " Architecture for enabling Edge Applications; Stage 2".

[y]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services; Stage 2".

[z]	3GPP TS 28.538: "Management and orchestration; Edge Computing Management".