3GPP TSG-SA WG6 Meeting #59	S6-240268

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0030 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Clarification on media delivery functionality in SEALDD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | As a general data delivery enabler, SEALDD should support the data transmission for all traffic types, including media type (e.g. media streaming, real-time media), Based on the functionality mapping in Annex D and the current specified SEALDD procedure, this paper focuses on the media delivery procedure mapping between SEALDD and SA4, and clarify how the specified SEALDD transmission procedure to support the general media streaming defined in SA4. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper adds the related description to capture the procedure mapping between SEALDD and SA4, regarding of uplink media delivery and downlink media delivery. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | How the current specified SEALDD data transmission procedure support media delivery in SA4, is still unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.2, 7.4.4, 9.2.2.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

7.4.2	SEALDD-UU

Reference point between SEALDD client and SEALDD server used to transfer data content and exchange information for SEALDD service provisioning, control, reporting etc.

For media delivery, the SEALDD-UU utilizes the M4 and M5 interfaces of 5GMS according to 3GPP TS 26.501 [14], and the RTC-4 and RTC-5 interafaces of 5G RTC system according to 3GPP TS 26.506 [15].

# * * * Next Change * * * *

7.4.4	SEALDD-S

Reference point between SEALDD server and VAL server to enable northbound server side API exposed by SEALDD server to VAL server for data delivery and SEALDD service provisioning, control, reporting etc.

For media delivery, the SEALDD-S utilizes the M1 and M2 interfaces of 5GMS according to 3GPP TS 26.501 [14], and the RTC-1 interface of 5G RTC system according to 3GPP TS 26.506 [15].

# * * * Next Change * * * *

9.2.2.x	SEALDD enabled media transmission connection establishment procedure

When utilizing the SEALDD to deliver the media stream, the SEALDD server acts the 5GMS/RTC AF and 5GMS/RTC AS, the SEALDD client acts as the media session handler and the media player/framework, and follws the support the uplink media transmission and the downlink media transmission defined in TS 26.501 [14] and TS 26.506 [15].