3GPP TSG-SA WG6 Meeting #59	S6-240289

Athens, Greece 26th February – 1st March 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0015 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct edge load analytics |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2024-01-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | EES ID cannot be used for ADAES to contact the corresponding EES. EES endpoint is needed and consumer of ADAE service should have both EES ID and endpoint available. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Change EES ID to EES information which include EES ID and endpoint. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The current edge load analytics procedure is incomplete and unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.3.2, 8.8.3.4, 8.8.3.6, 8.8.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 8.8.3.2	Edge analytics subscription request

Table 8.8.3.2-1 describes information elements for the edge analytics subscription request from the VAL server / Consumer to the ADAE server.

Table 8.8.3.2-1: Edge analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, EAS). |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “edge performance analytics”. |
| Analytics type | M | Whether analytics event is about prediction or statistics. |
| Destination EAS information | O (NOTE) | This identifier shows the destination EAS information including destination EAS ID and destination EAS endpoint for which the analytics subscription applies. |
| Destination EES information | O (NOTE) | This identifier shows the destination EES information including destination EES ID and destination EES endpoint for which the analytics subscription applies. |
| DNN/DNAI | O (NOTE) | DNN or DNAIs information for which the subscription applies. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the subscription request. |
| NOTE: At least one of these shall be present. |  |  |

* * * Next Change * * * *

#### 8.8.3.4	Edge data collection subscription request

Table 8.8.3.4-1 describes information elements for the edge data collection subscription request from the ADAE server to the Data Producer at the EDN or the A-DCCF, or from ADAE server to ADAE client.

Table 8.8.3.4-1: Data collection subscription request

| Information element | Status | Description |
| --- | --- | --- |
| ADAE server ID | M | The identifier of the ADAE server |
| Data Collection Event ID | M | The identifier of the data collection event |
| Data Collection requirements | M | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Analytics ID | O | The identifier of the analytics event, for which the data collection is needed. |
| List of Data Producer IDs | O | In case when this request is performed via A-DCCF, then the list of Data Producer IDs is needed. |
| Destination EAS information | O (NOTE) | This identifier shows the destination EAS information including destination EAS ID and destination EAS endpoint for which the analytics subscription applies. |
| Destination EES information | O (NOTE) | This identifier shows the destination EES information including destination EES ID and destination EES endpoint for which the analytics subscription applies. |
| DNN/DNAI | O (NOTE) | DNN or DNAIs information for which the subscription applies. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Area of Interest | O | The geographical or service area for which the requirement request applies. |
| Time validity | O | The time validity of the request. |
| NOTE: At least one of these shall be present. |  |  |

* * * Next Change * * * *

#### 8.8.3.6	Data Notification

Table 8.8.3.6-1 describes information elements for the Data Notification from the Data Producer to the ADAE server.

Table 8.8.3.6-1: Data notification

| Information element | Status | Description |
| --- | --- | --- |
| Data Collection Event ID | M | The identifier of the data collection event. |
| Data Producer ID | M | The identity of Data Producer. |
| Destination EAS information | O (NOTE) | This identifier shows the destination EAS information including destination EAS ID and destination EAS endpoint for which the analytics subscription applies. |
| Destination EES information | O (NOTE) | This identifier shows the destination EES information including destination EES ID and destination EES endpoint for which the analytics subscription applies. |
| DNN/DNAI | O (NOTE) | DNN or DNAIs information for which the subscription applies. |
| Analytics ID | O | The identifier of the analytics event. |
| Data Type | M | The type of reported data samples which can be network data, application data, edge data, or different granularities / abstraction of data (e.g. real time, non-real time). This also indicates whether data are offline (from A-ADRF or not). |
| Data Output | M | The reported data, which can be inform of measurements or offline/historical data on the requested parameter based on subscription. Such data can be per EDN or per DNAI or per EAS/EES load statistics and edge computational resource utilization stats for a given time and area of interest. |
| NOTE: At least one of these shall be present based on the data collection event. |  |  |

* * * Next Change * * * *

#### 8.8.3.8	Get analytics data request

Table 8.8.3.8-1 describes information elements for the Get analytics data request from the analytics consumer to the ADAE server.

Table 8.8.3.8-1: Get analytics data request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, EAS, EES). |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “edge performance analytics”. |
| Analytics filter information | M | Filter information for the analytics event |
| Analytics type | M | Whether analytics event is about prediction or statistics. |
| Destination EASs information | O (NOTE) | This identifier provides the list of destination EASs information including destination EAS ID and destination EAS endpoint for which the analytics request applies. |
| Destination EESs information | O (NOTE) | This identifier provides the list of destination EESs information including destination EES ID and destination EES endpoint for which the analytics request applies. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Time duration | O | Time duration since when analytics data is required. |
| NOTE: At least one of these shall be present |  |  |

* * * Next Change * * * *