3GPP TSG-SA WG6 Meeting #59	S6-240339

Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0023 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Location Accuracy Analytics and API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT3 have LS sent to SA6 for questions on Issues related to ADAE server APIs (S6-240021/C3-240225), and on A-DCCF, A-ADRF and ADAES services (S6-240019/C3-240095). This paper proposes to update the Location Accuracy Analytics and API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Updated the description for “Analytics type” in clauses 8.5.3.2 and 8.5.3.6. - Added “Reporting requirements” to the table in clause 8.5.3.2. - Added “Data collection requirements” to the table in clause 8.5.3.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect stage 2 specification for Location Accuracy Analytics and API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.5.3.2, 8.5.3.4, 8.5.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR addresses LS S6-240021/C3-240225 and S6-240019/C3-240095. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

#### 8.5.3.2	Location accuracy analytics subscription request

Table 8.5.3.2-1 describes information elements for the location accuracy analytics subscription request from the VAL server to the ADAE server.

Table 8.5.3.2-1: Location accuracy analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the VAL server. |
| Analytics ID | M | The identifier of the location accuracy analytics event. This ID can be for example "location accuracy prediction" or "location accuracy sustainability" depending on the expected outcome. |
| Analytics type | M | The type of analytics for the event, which shall include category (e.g. statistics or predictive analytics), mode (e.g. offline or online), and may include indication on ML-enabled analytics. |
| VAL UE ID(s) or Group ID | M | The identity of the VAL UE(s) or group of UEs for which the analytics subscription applies |
| VAL service ID | O | The identifier of the VAL service for which location accuracy analytics is requested. |
| Location accuracy requirements | M | The accuracy threshold and VAL requirements. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the subscription request. |
| UE mobility / route information | O | Information on the target UE or group UE mobility including the expected route/set of waypoints. |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |

* * * Next change * * * *

#### 8.5.3.4	Location accuracy data request

Table 8.5.3.4-1 describes information elements for the location accuracy data request from the ADAE server to the A-ADRF.

Table 8.5.3.4-1: Location accuracy data request

| Information element | Status | Description |
| --- | --- | --- |
| ADAE server ID | M | The identifier of the ADAE server |
| Analytics ID | M | The identifier of the analytics event |
| List of VAL UE IDs and addresses | M | The VAL UE(s) identifiers and IP address(es) for which the data/analytics apply |
| VAL service ID | O | The service ID, in case of requesting historical data for a particular VAL service. |
| Reporting configuration | O | The configuration for data reporting. This requirement may include e.g. the frequency of reporting (periodic), the reporting periodicity in case of periodic, and reporting thresholds, whether data abstraction is needed or not. |
| Data collection requirements | O | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Area of Interest | O | The geographical or service area for which the subscription request applies |
| Time validity | O | The time validity of the request |

* * * Next change * * * *

#### 8.5.3.6	Location accuracy analytics notification

Table 8.5.3.6-1 describes information elements for the location accuracy analytics notification from the ADAE server to the VAL server.

Table 8.5.3.6-1: Location accuracy analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| Analytics Type | O | The type of analytics based on the event, which may include category (e.g. statistics or predictive analytics), mode (e.g. offline or online), and indication on ML-enabled analytics. |
| Analytics Output | M | The predictive or statistical parameter, which can be: - A predicted or expected location accuracy change (downgrade or upgrade) for a particular VAL service or UEs - the location accuracy sustainability for a VAL service or UE/group of UEs over a given time horizon/area |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

* * * End of changes * * * *