3GPP TSG-SA WG6 Meeting #59	S6-240340

Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0024 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Service API Analytics and API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT3 have LS sent to SA6 for questions on Issues related to ADAE server APIs (S6-240021/C3-240225), and on A-DCCF, A-ADRF and ADAES services (S6-240019/C3-240095). This paper proposes to update the Service API Analytics and API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Added “Analytics type” to the tables in clauses 8.6.3.2 and 8.6.3.6. - Added “Analytics ID” and “Reporting requirements” to clause 8.6.3.2. - Removed “Exposure level requirement” from the table in clause 8.6.3.2. - Changed “Reporting format configuration” to “Reporting configuration” and updated the description in clause 8.6.3.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect stage 2 specification for Service API Analytics and API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.6.3.2, 8.6.3.4, 8.6.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR addresses LS S6-240021/C3-240225 and S6-240019/C3-240095. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

The following information flows are specified for service API analytics based on 8.6.2.

#### 8.6.3.2	Service API event subscription request

Table 8.6.3.2-1 describes information elements for the service API event subscription request from the consumer (VAL server, API provider) to the ADAE server.

Table 8.6.3.2-1: Service API event subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The information to determine the identity of the subscribing entity (consumer). |
| Service API information | M | The service API name or type. |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “service API analytics”. |
| Analytics type | M | The type of analytics for the event, which shall include category (e.g. statistics or predictive analytics), and may include mode (e.g. offline or online), and indication on ML-enabled analytics. |
| Criteria | M | The event criteria include event type information relevant to the prediction or stats on the number of failure API invocations, API availability, frequency and occurrence of API version changes, API location changes for the target API, etc. |
| Time Validity and horizon | O | Time validity of the subscription request and time horizon for the predictions. |
| Area of interest | O | Geographical or topological area for which the subscription applies. |
| Notification reception information | O | The information of the subscribing entity for receiving the notifications for the event. |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |

* * * Next change * * * *

#### 8.6.3.4	Historical service API logs request

Table 8.6.3.4-1 describes information elements for the historical service API logs request from the ADAE server to the A-ADRF.

Table 8.6.3.4-1: Historical service API logs request

| Information element | Status | Description |
| --- | --- | --- |
| Service API log requestor information | M | Identity information of the originated application querying service API log request. |
| ADAES ID | M | Identity information of the ADAES. |
| Service ID or UE ID | M | Identity of the application service or UE for which the historical API invocations apply. |
| Target API(s) information | M | Information on target API or list of target APIs (name or type). |
| >Query information | O | List of query filters such as invoker's ID and IP address, service API name and version, input parameters, and invocation result. |
| > API aggregation abstraction flag | O | What type of aggregation or abstraction/filtering needs to be applied. |
| Reporting configuration | O | The configuration for the logs reporting. This requirement may include e.g. the frequency of reporting (periodic), the reporting periodicity in case of periodic, and reporting thresholds, whether data abstraction is needed or not. |
| Area of validity | O | The geographical area for which the request applies. |
| Time validity | O | The time validity for the request. |
| Exposure level requirement | O | The level of exposure requirement (e.g. permissions on the logs like read/write/delete) for the logs to be exposed. |

* * * Next change * * * *

#### 8.6.3.6	Service API analytics notification

Table 8.6.3.5-1 describes information elements for the service API analytics notification to the subscriber/consumer from the ADAE server.

Table 8.6.3.6-1: Service API analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The information to determine the identity of the subscribing entity (consumer) |
| Service API information | M | The service API name or type for which analytics apply |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “service API analytics”. |
| Analytics type | O | The type of analytics based on the event, which may include category (e.g. statistics or predictive analytics), mode (e.g. offline or online), and indication on ML-enabled analytics. |
| Analytics Output | M | Stats or predictions based on abstracted or anonymized API logs (for example number of failure API invocations, API availability, frequency and occurrence of API version changes, API location changes for the target API, API throttling events, number of API invocations for a given area and time, API load statistics for a given edge network, etc) |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |
| Area of validity | O | Geographical or topological area for which the analytics apply. |

* * * End of changes * * * *