3GPP TSG-SA WG6 Meeting #59	S6-240336

Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0020 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Application Performance Analytics and API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT3 have LS sent to SA6 for questions on Issues related to ADAE server APIs (S6-240021/C3-240225), and on A-DCCF, A-ADRF and ADAES services (S6-240019/C3-240095). This paper proposes to update the Application Performance Analytics and API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Updated the description for “Analytics type” in clauses 8.2.4.2 and 8.2.4.7. - Changed “Exposure level requirement” to “Reporting requirements” and updated the corresponding description in clause 8.2.4.2. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect stage 2 specification for Application Performance Analytics and API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.2.4.2, 8.2.4.7 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR addresses LS S6-240021/C3-240225. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

#### 8.2.4.2	VAL performance analytics subscription request

Table 8.2.4.2-1 describes information elements for the VAL performance analytics subscription request from the VAL server / Consumer to the ADAE server or from ADAE server to ADAE client.

Table 8.2.4.2-1: VAL performance analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The identifier of the analytics consumer |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “VAL server performance analytics” for procedure in 8.2.2, or “VAL session performance analytics” for procedure in 8.2.3. |
| Analytics type | M | The type of analytics for the event, which shall include category (e.g. statistics or predictive analytics), and may include mode (e.g. offline or online), and indication on ML-enabled analytics. |
| VAL service ID | M | The identifier of the VAL service for which analytics subscription apply. |
| Target VAL UE ID(s) | O | The VAL UE(s) for which the analytics subscription applies. |
| Target VAL server ID | O | If consumer is different from the VAL server, this identifier shows the target VAL server for which the analytics subscription applies (for procedure in 8.2.2). |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction. |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the subscription request |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |

* * * Next change * * * *

#### 8.2.4.7	Analytics Notification

Table 8.2.4.7-1 describes information elements for the Analytics Notification from the ADAE server to the VAL server / Consumer.

Table 8.2.4.7-1: Analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “VAL server performance analytics” for procedure in 8.2.2, or “VAL session performance analytics” for procedure in 8.2.3. |
| Analytics Type | O | The type of analytics based on the event, which may include category (e.g. statistics or predictive analytics), mode (e.g. offline or online), and indication on ML-enabled analytics. |
| Analytics Output | M | The predictive or statistical parameter, which can be: - A VAL server predicted or expected performance change or sustainability - A VAL session predicted or expected performance change of sustainability |
| Confidence level | O (see NOTE) | The achieved confidence level. |
| Time horizon | O (see NOTE) | The time horizon for predictive analytics. |
| > Start time | O | The start time point of predictive validity. If omitted, the default value is the current time. |
| > End time | M | The end time point of predictive validity. |
| NOTE: These information elements shall be provided for the predictive analytics. |  |  |

* * * End of changes * * * *