3GPP TSG-SA WG6 Meeting #59	S6-240342

Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0026 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Edge Load Analytics and API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT3 have LS sent to SA6 for questions on Issues related to ADAE server APIs (S6-240021/C3-240225), and on A-DCCF, A-ADRF and ADAES services (S6-240019/C3-240095). This paper proposes to update the Edge Load Analytics and API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Updated the figure of Request-response model for edge load analytics, and the discription of step 2 in clause 8.8.2.2. - Updated the description for “Analytics type” in clauses 8.8.3.2, 8.8.3.7, 8.8.3.8, and 8.8.3.9. - Added “Reporting requirements” to the table in clause 8.8.3.2. - Removed “Analytics filter information” from the table in clause 8.8.3.8. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect stage 2 specification for Edge Load Analytics and API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.8.2.2, 8.8.3.2, 8.8.3.7, 8.8.3.8, 8.8.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR addresses LS S6-240021/C3-240225. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

#### 8.8.2.2	Request-response model

Figure 8.8.2.2-1 illustrates the procedure for the analytics consumer to request analytics data of the application server(s) from the ADAE server.

Figure 8.8.2.2-1: ADAES support for edge analytics

1.	The analytics consumer sends a request message to the ADAE server to receive analytics data for one or more application servers. The request message includes the identity of the analytics consumer, security credential(s) for authorization and verification, identity of all the application server for which analytics data is requested, type of analytics data, time duration since when analytics data is required.

2.	Upon receiving the request, the ADAE server authenticates and authorizes the analytics consumer. If the analytics consumer is authorized, the ADAE server may get the analytics by performing steps 3 to 11 of clause 8.8.2.1. The ADAE server sends a response message including the statistical and predictive analytics of the edge performance/load for the edge platform or EES/EAS for the requested duration period (if the time duration is available).

* * * Next change * * * *

#### 8.8.3.2	Edge analytics subscription request

Table 8.8.3.2-1 describes information elements for the edge analytics subscription request from the VAL server / Consumer to the ADAE server.

Table 8.8.3.2-1: Edge analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, EAS). |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “edge performance analytics”. |
| Analytics type | M | The type of analytics for the event, which shall include category (e.g. statistics or predictive analytics), and may include mode (e.g. offline or online), and indication on ML-enabled analytics. |
| Destination EAS information | O (NOTE) | This identifier shows the destination EAS information including destination EAS ID and destination EAS address for which the analytics subscription applies. |
| Destination EES ID | O (NOTE) | This identifier shows the destination EES for which the analytics subscription applies. |
| DNN/DNAI | O (NOTE) | DNN or DNAIs information for which the subscription applies. |
| Target data producer profile criteria | O | Characteristics of the data producers to be used. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the subscription request. |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |
| NOTE: At least one of these shall be present. |  |  |

* * * Next change * * * *

#### 8.8.3.7	Edge analytics Notification

Table 8.8.3.7-1 describes information elements for the Edge analytics Notification from the ADAE server to the VAL server / Consumer.

Table 8.8.3.7-1: Edge analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| Analytics Type | O | The type of analytics based on the event, which may include category (e.g. statistics or predictive analytics), mode (e.g. offline or online), and indication on ML-enabled analytics. |
| Analytics Output | M | The predictive or statistical parameter, which can be stats or prediction related to the edge performance/load for the edge platform or EES/EAS for a given area/time and based on the event type. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

* * * Next change * * * *

#### 8.8.3.8	Get analytics data request

Table 8.8.3.8-1 describes information elements for the Get analytics data request from the analytics consumer to the ADAE server.

Table 8.8.3.8-1: Get analytics data request

| Information element | Status | Description |
| --- | --- | --- |
| Analytics Consumer ID | M | The identifier of the analytics consumer (VAL server, EAS, EES). |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “edge performance analytics”. |
| Analytics type | M | The type of analytics, which shall include category (e.g. statistics or predictive analytics), and may include mode (e.g. offline or online), and indication on ML-enabled analytics. |
| Destination EASs information | O (NOTE) | This identifier provides the list of destination EASs information including destination EAS ID and destination EAS address for which the analytics request applies. |
| Destination EES IDs | O (NOTE) | This identifier provides the list of destination EESs for which the analytics request applies. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction). |
| Time duration | O | Time duration since when analytics data is required. |
| NOTE: At least one of these shall be present |  |  |

* * * Next change * * * *

#### 8.8.3.9	Get analytics data response

Table 8.8.3.9-1 describes information elements for the edge analytics subscription response from the ADAE server to the consumer.

Table 8.8.3.9-1: Edge analytics subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result of the analytics data request (positive or negative acknowledgement). |
| Analytics ID | O | The identifier of the analytics event. |
| Analytics Type | O | The type of analytics, which shall include category (e.g. statistics or predictive analytics), and may include mode (e.g. offline or online), and indication on ML-enabled analytics. |
| Analytics Output | O | The predictive or statistical parameter, which can be stats or prediction related to the edge performance/load for the edge platform or EES/EAS for a given area/time and based on the event type. |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

* * * End of changes * * * *