3GPP TSG-SA WG6 Meeting #59	S6-240341

Athens, Greece, 26th February – 1st March 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0025 | rev | - | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to Slice Usage Pattern Analytics and API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT3 have LS sent to SA6 for questions on Issues related to ADAE server APIs (S6-240021/C3-240225), and on A-DCCF, A-ADRF and ADAES services (S6-240019/C3-240095). This paper proposes to update the Slice Usage Pattern Analytics and API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Updated the description for “Analytics type” in clause 8.7.4.2. - Added “Analytics type” to the table in clause 8.7.4.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect stage 2 specification for Slice Usage Pattern Analytics and API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.7.4.2, 8.7.4.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR addresses LS S6-240021/C3-240225. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

#### 8.7.4.2	Network slice usage pattern analytics subscription request

Table 8.7.4.2-1 describes information elements for the network slice usage pattern analytics subscription request from the analytics consumer (VAL server / NSCE server) to the ADAE server.

Table 8.7.4.2-1: Network slice usage pattern analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| Consumer ID | M | The identifier of the analytics consumer |
| Analytics ID | M | The identifier of the analytics event. This ID can be for example “Network slice usage pattern analytics” |
| Analytics type | M | The type of analytics for the event, which shall include category (e.g. statistics or predictive analytics), mode (e.g. offline or online), and may include indication on ML-enabled analytics. |
| Analytics filter information | M | Filter information for the analytics event |
| >Slice identifier | M | The identifier of the target slice or slice instance, i.e. S-NSSAI. |
| >Slice requirement | O | The requirement of network requirements or updated requirements when the network slice was created. The GST defined by GSMA (see clause 2.2 in [10]) and the performance requirements defined in clause 7 TS 22.261 [11] are all considered as input for the network slice related requirements. |
| >DNN | O | The target DNN for which the request applies |
| >Target VAL UE ID(s) | O | The VAL UE(s) for which the analytics subscription applies |
| >Target VAL server ID | O | If consumer is different from the VAL server, this identifier shows the target VAL server for which the analytics subscription applies (for procedure in 8.2.2) |
| >Area of Interest | O | The geographical or service area for which the subscription request applies |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction. |
| Time validity | O | The time validity of the request |
| Interest time period of the historical data | O | Interest time period of the historical data (e.g. last year), |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |

* * * Next change * * * *

#### 8.7.4.4	Network slice usage pattern analytics notification

Table 8.7.4.4-1 describes information elements for the network slice usage pattern analytics notification from the ADAE server to the analytics Consumer.

Table 8.7.4.4-1: Network slice usage pattern analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. |
| > Slice usage pattern analytics | M | Analytics of network slice usage pattern (e.g., periodicity of slice usage peak). |
| Analytics type | O | The type of analytics based on the event, which may include category (e.g. statistics or predictive analytics), mode (e.g. offline or online), and indication on ML-enabled analytics. |

* * * End of changes * * * *