3GPP TSG-SA WG6 Meeting #59	S6-240354

Athens, Greece, 26th February – 1st March 2024	(was S6-24xxxx)

Source:	Convida Wireless, LLC

Title:	Discussion paper: AIML

Agenda Item:	8.3

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

Quang Ly, ly.quang@convidawireless.com

# Abstract:

Based on SA6 #58 in Chicago discussions, some clarifications of the generic AIML process/workflow and of the scope of AIMLAPP layer are necessary in order to be able to integrate/ harmonize the various KIs and solutions.

This discussion paper has two parts: The first part discusses the relationship between the two architectural models in solution 1. Companion contributions are providing a minimal set of changes in the existing text for alignment based on these observations and proposals.

The second part discusses how the AIML functionality discussed so far in the TR aligns with industry-established concepts, and then uses those external references to make some proposals. The corresponding TR proposals are also represented via companion contributions.

### Part 1: On the relationship between MTME-enabled ADAES and AIML enabler.

Observation 1a: The two architectural models in solution 1 are not used consistently thought the specification.

Observation 1b: AIML use cases discussed for AIMLApp work in Rel-19 include the following aspects:

- AIML support is to be provided broadly, including for VAL, SEAL, edge, etc. functionality

- AIML support for data analytics is one of the earliest, broadest, and most significant AIML uses. As such an AIML-to-ADAES functional link has been acknowledged, and it is within study scope to determine how to best use, enable and specify such a functional link.

- AIML support to functionality OTHER than ADAES may be best provided stand-alone, rather than as a ADAES “add-on” for multiple reasons including: logical separation, clarity of specification, ease of deployment, etc.

Observation 1c. Rel-18 ADAES functionality has been specified loosely aligned to NWDAF; the ADAES functionality is currently strongly tied to analytics ID.

Observation 1d. Machine Learning enhancements to NWDAF have formalized in SA2 an architecture with multiple logical functions, including analytics (AnLF) and model training (MTLF).

Observation 1e: To provide ADAES with machine learning capabilities, the initial support identified to be necessary was that of direct model training and management, hence the MTME-enhanced ADAE functional model was agreed. (This is in alignment with 1d.)  Given that the ADAES functionality is tied to analytics ID, the MTME enhancement is optimized for analytics ID- specific functionality which includes machine learning (this is in alignment with 1c).

Proposal 1: Based on observations above it is proposed to clarify the architecture in TR 23.700-82 solution 1 as follows:

- Confirm that both the 8.1.2.2 and the 8.1.2.3 options are valid and can co-exist by deleting or re-phrasing some Editor Notes.

- Update solution 4, which is specific for ML-enabled ADAE, i.e. MTME -enhanced ADAES, to clearly show its link to clause 8.1.2.2

- Update solution 5 to clarify that it applies to both of the architectural options.

### Part 2: On the support of AIML workflow by AIML enabler.

To be able to determine how several of the TR concepts and solutions fit together, we looked at some established models of ML processing. The figure below pertains to an established ML-enabled VAL service (see ML lifecycle architecture diagram - Machine Learning Lens (amazon.com). In current literature, ML processing is described as a workflow starting with the identification of a business goal for an ML application, which in turn is used to frame the ML problem. To implement the corresponding ML application, data scientists identify data and features pertinent to the business goal select appropriate ML model(s) to train the data and to deploy and monitor the ML model’s performance.

Figure 1. ML Workflow

Observation 2a: The current TR text aligns with enablement of the “AIML workflow” above as follows:

- Several ML operations (e.g. data preparation, training, etc.) have been referenced in the TR in a manner consistent to this industry definition.

- An workflow preparation phase (corresponding to the first 2 steps in the figure)  has been introduced in the TR as information provisioning (i.e. of model profile, etc.)

- The following profiles have been introduced: model profile, UE ML client profile. This is consistent with the use of profiles (e.g. EEC, EAS, EES, etc.) in SA6 edge work, for describing entities, capabilities, and  related functionality.

AIML may be performed in centralized, distributed, and/or federated manners. In both centralized and distributed training, a central dataset is provided e.g. to train ML models in centralized or distributed nodes, respectively. A VAL service offers AIML functionality by offering service users (e.g. data scientists) access and visibility into the AIML workflow and dataset-related functionality.

Federated learning (FL), on the other hand, is training ML models in a federated manner where datasets exist on individual FL clients and model parameters from each FL clients are aggregated into a global model at a FL server. Since the contents of datasets are local in each FL client and VAL service users do not have visibility into the dataset, it is important to expose information about the AIML workflow to the service users.

SA6 AIML enablement requires terminology to describe distribution of processing, differently than or in addition to distribution of entities.

Observation 2b: For further alignment and to bring consistency between solutions, “AIML workflow” and related concepts are needed in the TR to clarify the functionality.

For example:

AIML job: An instance of an AIML workflow based on a specific set of  identified business goals, identified ML problem, etc. An ML job may include the execution of SA6-specified AIML operations e.g. model distribution, data collection, model training, etc. An ML job may be divided into more granular AIML tasks and is described by an AIML job profile.

NOTE: The definition above is proposed only for discussion purposes. It is meant to clarify how the companion contributions fit together with and help clarify existing TR concepts.

Machine learning related to 3GPP-defined analytics IDs (i.e. MTME-enhanced ADAES) also uses the generic blocks composing the workflow above, In addition, AIMLApp  enabler functionality for generic use requires VAL service users to be able to monitor and affect the AIML workflow.

Observation 2c: As opposed to services provided by MTME-enhanced ADAES, the AIMLApp enabler provides value-add by exposing information about AIML workflow status and ML models in relationship to the learning requirements and to the performance of the ML model.

Proposal 2: It is proposed to adopt a framework for exposure of AIML information to allow VAL service users to interact with the AIMLAPP layer to have visibility into the AIML workflow, tying information pre-provisioned for an AIML job  (i.e. model profile, etc.), ML operations (e.g. AIML registration, data preparation, training, etc.) and the producers/ consumers profiles.

Contributions implementing each of the proposals have been provided as inputs to SA6 #59.

# Proposal

It is proposed to agree the contributions related with the following:

Proposal 1: Based on observations above it is proposed to clarify the architecture in TR 23.700-82 solution 1 as follows:

- Confirm that both the 8.1.2.2 and the 8.1.2.3 options are valid and can co-exist by deleting or re-phrasing some ENs.

- Update solution 4, which is specific for ML-enabled ADAE, i.e. MTME -enhanced ADAES, to clearly show its link to clause 8.1.2.2

- Update solution 5 to clarify that it applies to both of the architectural options.

Proposal 2: It is proposed to adopt a framework for exposure of AIML information to allow VAL service users to interact with the AIMLAPP layer to have visibility into the AIML workflow, tying information pre-provisioned for an AIML job  (i.e. model profile, etc.), ML operations (e.g. AIML registration, data preparation, training, etc.) and the producers/ consumers profiles.