3GPP TSG-SA WG6 Meeting #59	S6-240357

Athens, Greece, 26th February – 1st March 2024	(was S6-24xxxx)

Source:	Convida Wireless, LLC

Title:	Update to solution #5 AI/ML model management

Spec:	3GPP TS 23.700-82 V0.2.0

Agenda item:	8.3

Document for:	Approval

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

Lu Liu, liu.lu@convidawireless.com

1. Introduction

This contribution alignes solution 4 with updates to solution 1.

2. Reason for Change

Updated terms to be consistent with the updated architecture.

3. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.2.0.

# *** First Change ***

## 8.5	Solution #5: AI/ML model management

### 8.5.1	General

The following clauses specify procedures of AI/ML model management.

### 8.5.2	AI/ML model information storage procedure

Figure 8.5.2-1: AI/ML model information storage procedure

1.	The model repository consumer (e.g.  MTME-enhanced ADAES or AIML enablement server) sends an AI/ML model information storage request to the model repository to store a model. The model included in the request can be one trained by the repository consumer or its information can be provisioned to the repository consumer. The request contains information of the AI/ML model (or a model profile) such as the analytics ID(s) for which the model can be used, requirements of using the model (e.g., required computing power), information of the training data with which the model is trained, metadata, etc.

2.	Upon receiving the request, the model repository processes the request and records information of the AI/ML model (e.g., by creating an AI/ML model profile). The model repository sends a response to the model repository consumer (e.g., MTME-enhanced ADAES or AIML enablement server) with an identifier of the created AI/ML model profile.

NOTE 1:	The model repository may be implemented by A-ADRF.

NOTE 2:	It is to be determined in the evaluation phase which entities are can have direct interaction with the model repository, e.g., AI/ML enablement server/client and/ or MTME-enhanced ADAE server/client, etc. This procedure is assumed to also enable other AI/ML service consumers to also access this functionality indirectly, via these enablers with direct access.

### 8.5.3	AI/ML model information discovery procedure

Figure 8.5.3-1: AI/ML model information discovery procedure

1.	The model repository consumer (e.g., VAL server) sends an AI/ML model information discovery request to the model repository. The request contains filter information of the AI/ML model such as the analytics ID(s) associated with the model, requirements of the model (e.g., required computing power), information of the training data, etc. If ADAE is the AI/ML model consumer, then step 1 is skipped.

2.	Upon receiving the discovery request, the model repository processes the request and sends the response message to the model repository consumer. The response includes information of the discovered AI/ML model. Optionally, the model repository can initiate a model transfer process to the consumer.

NOTE:	It is to be determined in the evaluation phase which entities are can have direct interaction with the model repository, e.g., AI/ML enablement server/client and/ or MTME-enhanced ADAE server/client, etc. This procedure is assumed to also enable other AI/ML service consumers to access this functionality indirectly, via these enablers with direct access.

# *** End of Changes ***