3GPP TSG-SA WG6 Meeting #59	S6-240358

Athens, Greece, 26th February – 1st March 2024	(was S6-24xxxx)

Source:	Convida Wireless, LLC

Title:	Updates to solutions 6, 7, and 8

Spec:	3GPP TS 23.700-82 V0.2.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

This contribution updates solutions 6, 7, and 8 to add missing sections and corresponding API definitions, removal of editor’s notes, and clarifications to the procedure descriptions.

2. Reason for Change

For solution 6, the editor’s note in 8.6.1 was removed due to the addition of assumption 3. Figure 8.6.1-1 was updated to clarify a VAL server is making the request and updates to text description were made for clarification. Sections 8.6.2 to 8.6.4 from the template were added and updates were made to section 8.6.3.

For solution 7, the editor’s note in 8.7.1 was removed due to the addition of assumption 3. Sections 8.7.2 to 8.7.4 from the template were added and updates were made to section 8.7.3. Clarifications were made to the procedure description.

For solution 8, the editor’s note in 8.8.1 was removed as it does not depend on other procedures. The information about the AIML client capabilities is grouped together into an AIML client profile, which is provisioned to the AIML enablement client as clarify by the update to assumption 3. Sections 8.8.2 to 8.8.4 from the template were added and updates were made to section 8.8.3. Clarifications were made to the procedure description.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.2.0.

# *** First Change ***

## 8.6 	Solution #6: AIML enablement client selection

### 8.6.1	Solution description

The following clauses specify procedures, information flows, and APIs for Key Issue #3 to support AIML enablement client selection.

Assumptions:

1.	The proposed solution is based on client-server architecture for federated learning.

2.	The VAL server has already discovered and received a list of AIML enablement clients that are suitable and have available data for a particular AIML operation.

3.	The discovery operation may be performed as described in solution 7 to find a list of AIML enablement clients.

Figure 8.6.1-1: AIML enablement client selection procedure

1.	A VAL server sends a request to an AIML enablement server to select a list of AIML enablement clients that have been discovered to meet the requirements for AIML operations. The AIML enablement client selection request includes the VAL identifier, security credentials, application identifier, and a list of AIML client IDs for inclusion into an AIML set.

Editor's Note: The IEs in the request are FFS. It is also FFS how the request parameters are used in relationship to other member selection policy provisioned to the AIML enabler layer.

2.	The AIML enablement server validates the selection request and verifies the security credentials provided in the request. The AIML enablement server further performs authentication and authorization checks to determine if the requestor is able to create an AIML set. If authorized, the AIML enablement server creates an AIML set with the provided list of AIML enablement clients as members and assigns an AIML identifier for the set.

NOTE: The AIML enabler server can reuse SEAL group management for any necessary group management.

3.	The AIML enablement server sends an AIML enablement client selection response that includes the status of the selection request and the assigned AIML set identifier.

### 8.6.2	Architecture Impacts

Editor’s note:	This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 8.6.3	Corresponding APIs

Table 8.6.3-1 shows the request sent by a VAL server to an AIML enablement server for the AIML enablement client selection procedure.

Table 8.6.3-1: Request for AIML enablement client selection procedure

| Information element | Status | Description |
| --- | --- | --- |
| VAL server identifier | M | The identifier of the VAL server. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Application identifier | O | An identifier for the AIML application |
| List of AIML client IDs | M | A list of AIML client IDs that was previously discovered for inclusion into an AIML client set. |

Table 8.6.3-2 shows the response sent by the AIML enablement server to the VAL server for the AIML enablement client selection procedure.

Table 8.6.3-2: Response for AIML enablement client selection procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| AIML client set identifier | M | An identifier to associate with the set of AIML enablement clients that the VAL server has provided in the request. The AIML client set can be updated by using this identifier. |

### 8.6.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 8.7 	Solution #7: AIML enablement client discovery

### 8.7.1	Solution description

The following clauses specify procedures, information flows, and APIs for Key Issue #3 to support AIML enablement client discovery.

Assumptions:

1.	The proposed solution is based on client-server architecture for federated learning.

2.	The AIML enablement server has a repository of AIML enablement clients and associated AIML client profiles.

3.	AIML enablement clients that support AIML operations have registered with the AIML enablement server and included their AIML client profile. The registration procedure may be performed as described in solution 8.

Figure 8.7.1-1: AIML client discovery procedure

1.	A VAL server sends a request to an AIML enablement server to discover AIML enablement clients that are available to participate in federated learning operations, e.g. is available and have required data to train an ML model. The AIML enablement client discovery request includes client/server identifier, security credentials, discovery criteria, and application identifier.

Editor's Note: Other IEs in the request are FFS. Whether the IEs are included in the AIML member selection policy is FFS.

2.	The AIML enablement server validates the discovery request and verifies the security credentials provided in the discovery request. The AIML enablement server further performs authentication and authorization checks to determine if the requestor is able to discover AIML enablement clients. If authorized, the AIML enablement server determines a list of AIML enablement clients that meets the discovery criteria.

NOTE: The AIML enablement server may provide list of target UE(s) and utilize Member UE selection assistance procedure from TS 23.502, section 4.15.13 to get assistance in selecting candidate UEs. The AIML enablement server then determines a list of UEs that fulfils the discovery criteria based on information received from AIML enablement client registrations.

Editor's Note: It is FFS whether a FL client repository function is necessary to enable this step.

3.	The AIML enablement server sends an AIML enablement client discovery response that includes the status of the discovery request, a list of AIML client profiles matching the discovery criteria, and the number of matching AIML client profiles.

### 8.7.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 8.7.3	Corresponding APIs

Table 8.7.3-1 shows the request sent by a VAL server to an AIML enablement server for the AIML enablement client discovery procedure.

Table 8.7.3-1: Request for AIML enablement client discovery procedure

| Information element | Status | Description |
| --- | --- | --- |
| VAL server identifier | M | The identifier of the VAL server. |
| Security credentials | M | Security credentials to authenticate and authorize the AIML enabler client. |
| Application identifier | O | An identifier for the AIML application |
| Discovery criteria | M | Discovery criteria for finding suitable AIML enablement clients for AIML operations: |
|  |  | > Requested ML models |
|  |  | > Requested AIML roles: role for data collection, data preparation, training, or inferencing |
|  |  | > Target location (for data collection purposes) |
|  |  | > Client capabilities such as available compute, memory, and storage that may be dedicated to AIML operations |
|  |  | > Dataset requirements that includes: >> dataset availability, including dataset size, age, features list, and data distribution of each feature >> Data capabilities such as data collection and supported data processing capabilities |

Table 8.7.3-2 shows the response sent by the AIML enablement server to the VAL server for the AIML enablement client discovery procedure.

Table 8.7.3-2: Response for AIML enablement client discovery procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| List of AIML client IDs | M | A list of AIML client IDs that matches the AIML discovery criteria. |
| Number of discovered AIML clients | O | The number of AIML client profiles that was found to match the AIML discovery criteria. The number indicates the potential AIML enablement clients that could participate in AIML operations. |

### 8.7.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

## 8.8 	Solution #8: AIML enablement client registration

### 8.8.1	Solution description

The following clauses specify procedures, information flows, and APIs for Key Issue #3 to support AIML enablement client registration.

Assumptions:

1.	The proposed solution is based on client-server architecture for federated learning.

2.	The AIML enablement client has been pre-configured or has discovered the address (e.g. URI) of the AIML enablement server.

3.  The AIML enablement client has been pre-configured with an AIML client profile.

Figure 8.8.1-1: AIML enablement client registration procedure

1.	The AIML enablement client sends a registration request to the AIML enablement server, the registration request includes a AIML enablement client identifier, security credentials, application identifier, and AIML client profile .

Editor's Note: Other IEs in the request are FFS. Whether the IEs are included in the AIML member selection policy is FFS.

2.	The AIML enablement server validates the registration request and verifies the security credentials. The AIML enablement server further performs an authentication and authorization check to determine if the AIML enablement client is permitted to register to the AIML enablement server and participate in federated learning operations. The AIML enablement server assigns an identifier for the AIML client profile to associate it with the AIML enablement client.

Editor's Note: It is FFS whether a FL client repository function is necessary to enable this step.

3.	The AIML enablement server returns an AIML enablement registration response to the AIML enablement client with the status of the registration request and the identifier associated with the AIML client profile.

### 8.8.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 8.8.3	Corresponding APIs

Table 8.8.3-1 shows the request sent by an AIML enablement client to an AIML enablement server for the AIML enablement client registration procedure.

Table 8.8.3-1: Request for AIML enablement client registration procedure

| Information element | Status | Description |
| --- | --- | --- |
| AIML enablement client identifier | M | The identifier of the AI/ML enabler client. |
| Security credentials | M | Security credentials to authenticate and authorize the AIML enabler client. |
| Application identifier | O | An identifier for the AIML application |
| AIML client profile | M | Information about the capability of the AIML enablement client to support AIML operations: |
|  |  | > Supported ML models |
|  |  | > Supported AIML roles |
|  |  | > Availability schedule |
|  |  | > Client capabilities such as available compute, memory, and storage that may be dedicated to AIML operations |
|  |  | > Dataset availability, including dataset size, age, features list, and data distribution of each feature |
|  |  | > Data capabilities such as data collection and supported data processing capabilities |

Table 8.8.3-2 shows the response sent by the AIML enablement server to the AIML enablement client for the AIML enablement client registration procedure.

Table 8.8.3-2: Response for AIML enablement client registration procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| AIML client profile ID | O | An identifier for the AIML client profile |

### 8.8.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# *** End of Changes ***