3GPP TSG-SA WG6 Meeting #59	S6-240364

Athens, Greece, 26th February – 1st March 2024

Source:	Convida Wireless, LLC

Title:	Spatial Anchor Subscribe Notify

Spec:	3GPP TR 23.700-21 V0.1.0

Agenda item:	8.4

Document for:	Approval

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

1. Introduction

This contribution provides a solution to KI #1 - Enabler support for managing spatial anchors.

2. Reason for Change

Proposes a solution to enable VAL servers and MetaApp client with the capability to subscribe to spatial anchors on a MetaApp server and receive notifications based on spatial anchor events.

3. Proposal

It is proposed to agree the following changes to 3GPP 23.700-21 V0.1.0.

# *** All new text ***

## 7.x	Solution #x: Spatial Anchor Subscribe / Notify

### 7.x.1	Solution description

The following clauses specify procedures, information flows, and APIs for Key Issue #1 to support spatial anchor subscribe and notify operations.

### 7.x.1.1	Subscribe

Assumptions:

- The spatial anchor has already been created on a MetaApp server by a VAL server.

- The spatial anchor has already been discovered by a MetaApp client or another VAL server.

Figure 7.x.1.1-1: Spatial anchor subscribe procedure

- A VAL server or MetaApp client sends a spatial anchor subscribe request to a MetaApp server. The request includes information elements defined in Table 7.x.3-1.

- The MetaApp server performs an authorization check to verify whether the requester has authorization to perform spatial anchor subscribe requests on the MetaApp server.

- Upon successful authorization, the MetaApp server creates the subscription to the specified spatial anchor and begins to monitor for occurrence of spatial anchor events defined within the spatial anchor subscription.

- The MetaApp server sends a spatial anchor subscribe response to the requester. The response includes information elements defined in Table 7.x.3-2.

### 7.x.1.2	Notify

The following clauses specify procedures, information flows, and APIs for Key Issue #1 to support spatial anchor notifications.

Assumptions:

- The spatial anchor has already been created on a MetaApp server by a VAL server.

- The spatial anchor has already been discovered by a MetaApp client or another VAL server.

- The spatial anchor has already been subscribed to by a MetaApp client or another VAL server.

Figure 7.x.1.2-1: Spatial anchor notification procedure

- The MetaApp server detects a spatial anchor event has occurred which matches a spatial anchor event defined within a spatial anchor subscription.

- The MetaApp server sends a spatial anchor notification to the notification target (e.g., VAL server or MetaApp client) of the spatial anchor subscription. The notification includes information elements defined in Table 7.x.3-3.

### 7.x.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 7.x.3	Corresponding APIs

Table 7.x.3-1 shows the request sent by a requester to a MetaApp server to subscribe to a spatial anchor.

Table 7.x.3-1: Spatial anchor subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Requester identity | M | The identity of the VAL server or MetaApp client performing the request. |
| UE identifier | O | The identifier of the UE if the requester is a MetaApp client. |
| Requester security credentials | M | Security credentials of the requester. |
| Spatial anchor ID list | O | List of identifiers of the spatial anchors applicable to this subscription.. |
| Spatial anchor event IDs | O | Spatial anchor event(s) of interest to the requester. For example, - change in location of the spatial anchor, - UE(s) entering the service area of the spatial anchor, - UE(s) exiting the service area of the spatial anchor, - creation of new spatial anchors of interest. |
| Notification target address | M | Notification Target Address where the notification is to be sent by the MetaApp Server. |

Table 7.x.3-2 shows the response sent by the MetaApp server to the requester for the spatial anchor subscribe request.

Table 7.x.3-2: Spatial anchor subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| Subscription ID | M | Subscription identifier corresponding to the spatial anchor subscription. |
| Expiration time | O | Indicates the expiration time of the subscription. To maintain an active subscription, a subscription update is required before the expiration time. If the Expiration time IE is not included, it indicates that the subscription never expires. |

Table 7.x.3-3 shows the spatial anchor notification sent by the MetaApp server to the notification target.

Table 7.x.3-2: Spatial anchor notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the spatial anchor subscription. |
| Timestamp | O | The timestamp of the event. |
| Notification details | M | Information related to the spatial anchor notification |
| > Spatial anchor ID | M | Identifier of the spatial anchor applicable to this notification. |
| > Spatial anchor event ID | M | Identifier of the spatial anchor event |
| > Event details | O | Information related to the spatial anchor event. For example, - updated location of the spatial anchor, - ID(s) of UE(s) detected entering the service area of the spatial anchor, - ID(s) of UE(s) detected exiting the service area of the spatial anchor, - ID(s) of newly created spatial anchors. |

### 7.x.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# *** End of Changes ***