3GPP TSG-SA WG6 Meeting #59	S6-240366

Athens, Greece, 26th February – 1st March 2024

Source:	Convida Wireless, LLC

Title:	Multi-modal SEALDD policy configuration

Spec:	3GPP TR 23.700-23 V0.1.0

Agenda item:	8.5

Document for:	Approval

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

1. Introduction

This contribution provides a solution for KI #2 - E2E Multi-Modal Communication Flows.

2. Reason for Change

To effectively support multi-modal application traffic flows with SEALDD, SEALDD servers and clients must have awareness of the individual application flows having multi-modal dependencies on each another. To provide this awareness, the contribution proposes extending the current SEALDD policy functionality to support multi-modal SEALDD policy functionality.

3. Proposal

It is proposed to agree the following changes to 3GPP 23.700-23 V0.1.0.

# *** All new text ***

## 7.x	Solution #X: Multi-modal SEALDD Policy Configuration

### 7.x.1	Architecture Impacts

This solution adds support for multi-modal SEALDD policy configuration functionality. This solution builds upon the existing SEALDD policy configuration functionality supported in clause 9.10 of TS 23.433 and therefore is well-aligned with the existing SEALDD architecture.

### 7.x.2	Solution description

This solution is targeting KI #2: E2E Multi-Modal Communication Flows and specifies a multi-modal SEALDD policy configuration procedure and information flows. In this procedure, the VAL server is a specific server for configuring multi-modal SEALDD policies and is different from VAL servers used for VAL application processing.

Note, corresponding multi-modal SEALDD policy update and delete procedures will be defined during the normative phase.

#### 7.x.2.1	Multi-modal SEALDD policy configuration

Figure 7.x.2.1-1 illustrates the procedure for a VAL server to configure a multi-modal SEALDD policy to a SEALDD server.

Figure 7.x.2.1-1: Multi-modal SEALDD policy configuration

1.  The VAL server sends the multi-modal SEALDD policy configuration request to the SEALDD server. The request includes information elements as defined in Table 7.x.3-1.

2.  The SEALDD server performs an authorization check to verify whether the VAL server can be accepted/authorized to configure the multi-modal SEALDD policy.

3.  Upon successful authorization, the SEALDD server stores the multi-modal SEALDD policy for later use (e.g. for synchronization of the individual application flows of the multi-modal flow) and replies to the VAL server with the multi-modal SEALDD policy configuration response.

### 7.x.3	Information flows

#### 7.x.3.1	Multi-modal SEALDD policy configuration request

Table 7.x.3.1-1 describes the information flow from the VAL server to the SEALDD server for requesting the multi-modal SEALDD policy configuration.

Table 7.x.3.1-1: Multi-modal SEALDD policy configuration request

| Information element | Status | Description |
| --- | --- | --- |
| Multi-modal VAL flows list | M | List of individual VAL flows applicable to this multi-modal SEALDD policy. |
| > Application traffic identifiers | M | Identifier of the application traffic for this individual VAL flow (e.g. VAL server ID, VAL service ID) |
| > VAL UE identity | O | Identifier of the VAL UE for this individual VAL flow |
| > SEALDD policy | M | SEALDD policy associated with this individual VAL flow as defined in Table 9.10.3.1-1 of TS 23.433 |
| Multi-modal SEALDD policy | M | Multi-modal SEALDD policy applicable to Multi-modal VAL flows list. |
| > Burst policy | O (See NOTE 1) | Policy used by the SEALDD layer to determine multi-modal data burst operations to perform (e.g., data bursts comprising messages from different VAL flows). |
| > Synchronization policy | O (See NOTE 1) | Policy used by the SEALDD layer to determine multi-modal synchronization operations to perform (e.g., synchronizing arrival times of messages across different VAL flows). |
| > Measurement policy | O (See NOTE 1) | Policy used by the SEALDD layer to determine multi-modal measurements to perform to support multi-modal burst and synchronization operations. (e.g., latency or jitter of multi-modal data bursts) |
| > Policy expiration time | O | Indicates the validity period of the policy |
| NOTE 1: At least one of these multi-modal SEALDD policies shall be present in the request |  |  |

#### 7.x.3.2	Multi-modal SEALDD policy configuration response

Table 7.x.3.2-1 describes the information flow from the SEALDD server to the VAL server for responding to the multi-modal SEALDD policy configuration.

Table 7.x.3.2-1: Multi-modal SEALDD policy configuration response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| > Configuration ID | O (See NOTE) | Identifier of the multi-modal SEALDD policy configuration. |
| > Expiration time | O (See NOTE) | Indicates the expiration time of the configured multi-modal SEALDD policy |
| NOTE: These IEs are used for the successful case for multi-modal SEALDD policy configuration request. |  |  |

### 7.x.3	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# *** End of Changes ***