3GPP TSG-SA WG6 Meeting #59	S6-240378

Athens, Greece February 26th – 1st March 2024

Source:	Ericsson

Title:	Introducing AIML Enablement server initiated AIML service advertisement

Spec:	3GPP TR 23.700-82 v0.2.0

Agenda item:	8.3

Document for:	Approval

Contact:	ashish.sanjay.sharma@ericsson.com

1. Introduction

This pCR proposes a solution for the AIML Enablement server to advertise AIML services or AIML tasks to assist interested AIML Enablement consumers for the AIML task or services.

2. Reason for Change

AIML enablement client discovery and selection is critical for any AIML process. There are use cases initiated by consumers like the AIML client performing some model offload and requiring AIMLenabler server assistance to discover other consumers who can perform the task on model offload. To join any task, it is essential for other consumers to discover the AIML service first and associated information about the task like task requirements. The AIML enabler layer can facilitate such discovery of tasks for the consumers.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.2.0.

* * * First Change * * * *

## 8.x	Solution #x: AIML Enablement server initiated AIML services advertisements

### 8.x.1	General

The following clauses specify procedures, information flows and APIs for Key issues 3,7 to enable the AIML Enablement server initiated AIML services advertisements procedure

Pre-conditions:

1.	The AIML services and their information is available on the AIML Enablement server.

### 8.x.2	Procedures

Figure 8.x.2-1: AIML Enablement server initiated AIML services advertisements procedure

- The AIML Enablement server sends the AIML service advertisement subscribe request to the AIML Enablement consumer(e.g VAL server, AIML Enablement client). This request contains AIML service info like AIML type, AIML type requirements, AIML service duration, AIML service incentive info, AIML task type, AIML task KPI like latency, and AIML task requirements. The AIML service incentive info defines the available reward for the AIML service.

The AIML type include Horizontal Federated Learning, Vertical Federated Learning, Transfer Learning, Split Learning and AIML type requriements define the network(e.g. bandwidth, jitter, latency) and compute capabilities for the AIML type. The AIML task requirements include task related compute capabilities and network capabilities. The type of AIML tasks include model transfer, model training, model inference, model offload, model split. The AIML Enablement server sends the subscription request to one or more than one AIML Enablement consumers registered in the AIML server. The request contains immediate, periodic or event-based reporting conditions for the notification message. The periodic reporting is based on the configured intervals and event-based reporting is based on the configured triggers for the AIML Enablement consumer like changes in battery level, change in mode of power, changes in network parameters, changes in compute utilization etc.

Editor's Note 1: Whether the AIML Enablement server uses group based delivery or broadcast or unicast delivery to AIML Enablement consumer is FFS.

Editor's Note 2: other IEs in the request are FFS.

- The AIML Enablement consumer acknowledges the subscription request and provides the subscription response indicating the result of the acknowledgement.

- The AIML enablement consumer sends a AIML service advertisement notification to the AIML Enablement server. The notify message provides the AIML Enablement consumer identifier(URI/ID/IPaddress), supported AIML type and indication to participate in the supported AIML type, supported AIML task, supported compute capability, supported network capability, service availability which is defined by time duration for the AIML service.

Editor’s Note 3: other IEs in the response are FFS.

* * * Next Change * * * *

* * * End of Changes * * * *