3GPP TSG-SA WG6 Meeting #59	S6-240381

(revision of S6-234043)

Athens, Greece 26th February – 01st March  2024

Source:	Sateliot, CATT, TNO, Novamint

Title:	Pseudo-CR on new KI on satellite access with discontinuous coverage

Spec:	3GPP TR 23.700-01 v0.1.0

Agenda item:	8.6

Document for:	Approval

Contact:	Giovanni Rigazzi (giovanni.rigazzi@sateliot.com), Ramon Ferrús (ramon.ferrus@sateliot.com)

1. Introduction

This contribution proposes a new KI on the impacts to application enablers for supporting satellite access with discontinuous coverage for Cellular IoT (CIoT) / Machine Type Communication (MTC) services.

2. Reason for Change

In Rel-17 and Rel-18, 3GPP SA2 specified architecture and solutions considering the service requirements in TS 22.261 including the support of discontinuous coverage which is inherent to low density NGSO (Non-Geostationary Orbit) satellite deployments. Any specific information related to discontinuous coverage provided by SA2 or other sources may be utilized by the application enablement layer or mission critical applications.

The study of the impacts to application enablers for supporting discontinuous satellite coverage is under the scope of the SID“Study on application enablement for Satellite access enabled 5G Services” approved in SA#102 (SP-231738):

The SA6 objectives of this study item include the following:

1.	Application Enablers:

a.	Study the impacts to application enablers for supporting satellite access, such as:

i)	Use of satellite access characteristics to optimise application layer enablement behaviour

ii)	Discontinuous satellite coverage: Support the predictable discontinuous /intermittent connectivity patterns between UEs and AS/AFs for IoT services with satellite access e.g. asset tracking, critical infra asset monitoring delivered through low density NGSO satellite constellations.

iii)	Support delivery of content from a content caching application by taking advantage of satellites in supporting ubiquitous service.

b.	Identify key issues, deployment models and develop solutions for application enablers with satellite access based on impacts identified in 1.a.

In line with this objective, a discussion on the impacts to application enablers for supporting satellite access with discontinuous coverage is addressed in the following.

A. Notion of discontinuous coverage and illustrative patterns

A description of what discontinuous coverage is can be found in TS 36.300: “As an NTN payload moves on a specified orbit, for example in case of a NGSO satellite, the NTN payload beam(s) coverage area may move and cover different portions of a geographical area due to the orbital movement of the NTN payload. As a consequence, a UE located in the concerned geographical area may experience a situation of discontinuous coverage, due to e.g. a sparse satellite constellation deployment.”

An illustration of a discontinuous coverage pattern for a given UE location and a single example LEO satellite at a nominal 600 km altitude is represented in Figure 1 (source: R2-2107453). The figure shows the flyovers where the satellite passes above 30 degrees elevation as seen from the UE location. The rise time of the satellite is plotted on the x axis against the visibility duration on the y axis. The results do not consider link budget aspects, only geometrical visibility above the given elevation angle. In the provided example, the satellite is visible from the UE location twice on most days, and occasionally three times, and the interval between passes varies between ~9 to 13.3 hours, with a mean of 11.9 hours. The median duration of the visibility window lasts around 220 seconds (3.6 minutes), with 90% of flyovers longer than 110 seconds.

This sort of discontinuous coverage pattern between the UE and a satellite can be predicted ahead of time for periods of days or even a few weeks with good accuracy (e.g. time errors of a few tens of seconds in estimating the starting time of the satellite pass when considering prediction windows of up to a few tens of days) by means of orbit propagation models and satellite ephemeris information (see R2-2206115 for prediction estimation errors using TLE and SGP4 propagator).

Figure 1: Flyovers and duration of visibility (≥ 30 degrees elevation) for a single LEO-600 km satellite (source: R2-2107453)

B. Support for discontinuous coverage at network level

Several mechanisms to cope with the discontinuous coverage nature of the service link has been included in Rel-17 and Rel-18 specs:

- Within the radio access network:

  - For IoT NTN access in E-UTRAN, as specified in TS 36.300, to enable the UE, in RRC_IDLE, to save power during periods of no coverage, the network provides NTN payload assistance information (e.g., ephemeris parameters, the start-time of upcoming NTN payload coverage) to enable the UE to predict when coverage will be provided by upcoming NTN payloads. Predicting out of coverage and in coverage is up to UE implementation.

  - In NR NTN access in NG-RAN, there is no specified support for discontinuous coverage over NR in RAN specifications in TS 38.300.

- Within the core network, support for discontinuous coverage has been specified for both EPS and 5GS. This support mainly comes in the form of optional enhancements for:

  - Mobility management and UE power saving optimization.

  - Coverage availability information provisioning to the UE, which can be provided by an external server (via a PDN Connection or SMS in EPS, and via a PDU session or SMS in 5GS).

    - Note: The protocol and format of satellite coverage availability information to be provisioned to the UE is not defined in Rel-17 and Rel-18 specifications, but some examples on the information that constitutes the satellite coverage availability information is defined in Annex N of TS 23.401 and in Annex Q of TS 23.501.

  - Coverage availability information provisioning to the MME/AMF, which may be provisioned by O&M.

    - NOTE:	In this release of the specification there is no support for provisioning of satellite coverage availability information to an MME/AMF from an AF.

  - Paging

  - Overload control

C. Impact of discontinuous coverage to the application layer for CIoT/MTC

Regardless of the above enhancements at network level which help the UE and the network to gracefully operate under a discontinuous coverage pattern, it becomes clear from Figure 1 that the discontinuous coverage nature of the service link may also have a direct impact on the behaviour of the application layer, which will experience/suffer the sort of intermittent connectivity pattern between the UE and an Application Server (AS)/Application Function (AF) residing on the network side.

Supplying information on the discontinuous coverage pattern or related services to the application layer will help applications to design themselves for handling discontinuity accordingly. For instance, if the discontinuous /intermittent connectivity pattern can be predicted in advance and exposed to the application layer either in the UE side or in the network side or in both, the application layer could use this information to properly organize and schedule data transfers between the UE and the AS/AF (e.g. some information flows can be given precedence during short connectivity intervals, bulky data exchanges can be deferred and planned during longer connectivity intervals, notifications on expected data delivery times can be provided, etc.).

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 v0.1.0.

* * * First Change * * * *

### 4.x	Key issue #x: Satellite access with discontinuous coverage

### 4.x.1	Description

In a network with satellite access, a UE may experience a situation of discontinuous coverage, due to e.g., a sparse NGSO satellite constellation deployment. An illustration of a discontinuous coverage pattern for a given UE location and a single example LEO satellite at a nominal 600 km altitude is represented in Figure 4.x.2-1 (reproduced from R2-2107453). The figure shows the flyovers where the satellite passes above 30 degrees elevation as seen from the UE location. The rise time of the satellite is plotted on the x axis against the visibility duration on the y axis. The results do not consider link budget aspects, only geometrical visibility above the given elevation angle. In the provided example, the satellite is visible from the UE location twice on most days, and occasionally three times, and the interval between passes varies between ~9 to 13.3 hours, with a mean of 11.9 hours. The median duration of the visibility window lasts around 220 seconds (3.6 minutes), with 90% of flyovers longer than 110 seconds.

This sort of discontinuous coverage pattern between the UE and a satellite can be predicted ahead of time for periods of days or even a few weeks with good accuracy (e.g. time errors of a few tens of seconds in estimating the starting time of the satellite pass when considering prediction windows of up to a few tens of days) by means of orbit propagation models and satellite ephemeris information (see R2-2206115 for prediction estimation errors using TLE and SGP4 propagator).

Figure 4.x.2-1: Flyovers and duration of visibility (≥ 30 degrees elevation) for a single LEO-600 km satellite (source: R2-2107453)

From a network perspective, support to cope with the discontinuous coverage nature of the service link in sparse constellations has been specified in Rel-17 and Rel-18. This support includes optional enhancements within the radio access network (e.g., satellite ephemeris parameters are broadcast in SIBs for satellite pass predictions for IoT NTN access) and within the core network (e.g., mobility management and power saving optimization, coverage availability information provisioning, paging, overload control). Coverage availability information provisioning to the UE and to the MME/AF entities have been also been idenfified, though no specific protocol or format have been defined in the specifications. All these enhancements help the UE and the network to gracefully operate under a discontinuous coverage pattern.

However, regardless of these enhancements at the network level, the discontinuous coverage nature of the service link results into an intermittent connectivity pattern between the UE and the Application Server (AS)/Application Function (AF) that has also a direct impact on the behaviour of the application.

Supplying information on the discontinuous coverage pattern or related services to the application layer will help applications to design themselves for handling discontinuity accordingly. For instance, if the discontinuous /intermittent connectivity pattern can be predicted in advance and exposed to the application layer either in the UE side or in the network side or in both, the application layer could use this information to properly organize and schedule data transfers between the UE and the AS/AF (e.g. some information flows can be given precedence during short connectivity intervals, bulky data exchanges can be deferred and planned during longer connectivity intervals, notifications on expected data delivery times can be provided, etc.).

It is therefore necessary to study if some support to handle discontinuous coverage at application layer should be introduced in existing or a new application enabler.

### 4.x.2	Open issues

To support the discontinuous satellite coverage in application enabler, the following aspects need to be studied:

-    Whether and how the information on discontinuous satellite coverage needs to be exposed to the application layer (e.g. semantics, service-based interfaces, etc.).

- 	Whether and how the information on discontinuous satellite coverage needs to managed (e.g. transferred between the UE side and the network side, processed, stored, updated, etc.) within the application enablement layer.

-    	Whether and how to consolidate the format of information on discontinuous satellite coverage from different vertical application servers/operators.

-	Whether and how to support the above functions based on the existing or a new application enabler.

* * * End of Change * * * *

* * * Next Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[x]	3GPP TS 22.261: "Service requirements for next generation new services and markets; Stage 1".

[y]	3GPP TS 23.501: "System Architecture for the 5G System; Stage 2".

[x1]	3GPP TS 23.401: “General Packet Radio Service (GPRS) enhancements for Evolved Universal Terrestrial Radio Access Network (E-UTRAN) access; Stage 2”

* * * End of Change * * * *