3GPP TSG-SA WG6 Meeting #59	S6-240383

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.434 | CR | 0293 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancements to NRM reliable transmission API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eSEAL2 |  |  |  |  |  | Date: |  |  | 19-02-2024 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Fix support for requesting user plane end point information to enhance the reliable transmission procedure Addition of SEAL-X4 reference point between NRM server and SEALDD server. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The NRM server is a control plane entitiy which configures the 5GS for the reliable transmission on behalf of the VAL server. For VAL server to transfer application data, it requires the user plane endpoint resource information. In the exisitng procedure it is missing. The proposed solution enables VAL server to request the NRM server to provide the user plane end point information for the application layer data transmssion along with configuring the 5GS for reliable transfer. To establish the connection between NRM server and SEALDD server, SEAL-X4 reference point is defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The VAL server cannot transfer data without user plane end point resource information. The NRM server cannot communicate with SEALDD server over SEAL-X interface |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 14.3.10, 14.3.2.67,14.3.2.68, 6.5.2.9.x(new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 14.3.10	AF influence URSP procedure for reliable transmission

#### 14.3.10.1	General

The procedures related to the AF influence URSP for reliable transmission are described in the following subclauses.

#### 14.3.10.2	AF influence URSP procedure for reliable transmission

Pre-conditions:

1.	The SEALDD server (or VAL server) has decided to use reliable transmission for a specific SEALDD client (or VAL client) and has got the UE address or UE ID.

Figure 14.3.10.2-1: AF influence URSP procedure for reliable transmission

1.	The NRM server receives from the SEALDD server (or VAL server) about the request for reliable transmission service with the application descriptors for the two redundant transmission paths. The SEALDD client's current UE address or UE ID is also provided to identify the affected UE. The request also includes indication to request for user plane resource information.

2.	After receiving the request from SEALDD server (or VAL server), the NRM server associates the available DNN and S-NSSAI information for the two application descriptors. NRM server triggers via N5/N33 with the AF guidance for URSP procedure as described in 3GPP TS 23.502 [11] clause 4.15.6.10. The AF request includes the application traffic descriptors and the associated DNN, S-NSSAI, and also includes UE address or UE ID received in step 1.

3.	NRM server sends response about reliable transmission service. The response may contain information related to the user plane resource.

NOTE: The user plane endpoint resource information is only applicable if the NRM server receives request from VAL server for procedure defined in clause 9.x.3.1 TS 23.433

* * * Next Change * * * *

#### 14.3.2.67	Reliable transmission request

Table 14.3.2.67-1 describes the information flow for reliable transmission request from SEALDD server (or VAL server) to the NRM server.

Table 14.3.2.67-1: Reliable transmission request

| Information element | Status | Description |
| --- | --- | --- |
| Requester Identity | M | The identity of the SEALDD server (or VAL server) performing the request. |
| Application descriptors | O | A pair of applic traffic descriptors (e.g. address, port, transport layer protocol) of the SEALDD server (or single application descriptor of the VAL server), used to establish redundant transmission paths. |
| UE ID | O | The identity of one or more than one VAL UE. |
| UE address | O | The address of one or more than one VAL UE |
| Request for user plane endpoint resource information | O | Indicates yes or no for requesting user plane endpoint resource information |

#### 14.3.2.68	Reliable transmission response

Table 14.3.2.68-1 describes the information flow for the reliable transmission response from NRM server to SEALDD server (or VAL server).

Table 14.3.2.68-1: Reliable transmission response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | The result indicates success or failure of the reliable transmission request operation. |
| Application traffic user plane resource information | O | Indicate the end point(address-port) for the user plane application traffic like SEALDD server address/port to receive application data transfer |

* * * Next Change * * * *

##### 6.5.2.9.x	Reference point SEAL-X4 (between the network resource management server and the SEAL Data Delivery server)

The SEAL-X4 reference point enables the network resource management server to interact with the SEAL Data Delivery server.

The SEAL-X4 reference point supports:

-	the network resource management server to configure the SEAL Data Delivery server for the user plane related information and use SEALDD services.

-	the SEALDD server uses NRM services like reliable transmission service, QoS monitoring, etc.

* * * End of Changes * * * *