3GPP TSG-SA6 Meeting #59	S6-240388

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.255 | CR | 0053 | rev | - | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Simultaneous link support for UTM-Navigated C2 and clarifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | KPN N.V. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | UASAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The recently incorporated details regarding simultaneous link support require additional clarification across various subsections within Section 7.4. Further elaboration is necessary on different aspects of the section. It is imperative to enhance the UTM-Navigated C2 communications by enabling dual-network support. This augmentation aims to bolster reliability in the communication framework. |  |  |  |  |  |  |  |  |
| Summary of change: |  | New information on simultaneous links has recently been introduced. We have contributed additional clarifications and made corrections to information flows. Furthermore, recognizing the enhanced reliability that dual network support brings to Network-Assisted C2 communications, we have extended this support to benefit UTM-Navigated C2 communications in this contribution. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The specification for simultaneous links is currently unclear, and the support for dual links in the UTM-Navigated C2 communication mode has not been established. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 7.4.1, 7.4.2.1, 7.4.2.2, 7.4.2.4, 7.4.3.1, 7.4.3.4, 7.4.3.5, 7.4.3.7, 7.4.3.8 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * *   First Change   * * *

### 7.4.1	General

This feature introduces the UAS application enablement services for supporting the selection and re-selection of C2 communication modes. In particular, the UAE layer provides support for the following operations:

-	Support the switch between two Network-Assisted C2 communications (via different subscriptions/networks) or between a Network-Assisted C2 communication and a Direct C2 communication (e.g. when the direct link becomes feasible/available, or when a UAV is moving towards BVLOS or has poor direct link conditions, etc.) as described in clause 7.4.2.4. For redundancy purposes, two links can be activated to work simultaneously or as active/standby.

-	Support the switch between the Network-Assisted/Direct C2 communication and UTM-Navigated C2 communication (e.g. for air traffic control, the UAV is approaching a No Drone Zone, and detected potential security threats, etc.) as described in clause 7.4.2.5.

-	Support the selection of the communication mode between: more than one C2 communication links, and among applicable C2 communication links, selecting a mode as the primary one as described in clause 7.4.2.3. If two links are activated to work simultaneously or as active/standby, these links can belong to the same or different networks (dual subscription functionality).

-	Activation for the support of the above operations in the UAE Server and in the UAE client performed using procedures described respectively in clause 7.4.2.1 and clause 7.4.2.2.

-	Support for C2 direct mode availability reporting to provide the awareness to the UAE server to switch to direct C2 communication if it is possible, as described in clause  7.4.2.4.

Below, the different procedures for C2 communication mode selection and switching are described using UAE Client assisted and UAE Server controlled based mechanisms. Such functionality is supported by means of policies delivered to the UAV/UAV-C via the UAE layer and assisting the dynamic switching of C2 modes.

* * *   Next Change   * * *

#### 7.4.2.1	Management of C2 mode selection / switching capability

This procedure manages the C2 mode selection/switching capability at the UAE server, based on an application request from UAS application specific server (which can be the USS/UTM) to manage the C2 operation modes (direct, network-assisted, UTM-Navigated) of C2 communication for a UAS.

Figure 7.4.2.1-1 illustrates the procedure where the UAE server receives an application request for managing the operation mode for C2 communications for a UAS.

Pre-condition:

-	The UAV has received its UAS ID from the UAS application specific server.

Figure 7.4.2.1-1: C2 operation mode management request / response

1.	The UAS application specific server sends to the UAE Server a C2 operation mode management request for managing the operation modes for the C2 communication for a UAS (consisting a UAV and a UAV-C) and to subscribe for UAE notifications. If the C2 operation mode management container is provided, the request shall include the allowed C2 communication modes, the primary C2 communication mode and the policy for C2 switching, and may include the secondary C2 communication mode and the C2 service area. If ProSe/PC5 is supported for direct C2 communications, such request may also include a C2 direct mode availability reporting requirement including the ProSe configuration information for direct C2 operation, the UAV and UAV-C IDs and addresses, and the time for which the monitoring of availability will apply. If Dual Network-Assisted C2 communication link is supported (dual subscription functionality), such request may also include a C2 active/standby configuration information for C2 connectivity of the UAV to select the specific network based on QoS or specific time and also the area for which the respective network connectivity should be valid. Such request may also include a C2 connectivity via both active links via different networks. If Dual UTM-Navigated C2 communications is supported (dual subscription functionality), such request may also include a C2 active/standby configuration information for C2 connectivity of the UAV to select the specific network based on QoS or specific time and also the area for which the respective network connectivity should be valid. Such request may also include a C2 connectivity via both active links via different networks.

2.	The UAE Server sends to the UAS application specific server a C2 operation mode management response with a positive or negative acknowledgement of the request, based on capability of UAE server to undertake this task.

3.	UAE server executes C2 communication modes configuration according to clause 7.4.2.2.

4.	After execution of C2 communication modes configuration, the UAE server notifies the UAS application specific server with a C2 operation mode management complete.

* * *   Next Change   * * *

#### 7.4.2.2	C2 communication modes configuration

This procedure enables the configuration of the UAE Client, based on an application request from UAS application specific server (which can be the USS/UTM) to manage the C2 operation modes (direct, network-assisted, UTM-Navigated) of C2 communication for a UAS. For redundancy purposes, two links can be activated to work simultaneously or as active/standby. These links can belong to the same or different networks (e.g., dual subscription functionality) see Figure B.2.1-3.

Figure 7.4.2.2-1 illustrates the C2 communication modes configuration procedure.

Pre-conditions:

1.	The UAS UEs are connected to 5GS and authenticated and authorized by UAS application specific server as specified in clause 5.2 of 3GPP TS 23.256 [4].

2.	UAE Server has established a UAE session with the respective UAE Clients as the UAE clients are successfully registered to the UAE server.

3.	UAE Server has performed the C2 mode selection/switching capability initiation as in clause 7.4.2.1.

Figure 7.4.2.2-1: C2 communication modes configuration

1.	The UAE Server sends a C2 communication modes configuration request including the UAS identifier. If the C2 operation mode management container is provided, the request shall include the allowed C2 communication modes (e.g., direct, network assisted, UTM-Navigated), the primary C2 communication mode and the policy for C2 switching, and may include the secondary C2 communication mode. The UAE server may configure multiple Network-Assisted C2 communication links if the UAE client has indicated this capability to the UAE server at the UAE layer registration and if the UAE server has received such configuration via procedure described in clause 7.4.2.1. The UAE server may configure multiple UTM-Navigated C2 communication links if the UAE client has indicated this capability to the UAE server at the UAE layer registration and if the UAE server has received such configuration via procedure described in clause 7.4.2.1. In the case of removal of C2 communication mode configuration parameters from the UAV or UAV-C, then the request shall only include the UAS identifier.

2.	The UAE Client stores or removes the C2 communication mode configuration parameters as per the information received in step 1.

3.	The UAE Client sends a C2 communication modes configuration response to the UAE Server.

* * *   Next Change   * * *

#### 7.4.2.4	UAE-layer assisted dynamic C2 mode switching

This procedure provides a mechanism for supporting dynamic switching between two Network-Assisted C2 communications (via different networks) or two UTM-Navigated C2 communications (via different networks) or between a direct and a network assisted C2 communication or between a direct/network-assisted and UTM-Navigated, which may be required while the UAV flight is ongoing, due to possible change of network conditions, expected location/mobility of the UAV, unpredictable events etc. If two links are activated to work simultaneously or as active/standby, these links can belong to the same or different networks (dual subscription functionality).

Figure 7.4.2.4-1 illustrates the procedure where the UAE server supports the dynamic C2 mode switching for network-assisted C2 communications.

Pre-conditions:

1.	UAE Server has activated the dynamic C2 mode switching capability, as described in clause 7.4.2.1

2.	UAE Server has subscribed for using SEAL/LMS services and has configured the location event reporting, based on 3GPP TS 23.434 [5]. If two links are configured to work simultaneously or as active/standby, SEAL/LMS must have access to both networks (using deployment option shown in Figure 8.2.2-2 of 3GPP TS 23.434 [5] - deployment of SEAL servers with connections to 3GPP network system in multiple PLMN operators (networks) domains).

3.	UAE Client has selected a C2 communication mode as described in clause 7.4.2.3, and UAV and UAV-C are engaged in C2 communication. Two links may have been activated to work simultaneously or as active/standby.

Figure 7.4.2.4-1: UAE-assisted dynamic C2 mode switching

1.	The UAE Client detects a condition for switching C2 communication mode based on local conditions (e.g. using the C2 communication mode switching policy) or based on a command from the UAS application specific server (as described in clause 7.4.2.5). A C2-related trigger event report is sent from the UAE Client of the UAV and/or the UAV-C to the UAE Server. The report shall include a command from the UAS application specific server or an application QoS attribute change (experienced or expected) e.g. based on the experienced packet delay or packet loss for the Uu links or the direct link (e.g. packet loss greater than a pre-defined threshold).

If a C2 direct mode availability reporting requirement exists based on clause 7.4.2.1 step 1, the detection of UAV/UAV-C(s) can be based on the PC5 discovery due to the configurations received due procedures in clause 7.4.2.1 and clause 7.4.2.2. In this case, the C2-related trigger event report message shall include also a C2 direct mode availability report including PC5 related configuration information.

2.	Additionally, the UAE Server receives a location report for the UAV/UAV-C by the SEAL's LM server. The report can be either periodical or event-based (e.g. UAV moving towards an area covered by a different cell or different operator), as specified in 3GPP TS 23.434 [5] SEAL's LM server procedures (UAE Server acting as a VAL server).

3.	The UAE Server determines the switching of the C2 mode from direct to network assisted or vice versa or to UTM-Navigated. If the switching is from direct to network assisted or vice versa, this is done by calculating the relative actual or expected UAV-to-UAV-C location, as well as other factors like QoS fulfilment/unfulfilment, augmented location, mobility/speed, direction, topography, weather conditions. In case of simultaneous links, UAE Server determines the switching of the C2 mode between two network assisted C2 communications or between two UTM-Navigated C2 communications.

4.	The UAE Server sends a C2 mode switching confirmation request to the UAS application specific server, which includes the UAS identifier as well as the cause for switching and the switching option (between two network assisted, between two UTM-Navigated, direct to network-assisted, network-assisted to direct or to UTM-Navigated). The UAE Server sends this request to obtain confirmation from the UAS application specific server before proceeding with switching to UTM-Navigated. This step is optional in the case of switching from direct to network assisted or vice versa.

5.	Conditional on Step 3, the UAE Server receives from the UAS application specific server a C2 mode switching confirmation response indicating a positive or negative result for the requested change.

6.	The UAE Server sends to the involved UAE Clients, a C2 operation mode switching message which provides an instruction to the UAV and UAV-C to switch between the networks, to network-assisted mode or to direct mode or to UTM-Navigated mode. The UAV and UAV-C start C2 communication using the indicated C2 communication mode (e.g., active link via network 1 or network 2, direct, network assisted, UTM-Navigated).

7.	If an emergency switch of the C2 communication is deemed necessary by the UAE Client (e.g. sudden loss of the active C2 link), the UAE Client changes the link prior to the steps 1-6, which are skipped. The UAE Clients send a C2 operation mode switching performed message to the UAE Server to confirm the switching of the C2 communication mode.

* * *   Next Change   * * *

#### 7.4.3.1	C2 operation mode management request

Table 7.4.3.1-1 describes the information flow C2 operation mode management request from the UAS application specific server to the UAE server.

Table 7.4.3.1-1: C2 operation mode management request

| Information element | Status | Description |
| --- | --- | --- |
| UASS ID | M | Identity of the UAS application specific server which requests the C2 operation mode management. This ID can be the USS/UTM identifier, when the UAS application specific server is the USS/UTM. |
| UAS ID | M | The identification of the UAS for which the C2 QoS management request applies. This could be in form of identifier for the UAS, e.g group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI |
| C2 operation mode management container (see NOTE 2) | O | The C2 operation mode management container consists of the requirements and policy for C2 operation mode management |
| > C2 operation mode management requirement | M | Identification of the type of the C2 mode switching to be supported by the UAE server. This can be either from direct to network-assisted C2, or from network-assisted to direct C2 or to UTM-Navigated. |
| > Allowed C2 communication modes | M | direct, network assisted, UTM-Navigated |
| > Primary C2 communication mode | M | Primary C2 communication mode (direct, network assisted) |
| > Secondary C2 communication mode | O | Secondary C2 communication mode (direct, network assisted) |
| > Policy of C2 switching | M | Parameters for C2 switching - QoS thresholds on active link - QoS thresholds on target link |
| > C2 service area | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the capability is active. |
| > C2 direct mode availability reporting requirement | O | A requirement for C2 direct mode reporting. |
| >> ProSe application codes | O | The ProSe codes used for direct C2 communications. The ProSe codes are used for the ProSe Direct Discovery as specified in [4]. |
| >>Time of validity | O | The time for which the C2 direct mode availability/feasibility checking applies. |
| >>Reporting configuration | O | The configuration of the reporting and periodicity/frequency of reporting required. |
| > Dual Network-Assisted C2 communication links | O | Support for multiple Network-Assisted C2 communication links. |
| >> Policy of C2 switching in case of Network-Assisted C2 communication link 1 | O | Parameters for C2 switching for Network-Assisted C2 communication link 1: - QoS threshold needed for C2 communication per subscription/network, QoS threshold needed for payload (user plane) communication per subscription/network. |
| >> Policy of C2 switching in case of Network-Assisted C2 communication link 2 | O | Parameters for C2 switching for Network-Assisted C2 communication link 2: - QoS threshold needed for C2 communication per subscription to activate simultaneously both subscriptions/networks, QoS threshold needed for payload (user plane) communication per subscription to activate simultaneously both subscriptions/networks, |
| >> C2 service area for Network-Assisted C2 communication link 1 | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the connectivity via subscription/network 1 is active. |
| >> C2 service area for Network-Assisted C2 communication link 2 | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the connectivity via subscription/network 2 is active. |
| > Dual UTM-Navigated C2 communication links | O | Support for multiple UTM-Navigated C2 communication links. |
| >> Policy of C2 switching in case of UTM-Navigated C2 communication link 1 | O | Parameters for C2 switching for UTM-Navigated C2 communication link 1: - QoS threshold needed for C2 communication per subscription/network, QoS threshold needed for payload (user plane) communication per subscription/network. |
| >> Policy of C2 switching in case of UTM-Navigated C2 communication link 2 | O | Parameters for C2 switching for UTM-Navigated C2 communication link 2: - QoS threshold needed for C2 communication per subscription to activate simultaneously both subscriptions/networks, QoS threshold needed for payload (user plane) communication per subscription to activate simultaneously both subscriptions/networks, |
| >> C2 service area for UTM-Navigated C2 communication link 1 | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the connectivity via subscription/network 1 is active. |
| >> C2 service area for UTM-Navigated C2 communication link 2 | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the connectivity via subscription/network 2 is active. |
| NOTE 1: Void NOTE 2: If C2 operation mode management container IE is not included, it indicates removal of the C2 operation mode management related information. |  |  |

* * *   Next Change   * * *

#### 7.4.3.4	C2-related trigger event report

Table 7.4.3.4-1 describes the information flow C2-related trigger event report from the UAE client (of the UAV or the UAV-C) to the UAE server.

Table 7.4.3.4-1: C2-related trigger event report

| Information element | Status | Description |
| --- | --- | --- |
| UAE client ID | M | The identifier of the UAE client which indicates the QoS downgrade |
| UAS UE ID(s) | O | The identifier of the UE ID(s) for which the PC5 connectivity is feasible/available.. |
| Application QoS-related event | M | The report including the expected or actual application QoS / QoE parameters which were change (i.e. latency, throughput, reliability, jitter) of Network-Assisted C2 communication link 1 or UTM-Navigated C2 communication link 1. The event configuration (thresholds, policies) is provided to the UAE client as described in clause 7.4.2.4. |
| Application QoS-related event of Network-Assisted C2 communication link 2 or UTM-Navigated C2 communication link 2 | O | The report including the expected or actual application QoS / QoE parameters which were changed (i.e. latency, throughput, reliability, jitter) of Network-Assisted C2 communication link 2 or UTM-Navigated C2 communication link 2. The event configuration (thresholds, policies) is provided to the UAE client as described in clause 7.4.2.4. |
| PC5 availability indication | O | The PC5 availability indication for the direct C2 mode. |
| > PC5 capabilities | O | The PC5/ProSe capabilities and configuration information of the involved UAS UEs. |
| > PC5 QoS/access information | O | The PC5 QoS (list of PQIs) and access related information (e.g. DRX cycles, resource pools). |
| > Time of validity | O | The time for which the direct mode is available. |

* * *   Next Change   * * *

#### 7.4.3.5	C2 mode switching confirmation request

Table 7.4.3.5-1 describes the information flow C2 mode switching confirmation request from the UAE server to the UAS application specific server.

Table 7.4.3.5-1: C2 mode switching confirmation request

| Information element | Status | Description |
| --- | --- | --- |
| UAE server ID | M | The identifier of the UAE server which requests the C2 mode switching confirmation from USS/UTM |
| UAS ID | M | The identification of the UAS. This could be in form of identifier for the UAS, e.g. group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI. |
| C2 operation mode switching type | M | The type of the C2 mode switching to be applied (network-assisted to network-assisted, UTM-Navigated to UTM-Navigated, direct to network-assisted, network-assisted to direct, or UTM-Navigated). |
| Switching cause | O | Cause information for initiating the switching (e.g. poor radio link quality) |

* * *   Next Change   * * *

#### 7.4.3.7	C2 operation mode switching

Table 7.4.3.7-1 describes the information flow C2 operation mode switching from the UAE server to the UAE client(s) of the affected UAS.

Table 7.4.3.7-1: C2 operation mode switching

| Information element | Status | Description |
| --- | --- | --- |
| UAE server ID | M | The identifier of the UAE server which instructs the UAS to apply the C2 mode switching. |
| C2 operation mode switching requirement | M | The type of the C2 mode switching to be applied (network-assisted to network-assisted, UTM-Navigated to UTM-Navigated, direct to network-assisted, or network-assisted to direct). |
| Time Validity | O | Time validity for the C2 switching requirement |
| Geographical Area | O | Area for which the C2 switching applies |

* * *   Next Change   * * *

#### 7.4.3.8	C2 communication modes configuration request

Table 7.4.3.8-1 describes the information flow C2 communication modes configuration request from the UAE server to the UAE client.

Table 7.4.3.8-1: C2 communication modes configuration request

| Information element | Status | Description |
| --- | --- | --- |
| UAS ID | M | The identification of the UAS for which the C2 QoS management request applies. This could be in form of identifier for the UAS, e.g. group ID; or collection of individual identifiers for the UAV and UAV-C, e.g. CAA level UAV ID, GPSI. |
| C2 operation mode management configuration (see NOTE 2) | O | The C2 operation mode management configuration information to be configured at the UAS |
| > C2 operation mode management requirement | M | Identification of the type of the C2 mode switching to be supported by the UAE server. This can be either from direct to network-assisted C2, or from network-assisted to direct C2 or to UTM-Navigated. |
| > Allowed C2 communication modes | M | direct, network assisted, UTM-Navigated |
| > Primary C2 communication mode | M | Primary C2 communication mode (direct, network assisted) |
| > Secondary C2 communication mode | O | Secondary C2 communication mode (direct, network assisted) |
| > Policy of C2 switching | M | Parameters for C2 switching - QoS thresholds on active link - QoS thresholds on target link |
| > Dual Network-Assisted C2 communication links | O | Support for multiple Network-Assisted C2 communication links. This information element may only be present if the UAE client has indicated this capability to the UAE server at the UAE layer registration. |
| >> Policy of C2 switching in case of Network-Assisted C2 communication link 1 | O | Parameters for C2 switching for Network-Assisted C2 communication link 1: - QoS threshold needed for C2 communication per subscription/network, QoS threshold needed for payload (user plane) communication per subscription/network. |
| >> Policy of C2 switching in case of Network-Assisted C2 communication link 2 | O | Parameters for C2 switching for Network-Assisted C2 communication link 2: - QoS threshold needed for C2 communication per subscription to activate simultaneously both subscriptions/networks, QoS threshold needed for payload (user plane) communication per subscription to activate simultaneously both subscriptions/networks. |
| >> C2 service area for Network-Assisted C2 communication link 1 | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the connectivity via subscription/network 1 is active. |
| >> C2 service area for Network-Assisted C2 communication link 2 | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the connectivity via subscription/network 2 is active. |
| > Dual UTM-Navigated C2 communication links | O | Support for multiple UTM-Navigated C2 communication links. This information element may only be present if the UAE client has indicated this capability to the UAE server at the UAE layer registration. |
| >> Policy of C2 switching in case of UTM-Navigated C2 communication link 1 | O | Parameters for C2 switching for UTM-Navigated C2 communication link 1: - QoS threshold needed for C2 communication per subscription/network, QoS threshold needed for payload (user plane) communication per subscription/network. |
| >> Policy of C2 switching in case of UTM-Navigated C2 communication link 2 | O | Parameters for C2 switching for UTM-Navigated C2 communication link 2: - QoS threshold needed for C2 communication per subscription to activate simultaneously both subscriptions/networks, QoS threshold needed for payload (user plane) communication per subscription to activate simultaneously both subscriptions/networks. |
| >> C2 service area for UTM-Navigated C2 communication link 1 | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the connectivity via subscription/network 1 is active. |
| >> C2 service area for UTM-Navigated C2 communication link 2 | O | The area where the C2 operation mode management request applies. This can be geographical area, or topological area in which the connectivity via subscription/network 2 is active. |
| NOTE 1: Void NOTE 2: If C2 operation mode management configuration IE is not included, it indicates removal of the C2 operation mode management configuration at the UAS ID. |  |  |

* * *   End of Change   * * *