3GPP TSG-SA6 Meeting #59	S6-240396

# Athens, Greece, 26th Feb 2024 - 1st Mar 2024

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0532 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC Group ID(s) for location subscription and cancellation and affiliation in Location information |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | HOME OFFICE |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Control Rooms and Dispatchers have requested that group based features are supported for Location in the MC architecture and location information is obtained from currently affiliated MC Users. This document is proposing to include some missing aspects in TS23.280, to facilitate this. In the crrent specification, the Alias feature does provide some group behaviour, but it does not convey the user’s current affliation status to a group. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes relate to adding Information Elements (IEs) to the information flow tables in section 10.9.2 of TS 23.280. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The group based, dynamic location reporting behaviour requested by the Control Rooms and Dispatchers of emergency services cannot be supported. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  |  | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  |  | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  |  | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

### 10.9.2	Information flows for location information

#### 10.9.2.2	Location information report

Table 10.9.2.2-1 describes the information flow from the location management client to the location management server for the location information reporting.

Table 10.9.2.2-1: Location information report (LMC – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service ID (see NOTE 4) | O | Identity of the requesting MC service user. |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the reporting MC service ID. |
| Functional alias | O | Functional alias that corresponds to the requesting MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service UE label | O | Generic name of the reporting MC service UE |
| Triggering event (see NOTE 3) | M | Identity of the event that triggered the sending of the report |
| Affiliation information | O | Currently affiliated group(s) that corresponds to the reporting MC Service ID and/or group IDs |
| Location Information (see NOTE 2) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 3: An on-demand request may be the triggering event. NOTE 4: In case of an on-demand request of an MC service user the MC service ID shall be provided. In case of an MC service server request or an event-triggered report, no MC service ID is provided. |  |  |

Table 10.9.2.2-2 describes the information flow from the location management server to the location management client for location information reporting.

Table 10.9.2.2-2: Location information report (LMS – LMC)

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service ID | M | Identity of the requesting MC service user. |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the reporting MC service ID. |
| Functional alias | O | Functional alias that corresponds to the requesting MC service user (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| MC service UE label | O | Generic name of the reporting MC service UE |
| Triggering event (see NOTE 3) | M | Identity of the event that triggered the sending of the report |
| Affiliation information | O | Currently affiliated group(s) that corresponds to the reporting MC Service ID and/or group IDs |
| Location Information (see NOTE 2) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 3: An on-demand request may be the triggering event. |  |  |

Table 10.9.2.2-3 describes the information flow from the location management server to the MC service server for location information reporting.

Table 10.9.2.2-3: Location information report (LMS – MC service server)

| Information element | Status | Description |
| --- | --- | --- |
| Set of MC service IDs | M | Set of identities of the reporting MC service user on the MC service UE (e.g. MCPTT ID, MCVideo ID, MCData ID) |
| Functional alias(es) (see NOTE 1) | O | Functional alias that corresponds to the MC service ID. |
| Triggering event (see NOTE 3) | M | Identity of the event that triggered the sending of the report |
| Affiliation information | O | Currently affiliated group(s) that corresponds to the reporting MC Service ID and/or group IDs |
| Location Information (see NOTE 2) | M | Location information of the individual MC service user |
| NOTE 1: Each functional alias corresponds to an individual MC service ID. NOTE 2: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. NOTE 3: An on-demand request may be the triggering event. |  |  |

Table 10.9.2.2-4 describe the information flow to support the handling of location information across MC systems.

Table 10.9.2.2-4: Location information report (LMS – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the reporting MC service user at partner MC system |
| Functional alias | O (see NOTE 1) | Functional alias of the reporting MC service user at partner MC system |
| MC service ID | M (see NOTE 2) | Identity of the requesting MC service user at primary MC system |
| Functional alias | O (see NOTE 2) | Functional alias of the requesting MC service user at primary MC system |
| MC service UE label | O | Generic name of the reporting MC service UE at partner MC system |
| Triggering event (see NOTE 3) | M | Identity of the event that triggered the sending of the report |
| Affiliation information | O | Currently affiliated group(s) that corresponds to the reporting MC Service ID and/or group IDs |
| Location Information (see NOTE 4) | M | Location information of the individual MC service user at partner MC system |
| NOTE 1: It shall be present if used with the Functional alias associated request. NOTE 2: Only present, if provided with the associated request. NOTE 3: An on-demand request may be the triggering event. NOTE 4: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. |  |  |

* * * Next Change * * * *

#### 10.9.2.5	Location information subscription request

Table 10.9.2.5-1 describes the information flow from the MC service server to the location management server for location information subscription request.

Table 10.9.2.5-1: Location information subscription request (MC service server – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID list/ MC Group ID/ MC Group ID list | M | List of MC service users whose location information is requested |
| Time between consecutive reports | M (see NOTE) | Indicates the interval time between consecutive reports. The provided time is to be used for all MC service IDs provided in the MC service ID list. |
| NOTE: If the interval time has a value of zero then the location management server will send the Location information notification immediately the location information report is received from the MC service user in the MC service ID list. |  |  |

Table 10.9.2.5-2 describes the information flow from the location management client to the location management server for location information subscription request.

Table 10.9.2.5-2: Location information subscription request (LMC – LMS)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the requesting MC service user |
| MC service ID list/ MC Group ID/ MC Group ID list | M | List of MC service users whose location information is requested |
| Time between consecutive reports | M (see NOTE) | Indicates the interval time between consecutive reports. The provided time is to be used for all MC service IDs provided in the MC service ID list. |
| NOTE: If the interval time has a value of zero then the location management server will send the Location information notification immediately the location information report is received from the MC service user in the MC service ID list. |  |  |

* * * Next Change * * * *

#### 10.9.2.7	Location information notification

Table 10.9.2.7-1 and Table 10.9.2.7-2 describe the information flows from the location management server to the MC service server and from the location management server to the location management client.

Table 10.9.2.7-1: Location information notification (LMS to MC service server)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user whose location information is being notified |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Affiliation information | O | Currently affiliated group(s) that corresponds to the notifying MC Service ID and/or group IDs |
| Location Information (see NOTE) | M | Location information |
| NOTE: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. |  |  |

Table 10.9.2.7-2: Location information notification (LMS to LMC)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user whose location information is being notified |
| Functional alias | O | Functional alias that corresponds to the MC service user whose location information is being notified |
| MC service ID | M | Identity of the MC service user subscribed to location information |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information (see NOTE) | M | Location information |
| MC service UE label | O | Generic name of the reporting MC service UE |
| Affiliation information | O | Currently affiliated group(s) that corresponds to the notifying MC Service ID and/or group IDs |
| NOTE: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. |  |  |

Table 10.9.2.7-3 describes the information flow from the location management server in the partner MC system to the location management server in the primary MC system.

Table 10.9.2.7-3: Location information notification (LMS to LMS)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID | M | Identity of the MC service user whose location information is being notified |
| Functional alias | O | Functional alias that corresponds to the MC service user whose location information is being notified |
| Triggering event | M | Identity of the event that triggered the sending of the notification |
| Location Information (see NOTE) | M | Location information |
| MC service UE label | O | Generic name of the reporting MC service UE |
| Affiliation information | O | Currently affiliated group(s) that corresponds to the notifying MC Service ID and/or group IDs |
| NOTE: This may contain multiple sets of elements for the MC service user. The following elements shall accompany the location information elements: time of measurement and optional accuracy. The following location information elements shall be optional (configurable) present: longitude, latitude, speed, bearing, altitude, ECGI, MBMS SAIs, with at least one provided. |  |  |

#### 10.9.2.8	Location information cancel subscription request

Table 10.9.2.8-1 describes the information flow from the MC service server to the location management server for location information cancel subscription request.

Table 10.9.2.8-1: Location information cancel subscription request (MC service server to location management server)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID list/ MC Group ID/ MC Group ID list | M | List of identities of the MC service users for whom location information is no longer required |

Table 10.9.2.8-2 describes the information flow from the location management client to the location management server for location information cancel subscription request.

Table 10.9.2.8-2: Location information cancel subscription request (Location management client to location management server)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID (see NOTE 1) | M | Identity of the requesting MC service user |
| MC service ID (see NOTE 2) | O | Identity of the MC service user, whose subscription is to be cancelled |
| MC service ID list / MC Group ID/ MC Group ID list | M | List of identities of the MC service users for whom location information is no longer required |
| NOTE 1: Authorized or subscribed MC service user. NOTE 2: Only used with the authorized MC service user. |  |  |

Table 10.9.2.8-3 describes the information flow from the location management server in the primary MC system to the location management server in the partner MC system for the location information cancel subscription request.

Table 10.9.2.8-3: Location information cancel subscription request (Location management server to location management server)

| Information element | Status | Description |
| --- | --- | --- |
| MC service ID (see NOTE 1) | M | Identity of the requesting MC service user in the primary MC system |
| MC service ID (see NOTE 2) | O | Identity of the MC service user in the partner MC system, whose subscription is to be cancelled |
| MC service ID list/ MC Group ID/ MC Group ID list | M | List of the identities of MC service users in the partner MC system for whom location information is no longer required |
| NOTE 1: Authorized or subscribed MC service user and only present when the request was originated by an MC service client. NOTE 2: Only used with the authorized MC service user. |  |  |

* * * End of Changes * * * *