3GPP TSG-SA WG6 Meeting #59	S6-240427

Athens, Greece 26th February – 1st March 2024	(revision of S6-240162)

| CR-Form-v12.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0521 | rev | 1 | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Minor corrections in clause 10.11 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Some minor corrections are introduced by this CR to the procedures in clauses 10.11.2 and 10.11.6.2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - redundant verbs in clause 10.11.2 step 2 - “answers” instead of “answer” in step 3 of clause 10.11.6.2 - adding a new line before step 5 of clause 10.11.6.2 to have step 4 and step 5 in separate lines. - change “cell” to “call” in step 6 of clause 10.11.6.2 - further reformulation and editorial corrections |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Poor quality of specs |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.11.2 and 10.11.6.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

### 10.11.2	Request for unicast resources at session establishment – SIP core based

The procedure defined in this subclause specifies how network resources are requested at session establishment. If concurrent sessions are used the MC service server may utilize the capability of resource sharing specified in 3GPP TS 23.203 [8]. The request for resources is sent to the PCRF on the Rx reference point and includes media type, bandwidth, priority, application identifier and resource sharing information.

The procedure is generic to any type of session establishment that requires requests for network resources.

Procedures in figure 10.11.2-1 are the signalling procedures for the requesting resource at session establishment.

Figure 10.11.2-1: Resource request at session establishment – SIP core based

1.	MC service client sends a session establishment request.

2.	MC service server evaluates the need network for resources and the use of media resource sharing.

3.	MC service server send a session progress request containing request for resources.

4.	PCC procedures (as defined in 3GPP TS 23.203 [8]) initiated from SIP core local inbound/outbound proxy over Rx.

5.	The SIP core local inbound / outbound proxy forwards the call control protocol request to the MC service client.

6.	The MC service client acknowledges the session progress request with an OK message.

7.	The SIP core local inbound / outbound proxy forwards the OK message to the MC service server.

8.	The MC service session is established and resources have been allocated.

# * * * * Second change * * * *

#### 10.11.6.2	Procedure

The figure below illustrates the procedure for resource reservation

Figure 10.11.6.2-1: Request for resources for transmission control and media plane

1.	The MC service client sends a request for group affiliation.

2.	The MC service client sends a request to the MC service server for establishment of a communication session.

3.	The MC service server answers the session establishment request and adjusts the bandwidth information in the session description. The requested bandwidth shall be minimized to cover the bandwidth requirements for floor control signalling (or transmission control for MCVideo or MCData).

4.	The SIP core requests resources towards the PCRF according to the session establishment request.

5.	The session establishment request is completed and a response is sent towards the MC service client.

6.	The MC service client sends a  call setup message according to existing procedures.

7.	The MC service server sends a session establishment request for resources for the media plane to PCRF, and the media plane is by that established. This request includes media description relevant for the media plane.

8.	Group call is ongoing on the group communication session.

9.	The MC service serve sends a release of media resources to PCRF, and the media plane is by that terminated.

10.	The PCRF performs session termination procedure according to 3GPP TS 23.203 [8].

11. The PCRF sends a session release response back to the MC service server.

NOTE 1:	The resources for transmission control are retained.

NOTE 2:	Step 6-9 can be repeated several times within the life cycle of one communication session.

# * * * * End of changes * * * *