3GPP TSG-SA WG6 Meeting #59	S6-240444

Athens, Greece, 26th February – 1st March 2024	(revision of S6-240246)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0535 | rev | 1 | Current version: | 18.8.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Reporting success or failure of an ad hoc group emergency alert cancel request (multiple MC systems) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia, Nokia Shanghai Bell |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MC_AHGC |  |  |  |  |  | Date: |  |  | 2024-02-19 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | If the initiator of an ad hoc group emergency alert cancel request is not authorised, current standard does not allow the MC service server to return a report in case of failure of the request. Similar changes were agreed in S6-233787/S6-233779 (adhoc alert establishment) and S6-233780/S6-233781 (adhoc alert cancel). |  |  |  |  |  |  |  |  |
| Summary of change: |  | Adding new step in clause 10.10.3.4.3 to report authorisation result of the ad hoc group emergency alert modify request. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The initiator will not be notified if the ad hoc group emergency alert cancel request has failed. The reason for denial will not be known by the initiator. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 10.10.3.4.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# * * * * First change * * * *

##### 10.10.3.4.3	Ad hoc group emergency alert cancel

Figure 10.10.3.4.3-1 illustrates the procedure once an MC service client cancels an ongoing ad hoc group emergency alert.

Preconditions:

-	An ad hoc group emergency alert is ongoing with participants within MC system A and MC system B.

-	MC system A and MC system B are interconnected.

Figure 10.10.3.4.3-1: Ad hoc group emergency alert cancel between multiple MC systems

1.	The MC service user 1 decides to cancel an ongoing ad hoc group emergency alert.

2.	MC service client 1 sends the MC service server A an ad hoc group emergency alert cancel request. If the cancel request is sent for another MC service user that initiated this emergency alert, that user`s MC service ID shall be included in the cancel request.

3.	MC service server A checks whether the MC service client 1 is authorized to cancel an ongoing ad hoc group emergency alert. The MC service server A checks whether the provided functional alias, if present, can be used and has been activated for the user.

4.	The MC service server A sends the ad hoc group emergency alert cancel request return to the MC service user 1 containing the result of whether the ad hoc group emergency alert cancel request is authorized or not. If the ad hoc group emergency alert cancel request is not authorized, the MC service server and the MC service client 1 shall not proceed with the rest of the steps.

5.	The MC service server A sends an ad hoc emergency alert cancel notification to the MC service server B at the partner MC system B.

6.	The MC service server A sends MC service client 2, MC service client 3 and MC service client 4 ad hoc group emergency alert cancel request.

7.	The MC service user 2, MC service user 3 and MC service user 4 are notified of the ad hoc group emergency alert cancellation.

8. 	The MC service client 2, MC service client 3 and MC service client 4 sends ad hoc group emergency alert cancel response to the MC service server A. If there is no other communication in the ad hoc group or once the communication in the ad hoc group is terminated, the ad hoc group ceases to exist.

9.	The MC service server A sends an ad hoc group emergency alert cancel response to MC service client 1.

# * * * * End of changes * * * *