3GPP TSG-SA WG6 Meeting #59	S6-240474

Athens, Greece 26th February – 1st March 2024	(revision of S6-240168)

Source:	China Mobile

Title:	pCR on Relationship between eMMTel Enabler architecture and architectures specified in GSMA NG.134

Spec:	3GPP TR 23.700-92 v0.3.0

Agenda item:	8.2

Document for:	Approval

Contact:	liuyueyjy@chinamobile.com

1. Introduction

Some architectures related to eMMTel service have been specified in both 3GPP and GSMA, including architecture specified in 3GPP TS 23.228, GSMA TGY.02, GSMA NG.134 and GSMA TS.66. All these architectures have not covered the enabler layer of eMMTel service. It is necessary to explain the relationship between the eMMTel Enabler architecture and the architecture specified in these specifications. Therefore, an informative annex is proposed in this pCR to explain the relationship.

2. Reason for Change

Contribution with new text.

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-92 v0.3.0.

* * * First Change * * * *

X.y1	Relationship between eMMTel Enabler architecture and architectures specified in GSMA NG.134

A profile for IMS data channel services over IMS is specified in GSMA NG.134 [X1]. A use case listed in GSMA NG.134 [X1] is that a IMS data channel application service logic and media content MAY be either provided by the Data Channel Server inside the operator’s administrative domain, or by an External Server which is deployed outside the operator’s administrative domain. This use case is list in Annex C and Annex D of GSMA NG.134 [X1] for information. When the GSMA NG.134 [X1] is released, the Rel-18 work on NG_RTC has not been finished, the architecture of Data Channel Capable PLMN in GSMA NG.134 [X1] is not detailed profiled. Since GSMA NG.134 [X1] will only define profiles based on 3GPP specifications but will not develop technical solutions, the architecture specified in Annex AC of 3GPP TS 23.228 [3] can be considered as the IMS Data Channel Capable PLMN in GSMA NG.134 [X1]. In the Annex C and Annex D of GSMA NG.134 [X1], how to provide service enabler layer capabilities exposure of eMMTel services are not described.

There are two Communication Model for Accessing External Server listed in Annex D of GSMA NG.134 [X1]. The Model A is HTTP Server without IETF RFC 8831 Data Channel Support and shown in Figure C.1-1 of GSMA NG.134 [X1]. The Model B is HTTP Server with IETF RFC 8831 Data Channel Support and shown in Figure C.2-1 of GSMA NG.134 [X1].

When eMMTel Enabler is introduced, the two different models listed in GSMA NG.134 [X1] may not impact the architecture of eMMTel Enabler. The External Server in GSMA NG.134 [X1] can be considered as Application Server in the architecture of eMMTel. Therefore Relationship between eMMTel Enabler architecture and architectures specified in GSMA NG.134 [X1] is shown in figure X.y1-1.

Figure X.y1-1: Relationship between eMMTel Enabler architecture and architectures specified in GSMA NG.134

The External Server in GSMA NG.134 [X1] acts as Application Server in the eMMTel Enabler architecture and communicates with the IMS Data Channel Capable PLMN in GSMA NG.134 [X1] via the eMMTel Enabler Server by communicating with the eMMTel-2 reference point.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.261: "Service requirements for the 5G system".

[3]	3GPP TS 23.228: "IP Multimedia Subsystem (IMS)".

[4]	OMA-AD-NGSI-V1_0-20120529-A: "Next Generation Service Interfaces Architecture".

[5]	OMA-TS-REST_NetAPI_ThirdPartyCall-V1_0-20130212-C: "RESTful Network API for Third Party Call".

[6]	OMA-TS-REST_NetAPI_CallNotification-V1_0-20200226-C: "RESTful Network API for Call Notification".

[7]	OMA-TS-REST_NetAPI_AudioCall-V1_0-20200226-C: "RESTful Network API for Audio Call".

[8]	3GPP TS 23.198: "Open Service Access (OSA)".

[9]	3GPP TS 23.222: "Functional architecture and information flows to support Common API Framework for 3GPP Northbound APIs; Stage 2".

[10]	3GPP TR 22.873: "Study on evolution of the IP Multimedia Subsystem (IMS) multimedia telephony service".

[11]	GSMA NG.129: "IMS Data Channel White Paper v0.1".

[12]	GSMA TGY.02: "Business Voice Calling".

[13]	3GPP TR 23.700-87: "Study on system architecture enhancement for next generation real time communication".

[14]	3GPP TS 26.114: "IP Multimedia Subsystem (IMS); Multimedia Telephony; Media handling and interaction"

[X1]	GSMA NG.134: "IMS Data Channel, Version 1.0".

…

[x]	<doctype> <#>[ ([up to and including]{yyyy[-mm]|V<a[.b[.c]]>}[onwards])]: "<Title>".