3GPP TSG-SA WG6 Meeting #59	S6-240480

Athens, Greece 26th February – 1st March 2024	(revision of S6-240275)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0036 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Complete API for regular data transmission procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the SEALDD connection status subscription between VAL server and SEALDD server in clause 9.2.2.3, there are missed information flow and API for the SEALDD connection status subscription update and the SEALDD connection status unsubscription. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to complete the information flow and API for the SEALDD connection status subscription update and the SEALDD connection status unsubscription. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For current specification, the information flow and API for the SEALDD connection status subscription are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.8, 9.2.3.x (new), 9.2.3.y (new), 9.2.3.z (new), 9.2.3.a (new), 9.2.4.1, 9.2.4.x (new), 9.2.4.y (new), 9.2.2.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.2.3.8	SEALDD connection status subscription response

Table 9.2.3.8-1 describes the information flow from the SEALDD server to VAL server for responding SEALDD connection status subscription request.

Table 9.2.3.8-1: SEALDD connection status subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| > Reporting ID | O (NOTE) | Indicates the reporting ID of the connection status per VAL UE |
| > Subscription ID | O (NOTE) | Subscription identifier corresponding to the subscription. |
| Expiration time | O (NOTE) | Indicates the expiration time of the subscription. |
| NOTE: These IEs shall be present when the result is success. |  |  |

* * * Next Change * * * *

9.2.3.x	SEALDD connection status subscription update request

Table 9.2.3.x-1 describes the information flow from the VAL server to SEALDD server to update subscription for SEALDD connection status information.

Table 9.2.3.x-1: SEALDD connection status subscription update request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated |
| SEALDD-S Data transmission connection information | O | Address/port of the VAL server to send/receive the application packets to/from the SEALDD server |
| Immediate reporting flag | O | Indicates the immediate reporting of connection status notification |
| SEALDD client connection status check periodicity | O | Indicates the frequency to perform SEALDD client connection status check |

9.2.3.y	SEALDD connection status subscription udpate response

Table 9.2.3.y-1 describes the information flow from the SEALDD server to VAL server for responding SEALDD connection status subscription update request.

Table 9.2.3.y-1: SEALDD connection status subscription update response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| Expiration time | O (NOTE) | Indicates the expiration time of the subscription. |
| NOTE: This IE shall be present when the result is success. |  |  |

* * * Next Change * * * *

9.2.3.z	SEALDD connection status unsubscribe request

Table 9.2.3.z-1 describes the information flow from the VAL server to SEALDD server to unscribe the SEALDD connection status information.

Table 9.2.3.z-1: SEALDD connection status unsubscribe request

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M | Subscription identifier corresponding to the subscription to be updated |

9.2.3.a	SEALDD connection status unsubscribe response

Table 9.2.3.a-1 describes the information flow from the SEALDD server to VAL server for responding SEALDD connection status unsubscribe request.

Table 9.2.3.a-1: SEALDD connection status unsubscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |

* * * Next Change * * * *

9.2.4	APIs

9.2.4.1	General

Table 9.2.4.1-1 illustrates the APIs exposed by SEALDD server for regular connection establishment.

Table 9.2.4.1-1: List of SEALDD server APIs for data distribution

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_RegularTransmission | Request | Request/Response | VAL server |
| Sdd_RegularTransmissionConnection | Establish | Request/Response | SEALDD client |
|  | Release | Request/Response | SEALDD client |
| Sdd_ConnectionStatusEvent | Subscribe | Subscribe/Notify | VAL server |
|  | Notify | Subscribe/Notify | VAL server |
|  | Update | Subscribe/Notify | VAL server |
|  | Unsubscribe | Subscribe/Notify | VAL server |

* * * Next Change * * * *

9.2.4.x	Sdd_ConnectionStatusEvent_Subscribe_Update operation

API operation name: Subscribe update

Description: The consumer requests to update the subscription of SEALDD connection status event.

Inputs: See clause 9.2.3.x.

Outputs: See clause 9.2.3.y.

See clause 9.2.2.3 for details of usage of this operation.

9.2.4.y	Sdd_ConnectionStatusEvent_Unsubscribe operation

API operation name: Unsubscribe

Description: The consumer requests to unsubscribe the subscription of SEALDD connection status event.

Inputs: See clause 9.2.3.a.

Outputs: See clause 9.2.3.b.

See clause 9.2.2.3 for details of usage of this operation.

* * * Next Change * * * *

9.2.2.3	SEALDD enabled regular data transmission connection establishment based on policy

The SEALDD servers has Data Delivery (DD) policy being provisioned. Before the application communication between VAL client and VAL server starts, the DD policy is enforced by the SEALDD server to establish the SEALDD connection.

Pre-conditions:

1.	The SEALDD server has DD policies available.

Figure 9.2.2.3-1: Policy enforced by SEALDD server for connectivity

1.	The VAL server subscribes to SEALDD event exposure for connection status using the procedure defined in clause 9.2.2.6.

NOTE 1:	The VAL server can update/delete an existing subscription at the SEALDD Server when required.

2.	When the time for data transmission is about to start, the SEALDD server enforces the policy to trigger regular data transmission connection establishment. If spatial condition for UE is provided, the SEALDD server also ensures the UE’s location requirement is satisfied when establishing regular data transmission connection (e.g. by using NEF service for monitoring UE location or SEAL location service for UE entering area of interest).

3.	If there is a special routing requirement for SEALDD user plane traffic (e.g. running on a specific slice and DNN), the SEALDD server interacts with 3GPP CN to provision service specific parameters with NEF as described in 3GPP TS 23.502 [6], clause 4.15.6.10 and clause 4.15.6.7.

If there are QoS requirements in the DD policy, the SEALDD server also applies QoS to ensure the quality for SEALDD traffic by utilizing NEF/PCF/NRM/EES service for QoS adjustment. Specifically, the SEALDD server relies on the northbound Policy Authorization Service API exposed by the PCF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via the N5 reference point, or the northbound AF Session with QoS Service API and/or the PFD Management northbound APIs exposed by the NEF as specified in 3GPP TS 23.502 [6] and 3GPP TS 23.503 [7], if the SEALDD server is connected to the PCF via NEF. SEALDD may also rely upon the EES Session with QoS API as specified in 3GPP TS 23.558 [10] and/or the NRM QoS functionality as described in 3GPP TS 23.434 [4].

If the DD policy specifies failure detection report, the SEALDD server may subscribe to CN analytics (e.g. DN performance analytics) from NEF/NWDAF and further notify data delivery status of application traffic to VAL client (via SEALDD client) and VAL server based on analytics result.

4.	The SEALDD server allocates an IP address and port for sending and receiving packet over SEAL-S reference point, then SEALDD server sends SEALDD connection establishment notification to the VAL server with VAL service ID, the IP address and port.

5-6.	The SEALDD server allocates an IP address and port for sending and receiving packet over SEAL-Uu reference point, then SEALDD server sends regular data transmission connection establishment request to the SEALDD client with SEALDD flow ID, VAL service ID, the IP address and port. The request is responded by the SEALDD client. UE IP address (and port) may be included by the SEALDD client in the response or sent in a separate update message by SEALDD client if a different UE IP address is to be used in SEALDD connection user plane.

NOTE 2:	Step 4 and step 5 can be done in parallel.

NOTE 3:	Step 5 can be sent via PDU session (if exist) or via application triggering (if no PDU session exists).

7.	The SEALDD client further notifies the VAL client about the SEALDD connection being established.

Upon receiving application traffic from VAL client (not shown in the figure), the SEALDD client sends it to SEALDD server in SEALDD traffic. The SEALDD server identifies application traffic based on the VAL service ID and further sends the application traffic to VAL server. The downlink application traffic sent from VAL server to VAL client is processed similarly.