3GPP TSG-SA WG6 Meeting #59	S6-240482

Athens, Greece 26th February – 1st March 2024	(revision of S6-240277)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0038 | rev | 1 | Current version: | 19.0.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Complete API for stored data transfer procedure |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD |  |  |  |  |  | Date: |  |  | 2024-02-07 |
| Category: | A |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | For the stored data transfer between VAL server via SEALDD server in clause 9.5.2.5, regarding of the SEALDD delivery connection establishment between source SEALDD server and target SEALDD server, the related information flow and API are missed. |  |  |  |  |  |  |  |  |
| Summary of change: |  | This paper is proposed to complete the information flow and API for the SEALDD delivery connection establishment between source SEALDD server and target SEALDD server. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | For current specification, the information flow and API for the SEALDD delivery connection establishment are incomplete. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.5.3.x (new), 9.5.3.y (new), 9.5.4.1, 9.5.4.x (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

9.5.3.x	SEALDD delivery connection establish request

Table 9.5.3.x-1 describes the information flow from the source SEALDD server to the target SEALDD server for requesting the SEALDD delivery connection establishment.

Table 9.5.3.x-1: SEALDD delivery connection establish request

| Information element | Status | Description |
| --- | --- | --- |
| Source SEALDD server information | M | Identify the source SEALDD server |
| Traffic descriptor of source SEALDD server | O | The traffic descriptor (e.g. address, port, transport layer protocol) of the source SEALDD server |
| Target VAL server information | M | Identify the target VAL server |

9.5.3.y	SEALDD delivery connection establish response

Table 9.5.3.y-1 describes the information flow from the target SEALDD server to the source SEALDD server for responding the SEALDD delivery connection establishment request.

Table 9.5.3.y-1: SEALDD delivery connection establish response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| Traffic descriptor of target SEALDD server | O | The traffic descriptor (e.g. address, port, transport layer protocol) of the target SEALDD server |

* * * Next Change * * * *

9.5.4	APIs

9.5.4.1	General

Table 9.5.4.1-1 illustrates the APIs exposed by SEALDD server for data storage.

Table 9.5.4.1-1: List of SEALDD server APIs for data storage

| API Name | API Operations | Operation Semantics | Consumer(s) |
| --- | --- | --- | --- |
| Sdd_ DataStorage_Creation | Request | Request/Response | SEALDD client, VAL server |
| Sdd_ DataStorage_Query | Request | Request/Response | SEALDD client, VAL server, other SEALDD server |
| Sdd_ DataStorage_Management | Request | Request/Response | SEALDD client, VAL server |
| Sdd_DataStorage_Delivery_Subscription | Request | Request/Response | VAL server |
| Sdd_DataStorage_Delivery_notification | Notify | Subscribe/notify | VAL server |
| Sdd_DataStorage_Delivery | Request | Request/Response | VAL server |
| Sdd_DeliveryConnection_Establish | Request | Request/Response | SEALDD server |

* * * Next Change * * * *

9.5.4.x	Sdd_DeliveryConnection_Establish Request operation

API operation name: Sdd_DeliveryConnection_Establish Request

Description: The consumer requests for one time for delivery connection establishment between SEALDD servers.

Inputs: See clause 9.5.3.x.

Outputs: See clause 9.5.3.y.

See clause 9.5.2.5 for details of usage of this operation.