3GPP TSG-SA WG6 Meeting #59	S6-240506

Athens, Greece, 26th February – 1st March 2024 	was S6-240338

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.436 | CR | 0022 | rev | 1 | Current version: | 18.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updates to UE-to-UE Application Performance Analytics and API |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | ADAES |  |  |  |  |  | Date: |  |  | 2024-02-12 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | CT3 have LS sent to SA6 for questions on Issues related to ADAE server APIs (S6-240021/C3-240225), and on A-DCCF, A-ADRF and ADAES services (S6-240019/C3-240095). This paper proposes to update the UE-to-UE Application Performance Analytics and API. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The changes include: - Updated the description for “Analytics type” in clause 8.4.3.2. - Removed “Analytics type” in clause 8.4.3.6. - Updated the description for “Reporting configuration” in clause 8.4.3.4. - Added “Data collection requirements” to the table in clause 8.4.3.4. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrect stage 2 specification for UE-to-UE Application Performance Analytics and API. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4.3.2, 8.4.3.4, 8.4.3.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | This CR addresses LS S6-240021/C3-240225 and S6-240019/C3-240095. |  |  |  |  |  |  |  |  |
| () |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# Additional discussion(if needed):

# Proposed changes:

* * * First Change * * * *

#### 8.4.3.2	UE-to-UE session performance analytics subscription request

Table 8.4.3.2-1 describes information elements for the UE-to-UE session performance analytics subscription request from the consumer (VAL server) to the ADAE server.

Table 8.4.3.2-1: UE-to-UE session performance analytics subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | The identifier of the analytics consumer (VAL server). |
| Analytics ID | M | The identifier of the analytics event. This ID can be equivalent to “UE-to-UE session performance analytics”. |
| Analytics type | M | The type of analytics for the event, e.g. statistics or predictions. |
| List of VAL UE IDs and addresses | M | The VAL UEs for which the analytics subscription applies. |
| VAL service ID | O | The VAL service for which the subscription applies. |
| Preferred confidence level | O | The level of accuracy for the analytics service (in case of prediction. |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the subscription request. |
| Exposure level requirement | O | The level of exposure requirement (e.g. condition on providing the analytics like threshold is reached) for the analytics to be exposed. |
| Reporting requirements | O | It describes the requirements for analytics reporting. This requirement may include e.g. the type and frequency of reporting (periodic or event triggered), the reporting periodicity in case of periodic, and reporting thresholds. |

* * * Next change * * * *

#### 8.4.3.4	UE-to-UE analytics request

Table 8.4.3.4-1 describes information elements for the UE-to-UE Analytics request from the ADAE server to the ADAE client.

Table 8.4.3.4-1: UE-to-UE analytics request

| Information element | Status | Description |
| --- | --- | --- |
| ADAE server ID | M | The identifier of the ADAE server. |
| Analytics ID | M | The identifier of the analytics event (Analytics ID=’UE to UE session analytics’). |
| List of VAL UE IDs and addresses | M | The VAL UE(s) identifiers and IP address(es) for which the data/analytics apply. |
| Application QoS attributes | M | The QoS attributes (latency, bitrate, jitter, application layer PER) to be analyzed at the ADAE client. |
| Reporting configuration | O | The configuration for analytics reporting. This requirement may include e.g. the frequency of reporting (periodic), the reporting periodicity in case of periodic, and reporting thresholds, whether data abstraction is needed or not. |
| Data collection requirements | O | The requirements for data collection, including the format of data, frequency of reporting, level of abstraction of data, level of accuracy of data. |
| Area of Interest | O | The geographical or service area for which the subscription request applies. |
| Time validity | O | The time validity of the subscription request. |

* * * Next change * * * *

#### 8.4.3.6	ADAE Analytics Notification

Table 8.4.3.6-1 describes information elements for the ADAE Analytics Notification from the ADAE server to the consumer (VAL server).

Table 8.4.3.6-1: ADAE Analytics notification

| Information element | Status | Description |
| --- | --- | --- |
| Analytics ID | M | The identifier of the analytics event. This ID can be “UE to UE session performance analytics”. |
| Analytics Output | M | The predictive or statistical parameter, which can be: - A VAL UE to UE session predicted or expected performance change - A VAL UE to UE session performance sustainability over a given time horizon/area |
| Confidence level | O | For predictive analytics, the achieved confidence level can be provided. |

* * * End of changes * * * *