3GPP TSG-SA WG6 Meeting #59	S6-240529

Athens, Greece 26th February – 1st March 2024	(revision of S6-240167)

Source:	NTT Docomo, Lenovo, Nokia, Nokia Shanghai Bell, Fogus innovations and services P.C.

Title:	Open source implementation of CAPIF for TR 23.946

Spec:	3GPP TR 23.946 v0.2.0

Agenda item:	9.7

Document for:	Approval

Contact:	junpei.uoshima.mt@nttdocomo.com

1	Introduction

This pCR provides the proposal for open-source implementation of CAPIF of TR 23.946.

2	Proposal

It is proposed to modify the text of TR 23.946 as follows.

* * * First Change * * * *

## 6		Exemplary Use Cases and Adoption

## Editor's Note: This clause will provide the main representative usage examples for CAPIF and adoption in 3GPP, ETSI, etc.

6.x	Open-source implementation of CAPIF

As stated in clause B.2.2.3 in 3GPP TS 23.222 [2], the NEF can implement the functionalities of the API provider domain functions, as reproduced in Figure 6.x-1.

Figure 6.x-1: NEF implements the service specific aspect compliant with the CAPIF architecture

Evolved-5G[r1] (ICT-41-2020 project from call H2020-ICT-2018-2020) developed an open-source project of the CAPIF Release 17 under the Apache-2.0 license. EVOLVED-5G implementation of CAPIF  works with an open-source implementation of a NEF emulator available in [r4]. The NEF emulator used for Evolved-5G implementation supports two services, namely Nnef_EventExposure and Nnef_AFsessionWithQoS [r4].

Evolved-5G implementation has been used as the seed code for OpenCAPIF SDG [r2] created by ETSI in January 2024.

OpenCAPIF has adopted the following APIs from CAPIF:

- JWT Authentication APIs

- CAPIF Invoker Management API

- CAPIF Publish API

- CAPIF Discover API

- CAPIF Security API

- CAPIF Events API

- CAPIF Provider Management API

The implementation also includes a Testing Suite of the above services.

Every CAPIF API is implemented in a separate container for future scalability adaptation. All APIs are exposed and support mutual TLS authentication as specified in 3GPP TS 33.122[4].

The Source Code of OpenCAPIF and the instructions to deploy it are included in the ETSI GitLab repository[r3].

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[3]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[4]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[r1]	EVOLVED-5G ICT-41 project, https://evolved-5g.eu

[r2]	ETSI Software Development Group: "OpenCAPIF", https://ocf.etsi.org

[r3]	ETSI Software Development Group GitLab repository: "OpenCAPIF", https://labs.etsi.org/rep/ocf/capif.

[r4]	NEF emulator version 2.2.3, February 2024. https://github.com/EVOLVED-5G/NEF_emulator

* * * End of Changes * * * *