3GPP TSG-SA WG6 Meeting #59	S6-240531

Athens, Greece 26th February – 1st March 2024	(revision of S6-240171)

Source:	NTT Docomo, Nokia, Nokia Shanghai Bell

Title:	Role of stakeholders for API exposure

Spec:	3GPP TR 23.946 v0.2.0

Agenda item:	9.7

Document for:	Approval

Contact:	junpei.uoshima.mt@nttdocomo.com

1	Introduction

This pCR introduces some relevant stakeholders and provides a proposal for placing them within for API exposure ecosystem in relationship with CAPIF.

2	Proposal

It is proposed to modify the text of TR 23.946 as follows.

* * * First Change * * * *

5	Role of Stakeholders for API Exposure

5.1	Stakeholders in CAPIF

Several stakeholders are considered to interact within the API exposure ecosystem, for instance: MNO, NPN owners, aggregators, Application Service Developers (ASP), authorization service providers, identity service providers, resource owners, API invokers.

The functional model description for CAPIF is given in subclause 6.2.0 of 3GPP TS 23.222 [2]. Different business relationships are considered for CAPIF as follows:

- Basic CAPIF business relationships are given in subclause 5.1 of 3GPP TS 23.222 [2].

- CAPIF business relationships for RNAA are provided in subclause 5.2 of 3GPP TS 23.222 [2].

- In case of interconnection among CAPIF providers, CAPIF business relationships are given in subclause 4.12 3GPP TS 23.222 [2]

Some exemplary business relationships are reproduced in Figure 5.1-1.

Figure 5.1-1: Basic business relationships in CAPIF

CAPIF has three main stakeholder roles (CAPIF provider, API provider, and API invoker) and RNAA has resource owner role.

5.2	Basic roles

5.2.1	CAPIF provider

The CAPIF provider contains an instance of CAPIF core function and may contain API provider and API invokers and it could be a PLMN, SNPN or 3rd party. The CAPIF provider and the API provider can be part of the same organization (e.g. PLMN operator), in which case the business relationship between the two is internal to a single organization. The CAPIF provider and the API provider can be part of different organizations, in which case the business relationship between the two must exist [2].

5.2.2	API provider

The API provider hosts one or more service APIs and has a service API arrangement with CAPIF provider to offer the service APIs to the API invoker.

Requirements for enabling API invoker is given in subclause 4.1.2 3GPP TS 23.222 [2].

5.2.3	API invoker

The API invoker is typically provided by a 3rd party application provider who has service agreement with PLMN operator. The API invoker may reside within the same trust domain as the PLMN operator network. The API invoker may be either an application on a server or an application on a UE [2].

Requirements for supporting 3rd party API providers is given in subclause 4.1.3 3GPP TS 23.222 [2].

5.2.4	Resource owner

The resource owner is an entity capable of granting access to a protected resource related to the resource exposed by the API provider. In the current release, it is a user of the UE hosting the API invoker that can authorize the API access [2].

Requirements for supporting RNAA is given in subclause 4.17.2 3GPP TS 23.222 [2].

5.3	Mapping of stakeholders to CAPIF roles

NOTE: The mapping of the stakeholders to the CAPIF roles is FFS.