3GPP TSG-SA WG6 Meeting #59	S6-240573

(Revision of S6-240393)

Athens, Greece, 26th February – 1st March 2024

Source:	KPN N.V., Convida Wireless

Title:	Solution #7 update

Spec:	3GPP TR 23.700-82 v0.2.0

Agenda item:	8.3

Document for:	Approval

Contact:	Yonatan Shiferaw, yonatan.shiferaw@tno.nl

Nassima Toumi, nassima.toumi@tno.nl

1. Introduction

This contribution updates Solution #7 on F/AIML client discovery to clarify discovery criteria and include additional ones.

2. Reason for Change

Solution#7 specifies a procedure for AI/ML enablement client discovery, where the discovery is performed based on a set of discovery criteria, including the application identifier. However, the application identifier is not clear and not yet not defined in SA6. In this contribution we introduced instead List of VAL service ID(s) which are the services that the the AI/ML client support. The client may also allow a certain service permission level per VAL service ID, so this parameter should be taken into account when selecting the AI/ML client based on the application needs.

Additionally, the AI/ML client may support a certain service type instead of specific application IDs. In that manner, the client can be selected for any application that matches the specific service type.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82.

* * * First Change * * * *

## 8.7 	Solution #7: AIML enablement client discovery

### 8.7.1	Solution description

The following clauses specify procedures, information flows, and APIs for Key Issue #3 to support AIML enablement client discovery.

Assumptions:

1.	The proposed solution is based on client-server architecture for federated learning.

2.	The AIML enablement server has a repository of AIML enablement clients and associated AIML client profiles.

3.	AIML enablement clients that support AIML operations have registered with the AIML enablement server and included their AIML client profile. The registration procedure may be performed as described in solution 8.

Figure 8.7.1-1: AIML client discovery procedure

1.	A VAL server sends a request to an AIML enablement server to discover AIML enablement clients that are available to participate in federated learning operations, e.g. is available and have required data to train an ML model. The AIML enablement client discovery request includes client/server identifier, security credentials, discovery criteria.

Editor's Note: Other IEs in the request are FFS. Whether the IEs are included in the AIML member selection policy is FFS.

2.	The AIML enablement server validates the discovery request and verifies the security credentials provided in the discovery request. The AIML enablement server further performs authentication and authorization checks to determine if the requestor is able to discover AIML enablement clients. If authorized, the AIML enablement server determines a list of AIML enablement clients that meets the discovery criteria.

NOTE: The AIML enablement server may provide list of target UE(s) and utilize Member UE selection assistance procedure from TS 23.502, section 4.15.13 to get assistance in selecting candidate UEs. The AIML enablement server then determines a list of UEs that fulfils the discovery criteria based on information received from AIML enablement client registrations.

Editor's Note: It is FFS whether a FL client repository function is necessary to enable this step.

3.	The AIML enablement server sends an AIML enablement client discovery response that includes the status of the discovery request, a list of AIML client profiles matching the discovery criteria, and the number of matching AIML client profiles.

### 8.7.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 8.7.3	Corresponding APIs

Table 8.7.3-1 shows the request sent by a VAL server to an AIML enablement server for the AIML enablement client discovery procedure.

Table 8.7.3-1: Request for AIML enablement client discovery procedure

| Information element | Status | Description |
| --- | --- | --- |
| VAL server identifier | M | The identifier of the VAL server. |
| Security credentials | M | Security credentials to authenticate and authorize the AIML enabler client. |
| Discovery criteria | M | Discovery criteria for finding suitable AIML enablement clients for AIML operations: |
|  |  | > Requested ML models |
|  |  | > Requested AIML roles: role for data collection, data preparation, training, or inferencing |
|  |  | > Target location (for data collection purposes) |
|  |  | > Client capabilities such as available compute, memory, and storage that may be dedicated to AIML operations |
|  |  | > Dataset requirements that includes: >> dataset availability, including dataset size, age, features list, and data distribution of each feature >> Data capabilities such as data collection and supported data processing capabilities |
|  |  | > Required service: >> Required VAL service ID or Service type (e.g. Slice/Service Type SST) that the client support >> Required corresponding service permission level (e.g. raw data, processed data, premium resource usage standard resource usage, limited resource usage) |

Editor’s Note: Which IE’s in Table 8.7.3-1 discovery criteria are optional and mandatory is FFS.

Table 8.7.3-2 shows the response sent by the AIML enablement server to the VAL server for the AIML enablement client discovery procedure.

Table 8.7.3-2: Response for AIML enablement client discovery procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| List of AIML client IDs | M | A list of AIML client IDs that matches the AIML discovery criteria. |
| Number of discovered AIML clients | O | The number of AIML client profiles that was found to match the AIML discovery criteria. The number indicates the potential AIML enablement clients that could participate in AIML operations. |

### 8.7.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Changes * * * *