3GPP TSG-SA WG6 Meeting #59	S6-240574

(Revision of S6-240394)

Athens, Greece, 26th February – 1st March 2024

Source:	KPN N.V., Convida Wireless

Title:	Solution #8 update

Spec:	3GPP TR 23.700-82 v0.2.0

Agenda item:	8.3

Document for:	Approval

Contact:	Yonatan Shiferaw, yonatan.shiferaw@tno.nl

Nassima Toumi, nassima.toumi@tno.nl

1. Introduction

This contribution updates Solution #8 on F/AIML enablement client registration to clarify discovery criteria and include additional ones.

2. Reason for Change

Solution#8 specifies a procedure for AI/ML enablement client registration, where the client provides a set of information elements to use as discovery criteria for filtering available clients, including the application identifier. However, the application identifier is not clear and not yet not defined in SA6. In this contribution we introduced instead List of VAL service ID(s) which are the services that the the AI/ML client support. The client may also allow a certain service permission level per VAL service ID, so this parameter should be taken into account when selecting the AI/ML client based on the application needs.

Additionally, the AI/ML client may support a certain service type instead of specific application IDs. In that manner, the client can be selected for any application that matches the specific service type.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82.

* * * First Change * * * *

## 8.8 	Solution #8: AIML enablement client registration

### 8.8.1	Solution description

The following clauses specify procedures, information flows, and APIs for Key Issue #3 to support AIML enablement client registration.

Assumptions:

1.	The proposed solution is based on client-server architecture for federated learning.

2.	The AIML enablement client has been pre-configured or has discovered the address (e.g. URI) of the AIML enablement server.

3.  The AIML enablement client has been pre-configured with an AIML client profile.

Figure 8.8.1-1: AIML enablement client registration procedure

1.	The AIML enablement client sends a registration request to the AIML enablement server, the registration request includes a AIML enablement client identifier, security credentials, and AIML client profile .

Editor's Note: Other IEs in the request are FFS. Whether the IEs are included in the AIML member selection policy is FFS.

2.	The AIML enablement server validates the registration request and verifies the security credentials. The AIML enablement server further performs an authentication and authorization check to determine if the AIML enablement client is permitted to register to the AIML enablement server and participate in federated learning operations. The AIML enablement server assigns an identifier for the AIML client profile to associate it with the AIML enablement client.

Editor's Note: It is FFS whether a FL client repository function is necessary to enable this step.

3.	The AIML enablement server returns an AIML enablement registration response to the AIML enablement client with the status of the registration request and the identifier associated with the AIML client profile.

### 8.8.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 8.8.3	Corresponding APIs

Table 8.8.3-1 shows the request sent by an AIML enablement client to an AIML enablement server for the AIML enablement client registration procedure.

Table 8.8.3-1: Request for AIML enablement client registration procedure

| Information element | Status | Description |
| --- | --- | --- |
| AIML enablement client identifier | M | The identifier of the AI/ML enabler client. |
| Security credentials | M | Security credentials to authenticate and authorize the AIML enabler client. |
| AIML client profile | M | Information about the capability of the AIML enablement client to support AIML operations: |
|  |  | > Supported ML models |
|  |  | > Supported AIML roles |
|  |  | > Availability schedule |
|  |  | > Client capabilities such as available compute, memory, and storage that may be dedicated to AIML operations |
|  |  | > Dataset availability, including dataset size, age, features list, and data distribution of each feature |
|  |  | > Data capabilities such as data collection and supported data processing capabilities |
|  |  | > List of supported services: >> List of VAL service ID(s) or List of Service type(s) (e.g. Slice/Service Type SST) that the client support >> Corresponding service permission level(s) (e.g. raw data, processed data, premium resource usage standard resource usage, limited resource usage |

sEditor's Note: Which IE’s in Table 8.8.3-1 AIML client profile are optional and mandatory is FFS.

Table 8.8.3-2 shows the response sent by the AIML enablement server to the AIML enablement client for the AIML enablement client registration procedure.

Table 8.8.3-2: Response for AIML enablement client registration procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| AIML client profile ID | O | An identifier for the AIML client profile |

### 8.8.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

* * * End of Changes * * * *