3GPP TSG-SA WG6 Meeting #59	S6-240575

Athens, Greece 26th February – 1st March 2024	(revision of S6-240144/0514)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.435 | CR | 0010 | rev | 2 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Add late notification to the network slice adaptation procedures |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | NSCALE |  |  |  |  |  | Date: |  |  | 2024-02-18 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Network slice adaptation for VAL application procedures defined in claue 9.11, a request is responded with the immediate response with the adaptation result. However, sometimes, the adaptation may take more time, it is not possible to provide the result in the immediate response. So a late notification is needed to provide such adaptation to the requesting entity. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce the subsricption and notification procedures. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Cannot support the case if the adaptation may take more time. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.11.2.1, 9.11.2.aa (new), 9.11.2.2, 9.11.2.bb (new), 9.11.3.aa(new), 9.11.3.bb(new), 9.11.3.cc(new), 9.11.3.dd(new),9.11.3.ee(new), 9.11.4.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************** 1st changes ****************************/

9.11.2.1	Procedure for VAL server-triggered and network-based network slice adaptation for VAL application- request and response model

# /************************** Next changes ****************************/

9.11.2.aa	Procedure for VAL server-triggered and network-based network slice adaptation for VAL application – subscribe and notify model

Figure 9.11.2.aa-1 illustrates the VAL server-triggered and network-based procedure with subscribe and notify model where the NSCE server supports the network slice adaptation with the underlying 3GPP system for the VAL UEs of the VAL application.

Whether and how the service continuity procedures are impacted

VAL Server

1. Network slice adaptation subscribe request

Whether and how the service continuity procedures are impacted

NSCE Server

Whether and how the service continuity procedures are impacted

5GC

Whether and how the service continuity procedures are impacted

OAM

Whether and how the service continuity procedures are impacted

3. Network adaption process

2. Network slice adaptation subscribe response

4. Network slice adaptation notification

Figure 9.11.2.aa-1: Network slice adaptation for VAL application

1.	The VAL server sends a network slice adaptation subscribe request to the NSCE server for the VAL application (and the VAL UEs within the VAL application). This request may be in the form of exact requested network slice (and optionally DNN) for all the VAL UEs of the VAL application; or indication that the VAL application needs to be remapped to a different network slice (and optionally DNN). The request optionally includes the adaptation threshold of network slice adaptation as defined in Table 9.11.3.1-1.

2.	The NSCE server confirms the subscription and returns the network slice adaptation subscribe response.

3.	The NSCE server may interact with the 5GC and OAM to collects the network slice status and performs the network network slice adaptation per VAL UE as described in step 2- step 4 in clause 9.11.2.1.

4.	When the network slice adaptation completes, and the adaption result comes out and available, the NSCE server provides the network slice adaptation notification to the VAL server.

# /************************** Next changes ****************************/

9.11.2.2	Procedure for VAL UE-triggered and network-based network slice adaptation for VAL application - request and response model

# /************************** Next changes ****************************/

9.11.2.bb	Procedure for VAL UE-triggered and network-based network slice adaptation for VAL application - subscribe and notify model

Figure 9.11.2.bb-1 illustrates the VAL UE-triggered and network-based procedure with subscribe and notify modeld where the NSCE server supports the network slice adaptation with the underlying 3GPP system for the VAL UEs of the VAL application.

Pre-condition:

-	The NSCE client has connected to the NSCE server;

NSCE client

2. Network slice adaptation trigger subscribe request

NSCE Server

5GC

4. Network adaption process

3. Network slice adaptation trigger subscribe response

5. Network slice adaptation notification

VAL Client

1. VAL application requirement change

Figure 9.11.2.bb-1: Network slice adaptation for VAL application

1.	The VAL client provides a new application requirement to the NSCE client, indicating a new service profile for the VAL application. This may be in the form of a change at the application QoS requirements, location requirements, time window requirement, access type preference (e.g., 3GPP, non-3GPP or multi access) service operation change, or other application-related parameters.

2.	The NSCE client sends a network slice adaptation trigger subscribe request to the NSCE server for the VAL application. This trigger may be in the form of exact requested network slice (and optionally DNN) for the VAL UE of the VAL application; or indication that the VAL application needs to be remapped to a different network slice (and optionally DNN). The trigger may also include additional application requirements based on step1, e.g., the requested location criteria, time window.

NOTE 1：How the requested network slice is known by the NSCE client is out of scope of this release.

3.	The NSCE server confirms the subscription and responds with a network slice adaptation trigger subscribe response.

4.	The NSCE server performs the network network slice adaptation per VAL UE as described in step 3- step 4 in clause 9.11.2.2.

5.	When the network slice adaptation completes, and the adaption result comes out and available, the NSCE server provides the network slice adaptation notification to the NSCE client.

# /************************** Next changes ****************************/

9.11.3.aa	Network slice adaptation subscribe request

Table 9.11.3.aa-1 describes the information flow network slice adaptation subscribe request from the VAL server to the NSCE server.

Table 9.11.3.aa-1: Network slice adaptation subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| VAL service ID | M | The VAL service ID of the VAL application for which the network slice adaptation may corresponds to. |
| List of VAL UE IDs | M | List of the VAL UE IDs within the VAL service for which the slice adaptation request corresponds |
| Requested Network slice related identifier(s) | O | Identifier of network slice for which the VAL server requests to use for adaptation |
| Requested monitored Network slice related identifier(s) | O (see NOTE 1) | Identifier of the provisioned network slice(s) which are provisioned for the listed UE(s) and requested to be monitored by NSCE server |
| Requested DNN | O | Indication of the new DNN which is requested. |
| Requested Adaptation threshold | O (see NOTE 2) | The threshold of network slice adaptation |
| >Requested adaptation threshold of the delay of network slice | O | The network slice delay defined clause 5.1.1.1, 5.1.1.2, 5.1.1.3, in 3GPP TS 28.552 [19] and key performance indicators in clause 6.3, in TS 28.554 [20], and network slice related analytics report in clause 8.4.2.4.3, in 3GPP TS 28.104 [21]. |
| NOTE 1: If this IE is not present then the NSCE server monitors all the slices provisioned for the listed UE(s) mentioned in this request. If this IE is present, the NSCE server monitors the network slice(s) only indicated by the identifier(s). NOTE 2: The NSCE is requested to adapt UE to the requested network slice only when the status of the provisioned network slice for the listed UE(s) crosses the requested threshold. |  |  |

9.11.3.bb	Network slice adaptation subscribe response

Table 9.11.3.bb-1 describes the information flow network slice adaptation subscribe response from the NSCE server to the VAL server.

Table 9.11.3.2-1: Network slice adaptation subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M (NOTE) | Identifier of the subscription |
| Result | M | Result includes success or failure of subscription |
| Cause | O | Indicates the cause of failure |
| NOTE: Only present if the result is success. |  |  |

9.11.3.cc	Network slice adaptation trigger subscribe request

Table 9.11.3.cc-1 describes the information flow Network slice adaptation trigger subscribe request from the NSCE client to the NSCE server.

Table 9.11.3.dd-1: Network slice adaptation trigger subscribe request

| Information element | Status | Description |
| --- | --- | --- |
| VAL UE ID(s) | M | The VAL UE ID(s) within the VAL service, for which the network slice adaptation trigger applies |
| VAL service ID | M | The VAL service ID of the VAL application for which the network slice configuration may corresponds to. |
| Requested S-NSSAI | M | Indication of the new S-NSSAI which is requested. |
| Requested DNN | O | Indication of the new DNN which is requested. |
| Request application requirements | O | The application-related request parameters |
| >Requested time window | O | Indication of the new scheduled time window that is requested |
| >Requested location criteria | O | Indication of the new location criteria that is requested |
| >Requested access type reference | O | Indication of the new access type (3GPP, non-3GPP or multi-access) preference that is requested. |
| >Requested UE IP address preservation indicator | O | Indication that UE IP address preservation is requested |

9.11.3.dd	Network slice adaptation trigger subscribe response

Table 9.11.3.dd-1 describes the information flow network slice adaptation trigger subscribe response from the NSCE server to the NSCE client and optionally to the VAL client.

Table 9.11.3.dd-1: Network slice adaptation trigger subscribe response

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M (NOTE) | Identifier of the subscription |
| Result | M | Result includes success or failure of subscription |
| Cause | O | Indicates the cause of failure |
| NOTE: Only present if the result is success. |  |  |

9.11.3.ee	Network slice adaptation notification

Table 9.11.3.bb-1 describes the information flow network slice adaptation notification from the NSCE server to the VAL server or to the NSCE client.

Table 9.11.3.ee-1: Network slice adaptation notification

| Information element | Status | Description |
| --- | --- | --- |
| Subscription ID | M (NOTE) | Identifier of the subscription |
| Adaption result | M | Result includes success or failure of the network slice adaptation with the underlying network. |
| Cause | O | Indicates the cause of failure |

9.11.4	APIs

9.11.4.1	General

Table 9.11.4.1-1 illustrates the APIs for VAL server-triggered and network-based network slice adaptation.

Table 9.11.4.1-1: List of APIs for network slice adaptation

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_ NSCE_NetworkSliceAdaptation | Network_slice_adaptation | VAL server | Request /Response and Subscription/Notify |

Table 9.11.4.1-2 illustrates the APIs for VAL UE-triggered and network-based network slice adaptation

Table 9.11.4.1-2: List of APIs for network slice capability adaptation trigger

| API Name | API Operations | Known Consumer(s) | Communication Type |
| --- | --- | --- | --- |
| SS_ NSCE_NetworkSliceAdaptationTrigger | Network_slice_adaptation_trigger | VAL server | Request /Response and Subscription/Notify |