3GPP TSG-SA WG6 Meeting #59	S6-240624

Athens, Greece 26th February – 1st March 2024	(revision of S6-240061)

Source:	InterDigital Inc., Convida Wireless, LLC

Title:	Pseudo-CR on New solution on spatial anchor discovery

Spec:	3GPP TR 23.700-21 v0.1.0

Agenda item:	8.4

Document for:	Approval

Contact:	michel.roy@interdigital.com

1. Introduction

This contribution proposes a solution for discovering spatial anchors at the application enablement layer.

2. Reason for Change

KI#1 defines an open issue to support discovery of spatial anchors.

3. Conclusions

NA

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-21 v0.1.0.

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.156: "Mobile Metaverse Services; Stage 1".

[3]	3GPP TR 22.856: "Feasibility Study on Localized Mobile Metaverse Services".

[4]	3GPP TR 23.700-77: "Study on system architecture for next generation real time communication services; Phase 2".

[5]	3GPP TR 23.700-70: "Study on architecture enhancement for Extended Reality and Media service (XRM); Phase 2".

[6]	3GPP TR 26.813: "Study of Avatars in Real-Time Communication Services".

[x]	3GPP TS 23.273: "5G System (5GS) Location Services (LCS); Stage 2".

[y]	3GPP TS 23.434: "Service Enabler Architecture Layer for Verticals (SEAL); Functional architecture and information flows".

[z]	3GPP TS 23.502: "Procedures for the 5G System (5GS); Stage 2".

[zz]	3GPP TS 23.288: "Architecture enhancements for 5G System (5GS) to support network data analytics services".

* * * Next Change * * * *

# 7	Solutions

## 7.0	Mapping of solutions to key issues

Table 7.0-1 Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 |
| --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |
| Sol #2 |  |  |  |  |
| Sol #x | X |  |  |  |

* * * Next Change * * * *

## 7.x	Solution #x: Spatial anchor discovery

### 7.x.1	Solution description

#### 7.x.1.1	General

This solution is for KI#1 and addresses the open issue of spatial anchor discovery by the consumer.

This solution is based on the Mobile Metaverse enablement layer architecture. If it is concluded that the Mobile Metaverse service is specified as new SEAL service, then the following changes apply:

- the Mobile Metaverse Enabler Client may be a Mobile Metaverse SEAL client;

- the Mobile Metaverse Enabler Server may be a Mobile Metaverse SEAL server.

In this solution, the mobile metaverse application enablement layer provides capabilities to discover and determine the validity of spatial anchors. Spatial anchors may be discovered and validated based on consumer location and orientation, consumer capabilities and preferences, and spatial anchor configuration and policy.

#### 7.x.1.2	Procedure

Pre-conditions:

- Spatial anchors have been provisioned in the mobile metaverse enabler server

- A VAL client (e.g., an AR application) needs to display enriched localized content to a user

Figure 7.x.1.2-1: Spatial anchor discovery procedure

0.	A metaverse VAL client discover s spatial anchors. The VAL client performs SA discovery using the mobile metaverse enabler client (MMEC) of the UE.

Editor’s Note: It is FFS whether and what spatial anchor requirements may be provided by a VAL client for SA discovery.

1.	The MMEC sends a spatial anchor discovery request to the mobile metaverse enabler server (MMES). The request includes information elements defined in Table 7.x.3-1.

2.	Upon receiving the request, the MMES validates if the requestor is authorized to discover spatial anchors. If the requestor is authorized, the MMES uses the information included the spatial anchor discovery request to determine spatial anchors to include in the discovery response.

If spatial anchor discovery filters are provided in the request, the MMES determines any spatial anchors matching the discovery filters. For each determined spatial anchor, the MMES checks whether the requestor is authorized to discover the spatial anchor based on the identity and location of the requestor and the access controls and service area characteristics of the spatial anchor. To determine a location of interest to the requestor, the MMES may use location information in the request, if available, or obtain location information by invoking the 3GPP Core Network Location Services exposed by the NEF as described in TS 23.273 [x] and TS 23.502 [z], or by invoking the SEAL Location Management APIs as described in TS 23.434 [y].

Additionally, the MMES may obtain and consider data analytics, such as UE mobility prediction, during spatial anchor determination. The MMES may obtain data analytics by invoking NEF APIs as described in TS 23.288 [zz] and TS 23.502 [z].

3.	The MMES sends a spatial anchor discovery response to the MMEC. If the MMES has successfully determined spatial anchors, the response may include a success indication, a list of the determined spatial anchors as defined in Table 7.x.3-2. If the MMES has not determined any spatial anchor, the response may include a failure indication and a cause of failure.

4.	Upon receiving the discovery response, if the response indicates success, the MMEC may store the received spatial anchor information in a spatial anchor cache. The MMEC evaluates the validity conditions of discovered spatial anchors.

5.	The MMEC provides the valid spatial anchor information to the VAL client(s) based on VAL client requirement(s). Upon receiving valid spatial anchor information, the VAL client can access the service(s) associated with each spatial anchor.

NOTE: This procedure may be adapted to allow a VAL server to discover spatial anchor(s).

### 7.x.2	Architecture Impacts

Option #x in clause 6.1 is the basis for this solution.

### 7.x.3	Corresponding APIs

Table 7.x.3-1 shows the request sent by a MMEC to a MMES for the discovery of spatial anchor(s).

Table 7.x.3-1: Spatial anchor discovery request

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identity | M | The identity of the requestor (e.g., MMEC, UE, VAL server) |
| Requestor security credentials | M | The security credentials of the requestor. |
| Location of interest | O | Location of spatial anchors the requestor is interest in. |
| Spatial anchor discovery filters | O | Set of characteristics to determine matching spatial anchors (e.g., spatial anchor location, VAL client type). |

Table 7.x.3-2 shows the response sent by the MetaApp server to the requester for a spatial anchor discovery request.

Table 7.x.3-2: Spatial anchor discovery response

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request (e.g., success or fail), including failure reason if needed. |
| Discovered spatial anchor List | O | List of discovered spatial anchor(s). Each element includes the information described below |
| > Spatial anchor ID | M | Identifier of the spatial anchor. |
| > Spatial anchor location | M | Location of the spatial anchor (e.g., GPS coordinates, civic address, etc.). |
| > Spatial anchor validity criteria | M | The validity criteria of a spatial anchor (e.g., location criteria, spatial criteria, time criteria, orientation, etc.). |
| > VAL server endpoint | M | Endpoint information (e.g. URI, FQDN, IP address) of the service(s) associated with the spatial anchor. |
| > VAL service type | O | Unique identifier that identifies the type of VAL service associated with the spatial anchor (e.g., product locator service). |

### 7.x.4	Solution evaluation

Editor's note:	Evaluation of this solution is FFS.

* * * End of Changes * * * *