3GPP TSG-SA WG6 Meeting #59	S6-240652

Athens, Greece, 26th February – 1st March 2024	(was S6-240373)

Source:	Convida Wireless, LLC

Title:	Management of AIML operations

Spec:	3GPP TS 23.700-82 V0.2.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

This contribution provides a solution for KI #3.

2. Reason for Change

As discussed in a companion discussion paper, AIML enablement can support VAL service users by providing the ability to manage AIML operations . This solution to KI#3 provides the means by introducing  requests configuring the AIML enablement server to manage AIML operations (on bahalf of VAL service users) via operational management profiles.

3. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.2.0.

# *** All new text ***

## 8.x	Solution #x: AIML operational management.

### 8.x.1	Solution description

The following clauses specify procedures, information flows, and APIs for Key Issue #3 to support AIML operational management.

Assumptions:

1.	The proposed solution is based on client-server architecture for AIML operations.

Figure 8.x.1-1: AIML operational management procedure

1.	A VAL server sends an AIML operational management subscription request to an AIML enabler server to schedule AIML operations. AIML operations can consist of data collection, data preparation, exploratory data analysis (EDA), AIML training, and AIML inference. AIML operations can also consist of participation in split AIML or transfer learning operations. The request includes the parameters listed in Table 8.x.3-1.

Editor’s Note: It is FFS whether the AIML operation list in this request includes training and inference.

2.	The AIML enablement server validates the operational management subscription request and verifies the security credentials provided in the request. The AIML enablement server further performs authentication and authorization checks to determine if the requestor is able to schedule AIML operations. If authorized, the AIML enablement server creates an AIML operational management context with the provided information in the request and assigns an AIML operational management identifier.

3.	The AIML enablement server sends an AIML operational management subscription response that includes the status to the operational management request and the assigned AIML operational management identifier.

4.	The AIML enablement server uses the information in the AIML operational management profile and other information provided in step 1 to determine the set of AIML operations (which it needs to perform itself) and of AIML tasks (e.g. for AIML enabler clients).

NOTE 1: The set of AIML operations to be performed at the AIML enabler server, together with the set of  AIML operations and AIML tasks determined based on the step 1 request is termed “AIML operational schedule” for ease of description. How the “AIML operational schedule” is implemented by the AIML enabler is up to implementation.

NOTE 2: “AIML task” is a generic term for AIML processing performed based on AIML enabler server requests (e.g. by AIML enabler clients). Examples of “AIML task” include triggering AIML enabler client to download an ML model and/or model parameters, triggering data collection, etc.

5.   Based on the AIML operational schedule, the AIML  enablement server performs necessary operations. For example, it may perform AIML enablement client selection, AIML model management, etc. This step reuses procedures described in other solutions for each  corresponding AIML operation.

6.	The AIML enablement server sends requests to the determined AIML enablement clients and is notified of task completion.

Editor's note:	The procedures and APIs used for this step are FFS.

7.  For AIML operational management requests that  resulted in  multiple AIML operations and/or requests sent to the AIML enabler clients , the AIML enablement server may update the operational schedule based on the received notifications, operational schedule and local policies. For incomplete// disrupted AIML tasks, the AIML enabler server may determine to perform actions such as changing the selected AIML enablement client, modifying AIML task parameters, etc

8.	The AIML enablement server sends an AIML operational management notification to the VAL server  as listed in Table 8.x.3-3.

### 8.x.2	Architecture Impacts

Editor's note:	This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 8.x.3	Corresponding APIs

Table 8.x.3-1 shows the request  for the AIML operational management procedure.

Table 8.x.3-1: Request for AIML operational management procedure

| Information element | Status | Description |
| --- | --- | --- |
| Requestor identifier | M | The identifier of the requestor. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Application identifier | O | An identifier for the AIML application |
| AIML operational management description | M | Information used to determine the AIML enabler operations to be performed. The parameter may provide a discrete list of AIML operations being requested to be managed by the AIML enablement server. AIML operations can be data collection, data preparation, exploratory data analysis (EDA), AIML training, AIML inference, and participation in split AIML or transfer learning operations. |
| AIML operational management profile | M | Information provided to the AIML enablement server to manage the AIML operational management request, e.g.: ML model parameters, Training requirements, Dataset requirements |
| AIML client selection criteria | M | Criteria use to select AIML clients that may be selected for AIML tasks, including location, connectivity schedule, etc. The AIML server can also use the selection criteria to maintain a AIML client pool to assist with performing the AIML task, e.g. due to inability to complete the AIML task as a result of disruptions in the UE or the network. The selection criteria may be used to specify an AIML client list pre-selected by the requestor. |
| Minimum number of AIML clients | M | A minimum number of AIML enablement clients required to participate in the AIML operation. |
| Scheduling criteria | O | Criteria to determine the AIML operational schedule. For example, data collection may require that AIML enablement clients collect data during certain times. An expiration time of the management request may be included |
| Notification criteria | O | A configuration for the frequency of notifications to the requestor and may be time-based and/or event based. Events may consist of AIML task completion, AIML task percentage completion, error conditions, and other events that may occur during AIML operation. |

Table 8.x.3-2 shows the response for the AIML operational management procedure.

Table 8.x.3-2: Response for AIML operational management procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| Request identifier | M | An identifier to associate with the management request and notifications. |

Table 8.x.3-3 shows the notification for the AIML operational management procedure.

Table 8.x.3-3: Notification for AIML operational management procedure

| Information element | Status | Description |
| --- | --- | --- |
| Request identifier | M | Identifier of the corresponding management request. |
| Completion status | M | Completion percentage of the management request. |
| Result | M | The result of the current AIML task: success or failure. |
| AIML management request status | O | Information related to the status of the management request completion, e.g. statistics, list of AIML enabler clients, etc. |

### 8.x.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# *** End of Changes ***