3GPP TSG-SA WG6 Meeting #59	S6-240671

Athens, Greece February 26th – 1st March 2024	(revision of S6-240378)

Source:	Ericsson

Title:	Introducing AIML Enablement server initiated AIML service advertisement

Spec:	3GPP TR 23.700-82 v0.2.0

Agenda item:	8.3

Document for:	Approval

Contact:	ashish.sanjay.sharma@ericsson.com

1. Introduction

This pCR proposes a solution for the AIML Enablement server to advertise AIML services, AIML tasks information to the AIML enablement consumers, who can provide those services to assist the other interested AIML Enablement consumers looking for the AIML task or services.

2. Reason for Change

There are use cases initiated by consumers like the AIML client to perform tasks like some model offload/training and require AIML enabler server assistance to discover other consumers who can provide task services and perform the task like model offload/training. One side there are consumers who wants other consumers to do the tasks on their behalf. And other side, there are potential consumers who can perform the tasks for other consumers. The potential consumers are not aware of what kind of AIML tasks are available from other consumers. Therefore to offer the service and join any task, it is essential for thepotential consumers(who can provide the task like model offload  service) to be aware of the AIML services and associated task information like task requirements, task KPIs etc. The AIML enabler layer can advertise and facilitate such discovery of tasks for the consumers.

3. Conclusions

<Conclusion part (optional)>

4. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-82 v0.2.0.

* * * First Change * * * *

## 8.x	Solution #x: AIML Enablement server initiated AIML services advertisements

### 8.x.1	General

The following clauses specify procedures, information flows and APIs for Key issues 3,7 to enable the AIML Enablement server initiated AIML services advertisements procedure

Pre-conditions:

1.	The AIML services and their information is available on the AIML Enablement server.

2.	The AIML Enablement server can use discovery process, registration, registration update as triggers to initiate the AIML service advertisement procedure.

### 8.x.2	Procedures

Figure 8.x.2-1: AIML Enablement server initiated AIML services advertisements procedure

- The AIML Enablement server sends the AIML service advertisement request to the AIML Enablement consumer(s)(e.g VAL server, AIML Enablement client). This request contains advertised AIML services info includes AIML type like FL, TL etc, AIML service duration, AIML task container like task compute/network KPI requirements like bandwidth/latency, task type like model training/offload/split, task performance preference criterias like requirement to perform the task in Energy-Efficient task manner etc. The request includes the configuration for the event-based trigger response like change in network parameters or periodic response configuration or immediate response.

Editor's Note 1: Whether the AIML Enablement server uses group based delivery or broadcast or unicast delivery to AIML Enablement consumer(s) is FFS.

Editor's Note 2: other IEs in the request are FFS.

- The AIML enablement consumer sends a AIML service advertisement response to the AIML Enablement server. The response message provides the AIML Enablement consumer identifier(e.g. URI/ID/IPaddress), supported AIML task type, service availability for the task

Editor’s Note 3: other IEs in the response are FFS.

* * * Next Change * * * *

* * * End of Changes * * * *