3GPP TSG-SA WG6 Meeting #59	S6-240677

Athens, Greece, 26th February – 1st March 2024	(was S6-240358)

Source:	Convida Wireless, LLC

Title:	Update to solution 6

Spec:	3GPP TS 23.700-82 V0.2.0

Agenda item:	8.3

Document for:	Approval

1. Introduction

This contribution updates solutions 6, 7, and 8 to add missing sections and corresponding API definitions, removal of editor’s notes, and clarifications to the procedure descriptions.

2. Reason for Change

For solution 6, the editor’s note in 8.6.1 was removed due to the addition of assumption 3. Figure 8.6.1-1 was updated to clarify a VAL server is making the request and updates to text description were made for clarification. Sections 8.6.2 to 8.6.4 from the template were added and updates were made to section 8.6.3.

For solution 7, the editor’s note in 8.7.1 was removed due to the addition of assumption 3. Sections 8.7.2 to 8.7.4 from the template were added and updates were made to section 8.7.3. Clarifications were made to the procedure description.

For solution 8, the editor’s note in 8.8.1 was removed as it does not depend on other procedures. The information about the AIML client capabilities is grouped together into an AIML client profile, which is provisioned to the AIML enablement client as clarify by the update to assumption 3. Sections 8.8.2 to 8.8.4 from the template were added and updates were made to section 8.8.3. Clarifications were made to the procedure description.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.2.0.

# *** First Change ***

## 8.6 	Solution #6: AIML enablement client selection

### 8.6.1	Solution description

The following clauses specify procedures, information flows, and APIs for Key Issue #3 to support AIML enablement client selection.

Assumptions:

1.	The proposed solution is based on client-server architecture for federated learning.

2.	The VAL server has already discovered and received a list of AIML enablement clients that are suitable and have available data for a particular AIML operation.

3.	The discovery operation may be performed as described in solution 7 to find a list of AIML enablement clients.

Figure 8.6.1-1: AIML enablement client selection procedure

1.	A VAL server sends a request to an AIML enablement server to select a list of AIML enablement clients that have been discovered to meet the requirements for AIML operations. The AIML enablement client selection request includes the VAL identifier, security credentials, application identifier, and a list of AIML client IDs for inclusion into an AIML set.

Editor's Note: The IEs in the request are FFS. It is also FFS how the request parameters are used in relationship to other member selection policy provisioned to the AIML enabler layer.

2.	The AIML enablement server validates the selection request and verifies the security credentials provided in the request. The AIML enablement server further performs authentication and authorization checks to determine if the requestor is able to create an AIML set. If authorized, the AIML enablement server creates an AIML set with the provided list of AIML enablement clients as members and assigns an AIML identifier for the set.

NOTE: The AIML enabler server can reuse SEAL group management for any necessary group management.

3.	The AIML enablement server sends an AIML enablement client selection response that includes the status of the selection request and the assigned AIML set identifier.

### 8.6.2	Architecture Impacts

Editor’s note:	This clause provides the architecture impacts of the solution and possible new SA6 capabilities and interfaces.

### 8.6.3	Corresponding APIs

Table 8.6.3-1 shows the request sent by a VAL server to an AIML enablement server for the AIML enablement client selection procedure.

Table 8.6.3-1: Request for AIML enablement client selection procedure

| Information element | Status | Description |
| --- | --- | --- |
| VAL server identifier | M | The identifier of the VAL server. |
| Security credentials | M | Security credentials to authenticate and authorize the requestor. |
| Application identifier | O | An identifier for the AIML application |
| List of AIML client IDs | M | A list of AIML client IDs that was previously discovered for inclusion into an AIML client set. |

Table 8.6.3-2 shows the response sent by the AIML enablement server to the VAL server for the AIML enablement client selection procedure.

Table 8.6.3-2: Response for AIML enablement client selection procedure

| Information element | Status | Description |
| --- | --- | --- |
| Status | M | The status for the request: success or fail. |
| AIML client set identifier | M | An identifier to associate with the set of AIML enablement clients that the VAL server has provided in the request. The AIML client set can be updated by using this identifier. |

### 8.6.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# *** End of Changes ***