3GPP TSG-SA WG6 Meeting #59	S6-240706

Athens, Greece, 26th February – 1st March 2024	(revision of S6-240366)

Source:	Convida Wireless, LLC

Title:	Multi-modal SEALDD policy configuration

Spec:	3GPP TR 23.700-23 V0.1.0

Agenda item:	8.5

Document for:	Approval

Contact:	Catalina Mladin, mladin.catalina@convidawireless.com

1. Introduction

This contribution provides a solution for KI #2 - E2E Multi-Modal Communication Flows.

2. Reason for Change

To effectively support multi-modal application traffic flows with SEALDD, SEALDD servers and clients must have awareness of the individual application flows having multi-modal dependencies on each another. To provide this awareness, the contribution proposes extending the current SEALDD policy functionality to support multi-modal SEALDD policy functionality.

3. Proposal

It is proposed to agree the following changes to 3GPP 23.700-23 V0.1.0.

# *** All new text ***

## 7.x	Solution #X: SEALDD Policy Configuration for multi-modal flows

### 7.x.1	Architecture Impacts

This solution adds support for multi-modal SEALDD policy configuration functionality by enhancing the existing SEALDD policy configuration functionality supported in clause 9.10 of TS 23.433[x]. There are no impacts to the  SEALDD architecture.

### 7.x.2	Solution description

This solution is targeting KI #2: E2E Multi-Modal Communication Flows and specifies enhancements to the SEALDD policy configuration information flows. In this procedure, the VAL server is a specific server for configuring multi-modal SEALDD policies and is different from VAL servers used for VAL application processing.

The SEALDD policy configuration procedure detailed in 3GPP TS23.433[x]  clause 9.10.2.1 is reused with the enhanced information flows below

NOTE: Corresponding enhancements to multi-modal SEALDD policy update and delete procedures will be completed  during the normative phase.

### 7.x.3	Information flows

#### 7.x.3.1	SEALDD policy configuration request for multi-modal flows

Table 7.x.3.1-1 describes the new information elements to be added to the SEALDD policy configuration request in 3GPP TS 23.434[x] Table 9.10.3.1-1  in order to support multi-modal flows.

Table 7.x.3.1-1:  SEALDD policy configuration request IEs for multi-modal flows

| Information element | Status | Description |
| --- | --- | --- |
| Multi-modal VAL flows list | M | List of individual VAL flows applicable to this multi-modal SEALDD policy. |
| > Application traffic identifiers | M | Identifier of the application traffic for this individual VAL flow (e.g. VAL server ID, VAL service ID) |
| > VAL UE identity | O | Identifier of the VAL UE for this individual VAL flow |
| > SEALDD policy | M | SEALDD policy associated with this individual VAL flow as defined in Table 9.10.3.1-1 of TS 23.433 |
| Multi-modal SEALDD policy | M | Multi-modal SEALDD policy applicable to Multi-modal VAL flows list. |
| > Burst policy | O (See NOTE 1, NOTE 2) | Policy used by the SEALDD layer to determine multi-modal data burst operations to perform (e.g., data bursts comprising messages from different VAL flows). |
| > Synchronization policy | O (See NOTE 1, NOTE 3) | Policy used by the SEALDD layer to determine multi-modal synchronization operations to perform (e.g., synchronizing arrival times of messages across different VAL flows). |
| > Measurement policy | O (See NOTE 1, NOTE 4) | Policy used by the SEALDD layer to determine multi-modal measurements to perform to support multi-modal burst and synchronization operations. (e.g., latency or jitter of multi-modal data bursts) |
| > Policy expiration time | O | Indicates the validity period of the policy |
| NOTE 1: At least one of these multi-modal SEALDD policies shall be present in the request NOTE 2: This IE is used for NOTE 3: This IE is used for the synchronization of data packets between NOTE 4: This IE is used for complementing other SEALDD measurement policies for |  |  |

### 7.x.4	Solution evaluation

Editor's note:	This clause provides an evaluation of the solution. The evaluation should include the descriptions of the impacts to existing architectures.

# *** End of Changes ***