3GPP TSG-SA WG6 Meeting #59	S6-240722

Athens, Greece, 26th February – 1st March 2024 	wasS6-240649

Source:	Ericsson

Title:	Solution on Analytics and Assistance Information Collection for Supporting FL Member (Re)Selection

Spec:	3GPP TS 23.700-82 V0.2.0

Agenda item:	8.3

Document for:	Approval

Contact:	Jing Yue (jing.yue@ericsson.com)

1. Introduction

For the AI/ML Enablement Server to coordinate a FL process, analytics and assistance informtion collection to support/assistant the FL member (re)selection is needed. For example, analytics from ADAE server/client can be used to support the AI/ML Enablement Server to get statistics and/predictions on FL member capability and decide or update potential FL member list. Assist from the 5GC NFs (e.g. NEF) on Member UE selection can be used by the AI/ML Enablement Server to obtain assistance informaiton on UE moving and network resource allocation status, etc., to decide or update the FL member list.

2. Reason for Change

Analytics and assistance information from ADAES and 5GC is necessary to support/assistant the AI/ML Enablement Server to coordinating a FL process by (re)selection of FL members. The procedure for AI/ML Enablement Server to get the analytics and assistance information for FL member (re)selection is introduced.

3. Conclusions

This pCR proposes the procedure on analytics and assistance information collection for supporting FL member (re)selection.

4. Proposal

It is proposed to agree the following changes to 3GPP 23.700-82 V0.2.0.

* * * First Change * * * *

## 8.0	Mapping of solutions to key issues

Table 8.1-1: Mapping of solutions to key issues

|  | KI #1 | KI #2 | KI #3 | KI #4 | KI #5 | KI #6 | KI #7 |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Sol #1 |  |  |  |  |  |  |  |
| Sol #2 |  |  |  |  |  |  |  |
| Sol #3 |  | X |  |  |  |  |  |
| Sol #4 |  | X |  |  |  |  |  |
| Sol #5 |  | X |  |  |  |  |  |
| Sol #6 |  |  | X |  |  |  |  |
| Sol #7 |  |  | X |  |  |  |  |
| Sol #8 |  |  | X |  |  |  |  |
| Sol #9 |  |  | X |  |  |  |  |
| Sol #10 |  |  |  |  |  |  |  |
| Sol #X2 | X |  | X |  |  |  |  |

* * * Next Change * * * *

## 8.X2	Solution #X2: Analytics and Assistance Information Collection for Supporting FL Member (Re)Selection

### 8.X2.1	General

The following clauses specify procedures, information flows and APIs for Key Issues 1 and 3 to support AI/ML Enablement Server interact with ADAES and NEF for analytics and assistance informaiton to support FL member (re)selection.

Pre-conditions:

-	FL member (e.g. AI/ML Enablement Client) is registered on the AI/ML Enablement Server to participate in the AI/ML Enablement operations (e.g. ML model training).

### 8.X2.2	Procedure on Analytics/Assistance Informaiton Collection for FL member (re)selection

Figure 8.X2.2-1: Analytics/Assistance informaiton collection procedure for FL member (re)selection

- The consumer (e.g. VAL server) may request AI/ML Enablement Server to start or the AI/ML Enablement Server may is coordinating the AI/ML operations for a FL process. The AI/ML Enablement Server may decide that analytics for analytics is needed.

- The AI/ML Enablement Server may retrieve information from ADAES, by sending analytics (update) request to ADAE server (e.g. UE-to-UE application performance analytics).

Editor’s Note:	It is FFS whether and what more analytics from ADAES is needed.

- The ADAES performs analytics relevant operations to generate the analytics.

- The ADAES provides response/notifications to the AI/ML Enablement Server with the required analytics.

- The AI/ML Enablement Server may retrieve assistance information on FL member (re)selection from NEF, by invoking the Nnef_MemberUESelectionAssistance_Subscribe service operation as defined in clause 5.2.6.32.2 of 3GPP TS 23.502 [6]. The subscription request may contain Member UE filtering criteria and a (update) UE list (which are registered as AI/ML Enablement Clients on the AI/ML Enablement Server).

After deriving (updating) list of candidate UE(s), the NEF notifies the AI/ML Enablement Server with the (derived/updated) list of candidate UE(s) by invoking the Nnef_MemberUESelectionAssistance_Notify service operation as defined in clause 5.2.6.32.4 of 3GPP TS 23.502 [6].

- The AI/ML Enablement Server decides/updates the FL member list for the FL process based on the collected analytics and/or assistance informaiton and may also according to the AI/ML member selection policies.

- If request in step 0, the AI/ML Enablement Server responses to the consumer with the FL member(s) information.

The steps 1-3 and 4 can be performed simultaneously, sequentially, or alternately. For example, if new analytics keeps generated at the ADAES during the FL process, the steps 2-5 may repeat until the end of the FL process. If the analytics request from the AI/ML Enablement Server update dynamically during the FL process, the steps 1-5 may repeat until the end of the FL process.

# *** End of Changes ***