3GPP TSG-SA WG6 Meeting #59	S6-240741

(revision of S6-240602, S6-240395)

Athens, Greece 26th February – 01st March 2024

Source:	KPN N.V., TNO, Novamint, 	Sateliot, Avanti, Novamint, Thales, Airbus, SES, DLR, Fraunhofer HHI, Fraunhofer IIS, IRT Saint Exupéry, ESA, Hughes, Satellite, Applications Catapult, Inmarsat, Viasat, CEWiT, Forsway, Omnispace, Lockheed Martin, OQ Technology, Gatehouse Satcom, Magister Solutions, Ericsson, China Mobile

Title:	New Key Issue on Edge on board NGSO Satellite

Spec:	3GPP TR 23.700-01 v0.1.0

Agenda item:	8.6

Document for:	Approval

Contact:	Yonatan Shiferaw (yonatan.shiferaw@tno.nl), Anthony Pages (anthony.pages@tno.nl)

1. Introduction

This contribution proposes a new KI on placing an Edge on-board an NGSO Satellite.

2. Reason for Change

NGSO satellites are satellites that orbit lower oribit than GSO setellite. The TR 38.811 detials latency and other aspects of NGSO sattilete. To utilize the low latency of NGSO satellite Edge can be place on-board. However, since the satellites are now moving, this may introduce impact on the service provisioning, discovery, and service continuity. This key issue proposes to study this along with deployment.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.700-01 v0.1.0.

* * * First Change * * * *

### 4.x	Key issue #x: Edge on board NGSO Satellite

### 4.x.1	Description

NGSO satellites are satellites moving with respect to the earth surface and they can be deployed in MEO (8,000 km – 20,000 km ) or LEO (400 km – 2000 km) orbits. This will reduce the one way latency. To utilize these reduced latencies UPFs as well as gNBs can be deployed on the NGSO.

### 4.x.2	Open issues

This key issue studies how to deploy EAS(s) and EES(s) on NGSO satellite and whether and how the corresponding discovery, service provisioning and service continuity are impacted.​

- How the EES(s) are placed on board the Satellite.

- How the EAS(s) are placed on board the Satellite.

- Whether and how the discovery and the service provisioning are impacted.​

- Whether and how the service continuity procedures are impacted while the UE is only connected with NTN.

* * * End of Change * * * *