3GPP TSG-SA WG6 Meeting #60	S6-241025

Changsha, China 15th – 19th April 2024	(revision of S6-241xxx)

Source:	NTT Docomo

Title:	Adoption of CAPIF within EDGEAPP for TR 23.946

Spec:	3GPP TR 23.946 v0.3.0

Agenda item:	9.7

Document for:	Approval

Contact:	junpei.uoshima.mt@nttdocomo.com

1. Introduction

This pCR presents the adoption of CAPIF within EDGEAPP, as well as the introduction of CAPIF deployments in relation with EDGEAPP.

2. Reason for Change

If readers want to understand how to adopt CAPIF related to EDGEAPP, they will not find any current description in this TR. Therefore, it is necessary to add information about EDGEAPP.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.946.

* * * First Change * * * *

## 6		Exemplary Use Cases and Adoption

## Editor's Note: This clause will provide the main representative usage examples for CAPIF and adoption in 3GPP, ETSI, etc.

6.x	EDGEAPP deployment in relation with CAPIF

The EDGEAPP provides the capabilities with easy to integrate APIs by allowing application developers to focus better on application features. There are some deployment models in relation with CAPIF in clause A.5 of  3GPP TS 23.558 [r1]. The EES can support EAS's access to northbound APIs exposed by SCEF/NEF by providing distributed CAPIF core functions as shown in Figure A.5.2-1. The EES can support EAS (owned by 3rd party or by PLMN operator) access to northbound APIs exposed by SCEF/NEF by using centralized CAPIF core functions as shown in Figure A.5.3-1. The EES provides support for an EAS to expose its Service APIs (i.e., EAS Service APIs) for consumption by the other EASs by providing CAPIF functions as shown in Figure A.5.4-1.

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[3]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[4]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[5]		ETSI GS MEC 011 V3.1.1: "Multi-access Edge Computing (MEC); Edge Platform Application Enablement".

[6]	3GPP TR 23.958: "Edge Application Standards in 3GPP and Alignment with External Organizations ".

[7]	EVOLVED-5G ICT-41 project, https://evolved-5g.eu

[8]	NEF emulator version 2.2.3, February 2024. https://github.com/EVOLVED-5G/NEF_emulator

[9]	ETSI Software Development Group: "OpenCAPIF", https://ocf.etsi.org

[10]	ETSI Software Development Group GitLab repository: "OpenCAPIF", https://labs.etsi.org/rep/ocf/capif.

[r1]		3GPP TS 23.558: "Architecture for enabling Edge Applications".