3GPP TSG-SA WG6 Meeting #60	S6-241075

Changsha, China 15th – 19th April 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.558 | CR | 0610 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Service continuity in ENS via leading ECSP |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Samsung |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | EDGEAPP_Ph3 |  |  |  |  |  | Date: |  |  | 2024-04-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The ENS service continuity via leading ECSP is not completed. Eees_ACRManagementEvent and Eees_EELManagedACR are the services exposed by EES towards S-EAS over Edge-3 interface. Similar services are required to expose towards partner EES for the scenario where ENS is supported via lead ECSP. For Eees_ACRManagementEvent, the partner EES may or may not have access to core network of lead operator. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Added EES as consumer for Eees_ACRManagementEvent and Eees_EELManagedACR APIs - Add more procedure description for ENS service continuity via leading ECSP. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Missing procedures in supporting service continuity for ENS. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.7.2, |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

6.7.2	APIs provided by the Edge Enabler Layer

Table 6.7.2-1 summarizes the APIs exposed by the ECS.

Table 6.7.2-1: APIs provided by the ECS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eecs_ServiceProvisioning | EEC | 8.3 |
| Eecs_EESRegistration | EES | 8.4.4 |
| Eecs_TargetEESDiscovery | EES, CES | 8.8.3.3 |
| Eecs_ECSRegistration | ECS | 8.17.2.2 |
| Eecs_ECSDiscovery | ECS | 8.17.2.3 |
| Eecs_ECSServiceProvisioning | ECS | 8.17.2.4 |
| Eecs_EASInfoManagement | EES | 8.20.2 |

Table 6.7.2-2 summarizes the APIs exposed by the EES.

Table 6.7.2-2: APIs provided by the EES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EECRegistration | EEC | 8.4.2 |
| Eees_EASRegistration | EAS | 8.4.3 |
| Eees_EASDiscovery | EEC | 8.5 |
| Eees_UELocation | EAS | 8.6.2 |
| Eees_ACRManagementEvent | EAS, EES, CAS | 8.6.3 |
| Eees_AppClientInformation | EAS | 8.6.4 |
| Eees_UEIdentifier | EEC, EAS | 8.6.5 |
| Eees_SessionWithQoS | EAS | 8.6.6 |
| Eees_TrafficInfluenceEAS | EAS | 8.6.7 |
| Eees_TargetEASDiscovery | EAS, EES, CAS | 8.8.3.2 |
| Eees_AppContextRelocation | EEC, EAS, EES | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_EELManagedACR | EES, EAS | 8.8.3.6 |
| Eees_SelectedTargetEAS | EAS, CAS, EES | 8.8.3.7 |
| Eees_ACRStatusUpdate | EAS, CAS, EES | 8.8.3.8 |
| Eees_ACRParameterInformation | EES, CES | 8.8.3.9 |
| Eees_EECContextPull | EES | 8.9.4.2 |
| Eees_EECContextPush | EES, CES | 8.9.4.3 |
| Eees_EASInformationProvisioning | EEC, EES | 8.15 |
| Eees_CommonEasAnnouncement | EES | 8.19 |

NOTE:	The event exposure related APIs (e.g. Eees_EASDiscovery and Eees_ACREvents) can be realized as single event subscription API.

Table 6.7.2-3 summarizes the APIs exposed by the CAS.

Table 6.7.2-3: APIs provided by the CAS

| API Name | Known Consumers | References |
| --- | --- | --- |
| Ecas_SelectedEES | EES | 8.8.3.10 |

Table 6.7.2-4 summarizes the APIs exposed the EEC.

Table 6.7.2-4: APIs provided by the EEC

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eeec_ACRegistration | AC | 8.14.4.2 |
| Eeec_EASDiscovery | AC | 8.14.4.3 |
| Eeec_ACRTrigger | AC | 8.14.4.4 |
| Eeec_Services | AC | 8.14.4.5 |
| Eeec_UEId | AC | 8.14.4.6 |

Table 6.7.2-2 summarizes the EES APIs re-used by the CES.

Table 6.7.2-5: APIs re-used by the CES

| API Name | Known Consumers | References |
| --- | --- | --- |
| Eees_EASRegistration | CAS | 8.4.3 |
| Eees_UELocation | CAS | 8.6.2 |
| Eees_ACRManagementEvent | CAS | 8.6.3 |
| Eees_AppClientInformation | CAS | 8.6.4 |
| Eees_UEIdentifier | CAS | 8.6.5 |
| Eees_SessionWithQoS | CAS | 8.6.6 |
| Eees_TrafficInfluenceEAS | CAS | 8.6.7 |
| Eees_TargetEASDiscovery | EES | 8.8.3.2 |
| Eees_AppContextRelocation | CAS | 8.8.3.4 |
| Eees_ACREvents | EEC | 8.8.3.5 |
| Eees_SelectedTargetEAS | CAS | 8.8.3.7 |
| Eees_ACRStatusUpdate | CAS | 8.8.3.8 |
| Eees_ACRParameterInformation | EES | 8.8.3.9 |
| Eees_EECContextPush | EES | 8.9.4.3 |

* * * First Change * * * *

8.18.2.4	Service continuity for ENS via leading ECSP

8.18.2.4.1	General

The AC connected EAS may be changed when the UE moves within the same partner ECSP' service coverage or to a different partner ECSP' service coverage or back to leading ECSP' service coverage.

S-EAS and T-EAS can be both provided by partner ECSP.

Some EEC request (over EDGE-1) or EAS request (over EDGE-3) may be relayed between leading EES and partner EES (over EDGE-9), with necessary modification (e.g. along with the ECSP ID of the sender). The following identifies a list of relayed procedures:

-	ACR launching

-	Selected T-EAS declaration

-	ACR status update

-	EAS Information provisioning

-	ACR management events

-	EELManagedACR procedure

* * * Next Change * * * *

8.18.2.4.X	ACR management events

Upon receiving ACR management event subscribe request from the EAS (either S-EAS or T-EAS), based on subscribed events, the S-EES may perform folloing:

-	If the EAS is subscribed for "ACT start/stop" or "ACR Selection" event, the partner EES uses the procedure as defined in clause 8.6.3.2.2 to send the subscribe request to the leading EES over EDGE-9 reference point. Upon receiving the response from the leading EES, the partner EES sends the response back to the EAS.

-	If the EAS is subscribed for "user plane path change" or "ACR monitoring" or "ACR facilitation" event, if based on policy, the partner EES can subscribed to the core network of the leading operator, then the partner EES may subscribe for user plane path change report. If based on policy, the partner EES can not subscribed to the core network of the leading operator, the partner EES uses the procedure as defined in clause 8.6.3.2.2 to send the subscribe request to the leading EES over EDGE-9 reference point. Upon receiving the response from the leading EES, the partner EES sends the response back to the EAS.

Upon receiving notification from the leading EES, the partner EES sends the notification to the S-EAS.

8.18.2.4.y	EELManagedACR procedure

Upon receiving EELManagedACR service request from the EAS S-EAS, the partner EES uses the procedure as defined in clause 8.8.3.6 to send the request to the leading EES over EDGE-9 reference point. Upon receiving the response from the leading EES, the partner EES sends the response back to the EAS.

* * * End of Change * * * *