3GPP TSG-SA WG6 Meeting #60	S6-241233

Changsha, P. R. China, 15th – 19th April 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.281 | CR | 0220 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Enhancement to MCVideo with 5G network capabilities |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, Hisilicom |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | enhMC |  |  |  |  |  | Date: |  |  | 2024-04-06 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | In Rel-18, several network capabilities are introduced to enhance the XR service as specified in clause 5.37 of 3GPP TS 23.501. Those capabilities also benefit the MCVideo service to improve the user experience in wirelesss scenario. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce a new clause to define the procedure of enhancing the MCVideo with the 5G network capabilities: L4S marking and QoS monitoring. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The MCVideo cannot benefit from the new 5G network capabilities to improve the user experience. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New {x, x.1, x.2, x.3, x.3.1} |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /******************** First Changes ********************/

x	MCVideo enhancement with 5G network capabilities

x.1	General

These clauses specify the procedure for the MCVideo enhancement with 5GS functionalities defined in clause 5.37 of 3GPP TS 23.501 [7]. With this procedure, the MCVideo service may request the QoS monitoring and obtain the information (i.e., UL/DLcongestion information, UL/DL data rate, UL/DL/RT delay, estimated bandwidth) exposed from 5GC, or trigger the L4S ECN marking and obtain the DL congestion information via MC client, or request and obtains the QoE metrics UE data from the Data collection AF. With the network exposed information, the MC service server may use such information to adapt e.g., the MCVideo media packet sending rate.

NOTE:	How the media packet sending rate is adapted is outside the scope of the present document.

The following MCVideo features are enhanced with 5G network capabilities:

-	One-from-server video pull

x.2	Information flows

Editor's Note:	The definition of this information flow is FFS. Coordination with SA4 may be required.

x.3	Procedures

x.3.1	One-from-server video pull enhancement with 5GS capabilities

Procedure in figure x.3.1-1 is the basic signalling control plane procedures for the one-from-server video pull enhancement with 5GS capabilities.

Pre-conditions:

- The same pre-condition from clause 7.3.2.4.2 of 3GPP TS 23.281 [4] is applied.

MCVideo

server

5GS

MCVideo client 1

1. MCVideo pull from server request

4. Media plane and transmission control established

2. Authorization check

6b. ECN marking for L4S in the 5GS

3. MCVideo pull from server response

5. Video stream from MCVideo server

6a. Request for L4S marking

6c. ECN feedback report

7a. Request for QoS monitoring

7b. QoS monitoring in the 5GS

7c. QoS monitoring notification

9. DL stream control, e.g., adjust packet sending rate

(A) via L4S ECN marking

(B) via QoS monitoring mechanism

8a. Request for QoE metrics

8b. QoE metric collection

8c. QoE metrics report

(C) via Data collection AF

Figure x.3.1-1: one-from-server video pull enhancement with 5GS capabilities

1-5.	Same as step 1 to step 5 in clause 7.3.2.4.2 of 3GPP TS 23.281 [4].

The MCVideo server may determine to use the following options (A) and (B) to obtains the network connectivity information, and (C) to obtain the application QoE metrics information to improve the downlink stream control, e.g.,  media packet sending rate control

- Obtain the network connectivity information via the L4S ECN marking (step 6a~6c)

6a.	The MCVideo server may determine to utilize the 5GS L4S capabilities to obtain the network connectivity status, e.g., congestion. The MCVideo server interacts with the 5GS to request the ECN marking for L4S as specified in 3GPP TS 23.502 [10].

6b.	The 5GS starts to performs the ECN marking as specified in 3GPP TS 23.501 [7].

6c.	The MCVideo client, based on the DL ECN marking, sends the L4S ECN feedback report towards the MCVideo server.

Editor’s Note: How this L4S ECN feedback report is defined is FFS. Coordination with SA4 may be required.

- Obtain the network connectivity information via the QoS monitoring mechanism (step 7a~7c)

7a.	The MCVideo server may determine to utilize the 5GS QoS monitoring mechanism to obtain the congestion information, data rate information, round trip delay information in UL and/or DL directions. The MCVideo server requests the 5GS to perform QoS monitoring as specified in 3GPP TS 23.501 [7], in 3GPP TS 23.502 [10].

7b.	The 5GS performs the QoS monitoring as requested.

7c.	The 5GS provides the QoS monitoring notification as specified in 3GPP TS 23.501 [7], in 3GPP TS 23.502 [10].

- Obtain the MCVideo QoE metric information via the Data Collection AF  (step 7a~7c)

8a.	The MCVideo server may determine to obtains the QoE metric information by invoking the service provided by Data Collection AF including reporting provisioning, event subscribe and notify as specified in 3GPP TS 26.531 [26531], in 3GPP TS 26d.532 [26532].

9.	The MCVideo server, based on information receive in (A), (B), and (C) to perform the DL media stream control, e.g., adjust the media packet sending rate.

Editor’s Note: How this media stream control, e.g., adjust the media packet sending rate is defined is FFS. Coordination with SA4 may be required.

NOTE:	Whether one or any combination of  (A), (B), and (C) is used by the MCVideo server is implementation specific.