3GPP TSG-SA WG6 Meeting #60	S6-241235

Changsha, P. R. China, 15th – 19th April 2024	(revision of S6-24xxxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.280 | CR | 0555 | rev | - | Current version: | 18.9.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | MC gateway UE corrections |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei,Hisilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | MCGWUE |  |  |  |  |  | Date: |  |  | 2024-04-06 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-18 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | Based on the LS from SA3 (S6-240551/S3‑240828), the following clean ups to R18 MCGWUE related content are required - Cl.3 - Definitions related with the function entities designed for the conneciton authorization (MC gateway client, MC gateway UE server, MC gateway UE function) - Cl.8.1.7 - GW MC service ID should be voided and its coressponding usage in other clauses i.e., 11.4.3 and clause 11.5 - Cl.11 11.1 – functionalities clear up to remove the functions cannot be supported in R18. 11.2 – Functional model update to remove the MC gateway client, MC gateway UE server, MC gateway UE function, related reference points, connection authorization of the association b/t MC client on the non-3GPP device and the MC gateway UE, media plane aspect clean up (only unicast delivery from the MC server to the MC client in the non-3GPP device is supported) 11.5 – The whole 11.5 should be voided. - Annex A.7 – The whole A.7 should be voided. - Annex D – should be updated to clean the conneciton authorization related aspects. In addition, the following aspects needs to be considered to improve the TS. (1) The functional model for the two use cases (non-3GPP device can/cannot host the MC client) should be shown individually, as the functions of the MC gateway UE are different. For the case the non-3GPP device can host the MC client, all the CSC-n, MCX-n, SIP-n traffic are end-to-end and terminated at the MC clients within the non-3GPP device, the MC gateway UE only performs the transparently IP packet routing. For the case the non-3GPP device cannot host the MC client, all the CSC-n, MCX-n, SIP-n traffic are terminated at the MC clients at the MC client located within the MC gateway UE, the MC gateway UE delivery the content to the non-3GPP device (outside the scope of SA6) (2) The signalling plane and the application plane are mixed in a single functional model, which may cause confusion. The signalling plane should be introduce to further clarify the content forward function of the MC gateway UE. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Clean up as described in the reason for change part. - Introduce a sperate fuctional model for non-3GPP device can not have the MC client, and introduce the signalling plane for the functional models |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Incorrection functions may be wrongly implemented by the downstream group as well as create confusions to the readers. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 3.1, 8.1.7, 11.1~11.3.3, 11.4.3, 11.5 (including all the subclauses), A.7, D.1, D.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

# /************************** 1st changes ****************************/

3.1	Definitions

For the purposes of the present document, the terms and definitions given in 3GPP TR 21.905 [1] and the following apply. A term defined in the present document takes precedence over the definition of the same term, if any, in 3GPP TR 21.905 [1]. Not all definitions are used in this document.

Accuracy: Reflects the uncertainty of the location at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Active MC service user profile: The MC service user profile that is currently used by an MC service client of an MC service user while receiving MC service.

Ad hoc Group Communication: The combining of a multiplicity of MC service users into a group for the duration of a communication. When the communication is released, the group no longer exists. If the communication is associated with an alert, then the group continues to exist until the alert is also canceled.

Ad hoc Group emergency alert: The combining of a multiplicity of MC service users into a group for sending an emergency alert. When the alert is cancelled, the group no longer exists. If the alert is associated with a communication, then the group continues to exist until the communication is also canceled.

Altitude: Third dimension for the geographical coordinates at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Bearing: Direction at the moment of location measurement, e.g. see 3GPP TS 25.305 [30].

Chat group: An MC service group that is pre-defined with MC service group ID and member list in the group management server. Group members must join the pre-estabslihed group call to participate.

ECGI: E-UTRAN Cell Global Identifier, which is used to identify cells globally, where the ECGI is constructed from the Mobile Country Code (MCC), Mobile Network Code (MNC) and the E-UTRAN Cell Identifier (ECI).

Interconnection: A means of communication between MC systems whereby MC service users obtaining MC service from one MC system can communicate with MC service users who are obtaining MC service from one or more other MC systems.

Interconnection group: An MC service group that is configured to allow inclusion of MC service group members who are MC service users from partner MC system(s).

LCS network: The 3GPP network that provides location service as defined in 3GPP TS 23.271 [29].

Location: The current physical location of the MC service UE.

MBMS SAI: Multimedia Broadcast Multicast Service Area Identity which is mapped to the MBMS service area.

MC gateway server: A server providing topology hiding for MC service interconnection with a partner MC system, where that partner MC system is in a different trust domain.

MC service: A generic name for any one of the three mission critical services: either MCPTT, or MCVideo, or MCData.

MC service affiliated group member: An MC service user who has indicated an interest in a particular MC service group and has been accepted to participate in MC service group communication for that MC service group.

MC service client: A generic name for the client application function of a specific MC service. MC service client could be replaced by MCPTT client, or MCVideo client, or MCData client depending on the context.

MC service group: A defined set of MC service users with associated communication dispositions (e.g. media restrictions, default priority and commencement directions) configured for the use with one or more MC services.

MC service group affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is determined.

MC service group call: A mechanism by which an MC service user can make a one-to-many MC service(s) transmission to other users that are members of MC service group(s).

MC service group de-affiliation: A mechanism by which an MC service user's MC service(s) communication interest in one or more MC service groups is removed.

MC service group home system: The MC system where the MC service group is defined.

MC service group host MC service server: The MC service server within an MC system which provides centralised support for a particular MC service of an MC service group defined in a MC service group home system.

MC service group member: An MC service user, whose MC service ID is listed in a particular MC service group.

MC service ID: A generic name for the user ID of a mission critical user within a specific MC service. MC service ID could be replaced by MCPTT ID, or MCVideo ID, or MCData ID depending on the context.

MC service server: A generic name for the server application function of a specific MC service. MC service server could be replaced by MCPTT server, MCVideo server, or MCData server depending on the context.

MC service user: An authorized user, who can use an MC service UE to participate in one or more MC services.

MC service user profile: The set of information associated to an MC service user that allows that user to employ one or more MC services in a given role and from a given MC service UE.

MC service UE: A UE that can be used to participate in one or more MC services.

MC service UE label: A generic name for identification of a specific MC service UE.

MC system: The collection of applications, services, and enabling capabilities required to provide a single mission critical service or multiple mission critical services to one or more mission critical organizations.

MC user: A user, identified by an MC ID, who, after authorization, obtains mission critical service(s).

Migration: A means for an MC Service user to obtain MC service directly from a partner MC system.

Partner MC system: Allied MC system that provides MC services to an MC service user based on the MC service user profiles that are defined in the primary MC system of that MC service user.

Preconfigured MC service group: an MC service group used only for regrouping that has been configured in advance of a group or user regrouping operation to serve as the source of regroup group configuration.

Pre-arranged group: An MC service group that is pre-defined with MC service group ID and member list in the group management server. Affiliated group members are invited when the group communication is setup.

Pre-selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile through configuration, and applicable for an authenticated MC service user upon MC service authorization.

Primary MC system: MC system where the MC service user profiles of an MC service user are defined.

Requested Priority: A value for use in a MC service group or MC private communication that, if accepted, is used by the MCX service server to temporarily replace the priority level that is predefined in the MC service group or MC service user profile. This value is used in combination with other factors to determine the application priority for the requested communication.

Selected MC service user profile: The MC service user profile that is to be selected as the active MC service user profile for an MC service upon request by an MC service user.

Serving MC service server: The MC service server which is providing MC service to an MC service client.

NOTE 1:	There is one serving MC service server for each MC service, which can be the primary MC service server of the MC service user of the MC service client, or can be a partner MC service server to which the MC service user has migrated.

Serving MC system: The MC system which is providing MC service to an MC user.

NOTE 2:	The MC system can be the primary MC system of the MC service user, or can be a partner MC system to which the MC service user has migrated.

Speed: Movement at the moment of location measurement, e.g. see 3GPP TS 25.305 [30] and 3GPP TS 23.032 [31].

Time of measurement: Date and time expressed with a certain precision to reflect the moment of the location measurement.

For the purposes of the present document, the following terms given in 3GPP TS 22.280 [3] apply

Mission Critical

Mission Critical Applications

Mission Critical Organization

Mission Critical Service

Functional alias

For the purposes of the present document, the following terms given in 3GPP TS 22.179 [2] apply

Multi-talker control

Group-broadcast group

For the purposes of the present document, the following terms related to a MC gateway UE function apply

MC gateway UE:  A functional entity that enables simultaneous access to the MC system for multiple MC clients.

MC client: Aggregates a set of clients (i.e. Group management client, Configuration management client, Identity management client, Key management client, Location management client and MC service client).

MC server: Aggregates a set of servers (i.e. Group management server, Configuration management server, Identity management server, Key management server, Location management server and MC service server) which serves the MC client accordingly.

Non-3GPP device: A device that enables connectivity towards an MC gateway UE using an access method not specified by 3GPP. A subset of these devices can host an MC client specified by 3GPP.

# /*********************** Next Changes ****************************/

8.1.7	Void

# /*********************** Next Changes ****************************/

11.1	General

An MC gateway UE enables MC service access for a MC service user residing on non-3GPP capable devices with the MC service clients and for devices which are unable to host MC service clients. In order to be able to participate in a MC service, the MC gateway UE provides the following necessary functions to enable the MC service access for such MC clients:

-	3GPP transport resources: Sharing of MC gateway UE 3GPP transport resources among the associated MC service clients.

-	MC client identification: Independent MC service client identification, MC service client authorisation and MC service profile association.

-	MC client communication integrity: Ensure that signalling, media content and traffic attributes, e.g. priority and QoS, of communications between the MC system and MC service client remains unaltered.

-	Transparently MC traffic routing:

The MC gateway UE provides the transparently routing without altering the content functions for the following traffic between the MC client at the Non-3GPP device and the MC servers for the case non-3GPP device hosts the MC client:

-	signalling between MC client on the non-3GPP device and MC service server/CSC server

-	signaling between the signaling user agent residing on the non-3GPP device and the SIP core

-	signaling between the HTTP client residing on the non-3GPP device and the HTTP proxy

-	Media between the MC service server and the MC clients on the non-3GPP device

For the case non-3GPP device without the MC client, the MC gateway UE provides the MC clients and the transparently routing without altering the content functions for the traffic exchanged between the non-3GPP device and the MC client at the MC gtateway UE.

NOTE 1:	The traffic exchange between the non-3GPP device and the MC client at the MC gtateway UE is outside the scope of the present document, and implementation specific.

NOTE 2:	Further information about MC gateway UE selection are summarized in Annex D.

11.2	Functional Model

11.2.0	General

The MC gateway UE offers access to the MC server for several MC clients as shown in Figure 11.2.0-1. The MC clients can be either located in the MC gateway UE or in non-3GPP devices connected to the MC gateway UE via non-3GPP access.

Figure 11.2.0-1: Functional architecture

For non-3GPP devices which can host an MC client, the MC client at non-3GPP device is used by the MC service user to access the MC service system.

For non-3GPP devices which cannot host MC clients, the MC gateway UE hosts the instantiation of the MC client which is used by the MC service user to access the MC system, i.e., the MC gateway UE instantiates an MC client, acting on behalf of the non-3GPP device, to request services (e.g. emergency call, group calls, short data messages services, etc.) towards the MC service system.

NOTE 1:	The communication interworking and the definition of associated procedures between the MC client (initiated at the MC gateway UE) and the non-3GPP devices is out of scope of this document.

NOTE 2:	MC clients residing on a non-3GPP device cannot use UICC credentials to perform authorisation with the 3GPP transport system.

If the MC service user on the non-3GPP device utilizes multiple MC services simultaneously, the MC service access may also be provided by one or multiple MC gateway UEs as shown in figure 11.2.0-2 while restricting each MC service to one MC gateway UE (e.g. MCPTT via MC gateway UE1, MCData via MC gateway UE2).

Figure 11.2.0-2: Simultaneous multiple MC gateway UE use by a single non-3GPP device

NOTE 2:	Even not shown in the above figure, the same principle of simultaneous use of multiple MC gateway UEs is applied for non-3GPP devices which cannot host an MC client.

11.2.1	Functional model

Before serving the MC service user using non-3GPP device behind the MC gateway UE, the MC gateway UE shall complete the UE authentication with the 3GPP network.

For both the non-3GPP device host the MC client and the non-3GPP device without hosting the MC client, the authentication and authorization of the MC client for access the MC service follows the procedure defined in clause 10.6.

Figure 11.2.1-1 illustrates the functional model for the application plane for the MC system to support a MC service user to participate in a MC service via the MC gateway UE using a non-3GPP device host MC clients.

CSC server(s)

3GPP network

MC service server

Common service core

MC gateway UE

Non-3GPP device

MC service client

CSC client

MCX-n

CSC-n

Figure 11.2.1-1: Functional model of MC gateway UE application plane (non-3GPP device host client)

Figure 11.2.1-2 illustrates the functional model for the signalling control plane corresponding to figure 11.2.1-1.

Common services core

MC service server

MC gateway UE

3GPP network

SIP database

Rx

SIP core

AAA-1

SIP-2

HTTP-1

To other SIP core

Registrar finder/

I-CSCF

Local proxy/P-CSCF

Registrar-

AS selection

/S-CSCF

HTTP client

SIP-3

HTTP server

HTTP Proxy

HTTP-3

HTTP-2

To other HTTP Proxy

MC gateway server

SIP AS

SIP AS

HTTP client

HTTP server

SIP AS

HTTP-1

HTTP-2

HTTP-3

SIP-3

Other MC system

SIP core

SIP-2

Diameter proxy

AAA-2

Non-3GPP device

Signalling user agent

HTTP client

SIP-1

HTTP-1

HTTP  proxy

Figure 11.2.1-2: Functional model of MC gateway UE signalling plane (N3GPP device host client)

Figure 11.2.1-3 shows the functional model for the application plane for the MC system to support a MC service user to participate in a MC service via the MC gateway UE using a non-3GPP device without MC clients.

MC service client

N3GPP device

CSC client

MC gateway UE

CSC server(s)

3GPP network

MC service server

Common service core

CSC--n

MCX-n

Figure 11.2.1-3: Functional model of MC gateway UE application plane (non-3GPP device not host MC client)

The functional model for the signalling control plane as described in figure 7.3.1-2 is applied to figure 11.2.1-3.

#### 11.2.2	Reference points

11.2.2.1	General

The reference points for the use of the MC gateway UE are described in the following subclauses.

11.2.2.2	Void

11.2.2.3	Void

11.2.2.4	Reference points MCX-n, CSC-n, SIP-1 and HTTP-1

The reference points MCX-n and CSC-n belonging to the application plane and the reference points SIP-1 and HTTP-1 belonging to the signaling control plane. The content over those reference points are transparently routed by the MC gateway UE between MC client on the non-3GPP device towards the MC gateway UE and the corresponding MC service server without altering the content.

MCX-n, CSC-n, SIP-1 and HTTP-1 reference points are specified in the present specification. The MCX-n reference points are specified in 3GPP TS 23.379 [16], 3GPP TS 23.281 [12] and 3GPP TS 23.282 [13].

NOTE:	The MCPTT-8/9, MCVideo-8/9, MCData-FD-5 are not supported by MCX-n.

#### 11.2.3	Media plane aspects

Only unicast media from the MC system to the MC client located in the non-3GPP device is supported.

NOTE 1:	Multicast/broadcast media (e.g., MCPTT-8/9, MCVideo-8/9, MCData-FD-5) from the MC system to the MC client in the non-3GPP device is supported in the present document.

For the case the non-3GPP device is able to host the MC client, all the CSC-n, MCX-n, SIP-n traffic are end-to-end and terminated at the MC clients at the non-3GPP device, the MC gateway UE only performs the transparently IP packet routing.

For the case the non-3GPP device is not able to host the MC client, all the CSC-n, MCX-n, SIP-n traffic are terminated at the MC clients at the MC client located within the MC gateway UE, the MC gateway UE exchange the content with the non-3GPP device.

11.3	Using identities behind the MC gateway UE

11.3.1	General

The MC gateway UE enables access to the MC system for MC clients either hosted on the MC gateway UE or hosted on non-3GPP devices using non-3GPP connectivity with the MC gateway UE.

An MC service user behind the MC gateway UE has no 3GPP transport access credentials (i.e., no UICC), whereas the MC system relies on private user identity associated with public user identities (i.e., IMPU/IMPI) needed for authentication purposes. Moreover, the one-to-one correspondence between MC service ID and IMPUs enables routing of signalling traffic between the MC service server and the MC service client.

The use of the various mission critical user identities is determined by clause 8 of the present document.

11.3.2	When using separate IMS subscriptions

If the MC clients are hosted by non-3GPP devices or the MC clients are hosted by the MC gateway UE, an MC service user corresponding to an MC client uses a dedicated IMS/SIP subscription without any relation to the MC gateway UE subscription.

Figure 11.3.2-1 depicts the use of the IMS Credential (IMC) application in accordance with 3GPP TS 23.228 [9].

Figure 11.3.2-1: IMC per MC service client

11.3.3	When sharing MC gateway UE's IMS subscription

The MC clients, instantiated at the MC gateway UE on behalf of the non-3GPP devices which cannot host an MC client, rely on MC service IDs provided by the MC gateway UE. In this case, the MC clients instantiated at the MC gateway UE share the credentials from the IMS subscription of the MC gateway UE, as shown in Figure 11.3.3-1.

Figure 11.3.3-1: Sharing MC gateway UE's IMC for non-3GPP devices which cannot host a client

# /*********************** Next Changes ****************************/

### 11.4.3	IP address association using the MC gateway UE

If the MC client relies on IP address provided by the MC gateway UE, the MC gateway UE shall store the correlation between the non-3GPP device and the IP address (MC gateway UE IP and the local IP) used by the MC client. Subsequent procedures initiated by the MC client, i.e. SIP registration, user authentication and service authorisation uses the MC gateway UE's IP address.

The MC gateway UE assigns the local IPs from address pool which it maintains to the individual MC clients. Managing the local IP addresses, re-using the local IP address, and how the traffic is routed between the MC clients and the network is left for implementation.

Whether and how the service continuity procedures are impacted

Whether and how the service continuity procedures are impacted

Hosted by a non-3GPP device

Whether and how the service continuity procedures are impacted

Whether and how the service continuity procedures are impacted

MC gateway UE

Whether and how the service continuity procedures are impacted

MC servers

MC client(s)

Hosted by a MC gateway UE

MC client(s)

Local IP

MC gateway UE IP

IP address mapping

MCX-n

CSC-n

SIP-n

MCX-n

CSC-n

SIP-n

Figure 11.4.3-1: non-3GPP device uses MC gateway UE's IP address

# /*********************** Next Changes ****************************/

## 11.5	Void

--

-

-

|  |  | - |
| --- | --- | --- |

-

-

-

-

-

-

-

-

-

-

# /*********************** Next Changes ****************************/

A.7	Void

# /*********************** Next Changes ****************************/

Annex D (informative):
Consideration MC gateway UE selection

D.1		General

Operating conditions can influence the selection and the use of the MC gateway UE and its response/decision on providing an MC client connection to the MC server.

Such status information can be considered by the MC client either to connect to another MC gateway UE (e.g. with better operating conditions for the requested service), or to re-connect to the same MC gateway UE (e.g. reattempt connection after a certain time).

D.2		Potential Conditions

Possible operating conditions information could belong to one of the following categories:

MC gateway UE capacity limits

The MC gateway UE might not be able to allow MC clients to connect when the limit of connected MC clients has been reached.

3GPP access congestion

The MC gateway UE can make use of to the special Categories/Access Identity, the preferential access applies based on its assigned Access Control Class/Access Identity during periods of congestion. The MC gateway UE subscription allows to obtain priority treatment for the required communication.

Nevertheless, the serving access network can indicate to the UEs (including MC gateway UE) that Access Class Barring applies and this can be passed by the MC gateway UE to the MC client located in the non-3GPP device.

PLMN reselection and migration

Depending on operator policy, roaming agreements, and on national/regional regulatory requirements MC gateway UE connection and reconection with the corresponding MC server are necessary (e.g. to handle the IP connectivity, QoS, etc.).

Network status information

If the network status information of an EPS/5GS capability available at MC gateway UE, it can be further propagated to the corresponding MC clients located in the non-3GPP device for further processing (e.g. if the requested QoS for service can be provided for MC client or not).