3GPP TSG-SA WG6 Meeting #60	S6-241279

Changsha, China 15th – 19th April 2024	(revision of S6-241xxx)

| CR-Form-v12.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.433 | CR | 0068 | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correct SEALDD connection status event |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | SA6 |  |  |  |  |  |  |  |  |  |
| Work item code: | SEALDD_Ph2 |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-16 (Release 16) Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) |  |
| Reason for change: |  | The IE tables needs to be corrected for SEALDD connection status event, e.g. report id can re-use event id. |  |  |  |  |  |  |  |  |
| , st |  |  |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Remove report id - Clarify the IE applicability |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | IE redundancy; unclarity for the IE related to SEALDD client connection status. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.2.3.7, 9.2.3.8, 9.2.3.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | N | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | N | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | N | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

#### 9.2.3.7	SEALDD connection status subscription request

Table 9.2.3.7-1 describes the information flow from the VAL server to SEALDD server to subscribe to SEALDD connection status information.

Table 9.2.3.7-1: SEALDD connection status subscription request

| Information element | Status | Description |
| --- | --- | --- |
| VAL server ID | M | Identity of the VAL server |
| Event ID list | M | Identifies a list of events such as establishment, release, congestion report, SEALDD client connection status. |
| VAL service ID | O | Identity of the VAL service |
| Identity | O | Identifier of VAL UE or VAL user. |
| SEALDD-S Data transmission connection information | M | Address/port of the VAL server to send/receive the application packets to/from the SEALDD server. |
| Immediate reporting flag | O | Indicates the immediate reporting of connection status notification |
| SEALDD client connection status check periodicity | O | Indicates the frequency to perform SEALDD client connection status check |

#### 9.2.3.8	SEALDD connection status subscription response

Table 9.2.3.8-1 describes the information flow from the SEALDD server to VAL server for responding SEALDD connection status subscription request.

Table 9.2.3.8-1: SEALDD connection status subscription response

| Information element | Status | Description |
| --- | --- | --- |
| Result | M | Success or failure. |
| Subscription ID | O (NOTE) | Subscription identifier corresponding to the subscription. |
| Expiration time | O (NOTE) | Indicates the expiration time of the subscription. |
| NOTE: These IEs shall be present when the result is success. |  |  |

#### 9.2.3.9	SEALDD connection status notification

Table 9.2.3.9-1 describes the information flow from the SEALDD server to the VAL server to notify SEALDD connection status.

Table 9.2.3.9-1: SEALDD connection status notification

| Information element | Status | Description |
| --- | --- | --- |
| Event ID | M | Identifies event such as establishment, release, congestion report, SEALDD client connection status. |
| Identity | M | Identifier of VAL UE or VAL user. |
| VAL service ID | M | Identity of the VAL service. |
| SEALDD connection establishment data | O (See NOTE 1) | Data related to SEALDD connection establishment. |
| > SEALDD-S Data transmission connection information | M | Address/port of the SEALDD server to send/receive the application packets to/from the VAL server. |
| > SEALDD communication lifetime | O | Identifies the DD communication lifetime. |
| Congestion level | O (See NOTE 2) | The congestion level of the VAL service |
| SEALDD client connection status | O (See NOTE 3) | Indicates the connection status of VAL UE. E.g. reachable, unreachable, sleeping |
| NOTE 1: This IE is used for the establishment event. NOTE 2: This IE is used for the congestion report event. NOTE 3: This IE is used for the SEALDD client connection status event. |  |  |

* * * End of Changes * * * *