3GPP TSG-SA6 Meeting #60	S6-241329

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.286 | CR | 0082 | rev | 1 | Current version: | 17.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network | X |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Correction of references to other specifications |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Vodafone |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | eV2XAPP |  |  |  |  |  | Date: |  |  | 2024-04-05 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-17 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | In Rel-17, references were added to SA2 spec. 23.287 "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services". Many references were incorrectly indicated as reference [8] (TS 23.682) instead of TS 23.287, which is reference [19]. In Rel-18, further references were added to SA2 spec. 23.287 "Architecture enhancements for 5G System (5GS) to support Vehicle-to-Everything (V2X) services" but with [x] and not a number. Also, the Rel-18 version of TS 23.286 has two instances of Annex D. |  |  |  |  |  |  |  |  |
| Summary of change: |  | (Rel-18 only) One instance of Annex D (Change history) renumbered to Annex E references to TS 23.287 corrected to [19] and added where missing. (Rel-18) one missing reference number ([21]) to TS 23.222 added |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Spec. contains two instances of Annex D, incorrect and missing references to TS 23.287, TS 23.222 (Rel-18). |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.6.2.2, 9.6.4.1, 9.6.4.2, 9.7.4.2, 9.17.2.3, 9.20.3, 9.20.4 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

3GPP TS 23.286 V17.4.0 (2022-06)

*** First Change ***

#### 9.6.2.2	Set PC5 parameters request

Table 9.6.2.2-1 describes the information flow set PC5 parameters request from the VAE server to the VAE client.

Table 9.6.2.2-1: Set PC5 parameters request

| Information element | Status | Description |
| --- | --- | --- |
| V2X UE ID | O | Identity of the V2X UE to which the configuration is required |
| PC5 parameters configuration data | M | The PC5 parameters configuration data as specified in 3GPP TS 23.285 [5] and 3GPP TS 23.287 [19]. For PC5 provisioning in multi-operator V2X services (as specified in clause 9.15), these parameters may be the joint provisioning policies/parameters to be used by the V2X-UEs within the V2X service. |

*** Next Change ***

### 9.6.4	PC5 parameters provisioning

#### 9.6.4.1	General

This subclause describes the procedure for the VAE server provisioning the PC5 parameter provisioning information to the V2X UE via V1-AE reference point for V2X communication on PC5. The configuration information includes the configuration parameters specified in 3GPP TS 23.285 [5], 3GPP TS 23.287 [19] and additional application related configuration parameters.

#### 9.6.4.2	Procedure

Figure 9.6.4.2-1 illustrates the procedure for provisioning PC5 parameters to the V2X UE via V1 reference point.

Pre-conditions:

1.	The V2X UE has discovered and connected to the VAE server.

2.	If multicast delivery mode is used, the MBMS bearer being used is activated by the VAE server.

Figure 9.6.4.2-1: PC5 parameters provisioning

1.	The VAE server sends the set PC5 parameters request to the VAE client with the information of the PC5 parameters. The PC5 parameters information are specified in subclause 4.4.1.1.2 in 3GPP TS 23.285 [5] and in subclause 5.1.2.1 in 3GPP TS 23.287 [19]. This message can be sent via unicast or multicast.

2.	Upon receiving the set PC5 parameters request, the VAE client stores the received PC5 parameters in the V2X UE.

3.	The VAE client provides an acknowledgement by sending the set PC5 parameters response to the VAE server.

*** Next Change ***

### 9.7.4	Notifications for network monitoring information

#### 9.7.4.1	General

This subclause describes the procedure for notification of network monitoring information from the VAE server.

#### 9.7.4.2	Procedure

Figure 9.7.4.2-1 illustrates the procedure where the VAE server sends notification of the network monitoring information to the V2X UEs.

Pre-conditions:

1.	The VAE server acting as a SCS/AS is configured with the SCEF information and is authorized to exchange information with the SCEF as specified in subclause 4.9 in 3GPP TS 23.682 [8]. The VAE server has subscribed to QoS notification service from 5GS (e.g. PCF/NWDAF). The notification may either include the request for QoS sustainability events as specified in 3GPP TS 23.288 [24] or can include a QoS change notification requests as provided by SMF and specified in 3GPP TS 23.287 [19];

2.	The V2X UE1 and V2X UE2 have subscribed for the network monitoring information at the VAE server.

3.	The VAE server may be aware of the RAT type the V2X UE supports from the registration of VAE client.

Figure 9.7.4.2-1: Notifications for network monitoring information

1.	The VAE server communicates with the SCEF to receive network monitoring information as per the procedure in subclause 4.9 of 3GPP TS 23.682 [8], and/or communicates with NEF or NWDAF to receive QoS monitoring information and/or data analytics information of 5GS as defined in 3GPP TS 23.288 [24]. The VAE server may determine to receive network monitoring information from EPC or 5GC or both considerting the RAT type provided by the UE. The VAE server subscription to QoS monitoring service from 5GS (e.g. PCF/NWDAF) may be active for a certain period of time or a given geographical area. The monitoring may either include the request for QoS sustainability events as specified in 3GPP TS 23.288 [24], or can include a QoS change notification requests as provided by SMF and specified in 3GPP TS 23.287 [19]. The reporting may be configured by the application enabler layer for a given area, time, periodicity etc taking into account the service requirement and other parameters (e.g. expected congestion in certain area, time of the day, road conditions). Based on the subscription, as specified in 3GPP TS 23.287 [19], 5GS provides the extended QoS monitoring report, over N33 interface. This report may come either from NWDAF or SMF via PCF/NEF.

2.	The network monitoring information (e.g. uplink or downlink link degradations, congestions, etc.) available at the VAE server about the on-going V2V session(s) and the network status is based on the original network monitoring information provided by the SCEF in step 1, and/or based on QoS monitoring report or data analytics from 5GS as specified in step 1. The VAE server determines the network monitoring information to send to the UE.

3.	The monitoring information is sent to the subscribed V2X UEs via network monitoring information notification.

*** Next Change ***

#### 9.17.2.3	V2X groupcast/broadcast configuration request

Table 9.17.2.3-1 describes the information flow V2X groupcast/broadcast configuration request from the VAE server to the VAE client.

Table 9.17.2.3-1: V2X groupcast/broadcast configuration request

| Information element | Status | Description |
| --- | --- | --- |
| VAE Server ID | M | Identity of the VAE server which is requester of the V2X groupcast/broadcast configuration |
| V2X group ID | M (NOTE) | Identity of the V2X group for which the V2X groupcast/broadcast configuration is requested. |
| V2X service ID | M (NOTE) | The V2X service ID for which the groupcast/broadcast configuration is requested |
| PC5 Provisioning policies/ parameters | M | PC5 provisioning policies/parameters to be used by the V2X-UEs within the V2X service (list provided in TS 23.287 [19]) |
| Relay V2X-UE ID list | O | List of identities of the V2X UEs to serve as application layer relays. |
| Minimum number of re-transmissions | O | Minimum Number of allowed re-transmissions for the V2X message delivery. |

*** Next Change ***

### 9.20.3	Subscription Procedure

Pre-conditions:

1.	The VAE server includes an Application Function (AF) functionality, as defined in 3GPP TS 23.287 [19].

2.	The VAE server has subscriptions with SEAL servers to receive V2X service-related information (e.g. UE IDs, location, group, configuration info, etc.).

Figure 9.20.3-1: Subscription for monitoring and control of QoS for eV2X communications

1.	The V2X application specific server sends a service adaptation and QoS control subscription request to the VAE server.

2.	The VAE server, who acts as an AF, subscribes to QoS monitoring service from 5GS (e.g. PCF/NWDAF). This subscription may be active for a certain period of time or a given geographical area. The monitoring may either include the request for QoS sustainability events as specified in 3GPP TS 23.288 [24], or can include a QoS change notification requests as provided by SMF and specified in 3GPP TS 23.287 [19]. The reporting may be configured by the application enabler layer for a given area, time, periodicity etc taking into account the service requirement and other parameters (e.g. expected congestion in certain area, time of the day, road conditions).

3.	The VAE server sends a service adaptation and QoS control subscription response indicating success or failure of subscription.

### 9.20.4	Notification Procedure

Pre-condition:

1.	The subscription procedure specified in clause 9.20.3 is performed.

Figure 9.20.3-1: Notification for monitoring and control of QoS for eV2X communications

1.	Based on the subscription, as specified in 3GPP TS 23.287 [19], 5GS provides the extended QoS monitoring report, over N33 interface. This report may come either from NWDAF or SMF via PCF/NEF.

2	The VAE server, based on the monitoring events (step 1) for one or multiple V2X-UEs within one service or across multiple services, may trigger a service requirement adaptation (step 3) based on the actual or expected QoS change for one or more V2X services.

3.	The VAE server sends a service requirement adaptation notification request to V2X application specific server to identify the action which is needed, based on the actual or expected QoS change. This could be the adaptation of the Level of Automation (LoA) for one or more V2X-UEs or V2X services.

4.	The V2X application specific server decides whether to adapt the service requirement based on step 3.

5.	The V2X application specific server sends a response to VAE server with a positive or negative acknowledgment. Optionally, this may include the list of a sub-set of V2X-UEs within a service for which the requirement change should be applied

6.	If no QoS adaptation is required, V2X application layer may adapt based on the adapted service requirements; and steps 7-10 are optional.

7.	If a QoS adaptation is required, the VAE server, triggers the adaptation of QoS for the affected V2X-UE(s) within the service or across multiple services in close vicinity.

8.	The VAE server performs network resource adaptation by interacting with the NRM server as specified in the 3GPP TS 23.434 [10].

9.	The VAE server also notifies about the QoS adaptation to the V2X application specific server.

10.	The V2X application layer adapts based on the adapted service and QoS requirements.

*** End of Changes ***