3GPP TSG-SA WG6 Meeting #60	S6-241351

Changsha, China 15th – 19th April 2024	(revision of S6-241024)

Source:	NTT Docomo

Title:	Adoption of CAPIF within GSMA OP for TR 23.946

Spec:	3GPP TR 23.946 v0.3.0

Agenda item:	9.7

Document for:	Approval

Contact:	junpei.uoshima.mt@nttdocomo.com

1. Introduction

This pCR presents the adoption of CAPIF within GSMA Operator Platform, as well as the introduction of a CAPIF deployment via GSMA Operator Platform.

2. Reason for Change

If readers want to understand how to adopt CAPIF related to GSMA Operator Platform, they will not find any current description in this TR. Therefore, it is necessary to add information about GSMA Operator Platform.

3. Proposal

It is proposed to agree the following changes to 3GPP TR 23.946.

* * * First Change * * * *

## 6		Exemplary Use Cases and Adoption

## Editor's Note: This clause will provide the main representative usage examples for CAPIF and adoption in 3GPP, ETSI, etc.

6.x	GSMA Operator Platform deployment via CAPIF

The GSMA Operator Platform defines a common platform exposing operator services/capabilities to customers/application providers in the 5G-era. It is defined the requirement of an Operator Platform’s SBI-NR with CAPIF in clause 5.1.4.2.2 of GSMA PRD OPG.02 [r1] and it is described the application session continuity support for handovers between 4G and 5G by based on combined SCEF+NEF via CAPIF in clause 5.2.2.6.7 of GSMA PRD OPG.02 [r1].

Figure 6.x-1 illustrates integrated fundamental GSMA Operator Platform deployment of the SCEF and the NEF with the CAPIF. In this deployment model, the GSMA Operator Platform functions as the API invoker.

NOTE:	An Operator Platform’s SBI-NR can work without CAPIF. If CAPIF is not supported, the SBI-NR API will provide an alternate means of providing these functions.

Figure 6.x-1: Integrated GSMA Operator Platform deployment of the SCEF and the NEF with the CAPIF

* * * Next Change * * * *

2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 23.222: "Common API Framework for 3GPP Northbound APIs; Stage 2".

[3]	3GPP TS 29.222: "Common API Framework for 3GPP Northbound APIs; Stage 3".

[4]	3GPP TS 33.122: "Security Aspects of Common API Framework for 3GPP Northbound APIs".

[5]		ETSI GS MEC 011 V3.1.1: "Multi-access Edge Computing (MEC); Edge Platform Application Enablement".

[6]	3GPP TR 23.958: "Edge Application Standards in 3GPP and Alignment with External Organizations ".

[7]	EVOLVED-5G ICT-41 project, https://evolved-5g.eu

[8]	NEF emulator version 2.2.3, February 2024. https://github.com/EVOLVED-5G/NEF_emulator

[9]	ETSI Software Development Group: "OpenCAPIF", https://ocf.etsi.org

[10]	ETSI Software Development Group GitLab repository: "OpenCAPIF", https://labs.etsi.org/rep/ocf/capif.

[r1]		GSMA PRD OPG.02 - “Operator Platform Telco Edge Requirements Version 6.0”, https://www.gsma.com/futurenetworks/wp-content/uploads/2024/02/OPG.02-v6.0-Operator-Platform-Requirements-and-Architecture.pdf.