3GPP TSG-SA6 Meeting #60	S6-241366

# Changsha, Hunan Province, China, 15th Apr 2024 - 19th Apr 2024	was S6-241022

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 23.283 | CR | 0075 | rev | 1 | Current version: | 18.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network | x |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Updating information flows for private call to support interworking with GSM-R |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Kontron Transportation France, Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | S6 |  |  |  |  |  |  |  |  |  |
| Work item code: | FRMCS_Ph5 |  |  |  |  |  | Date: |  |  | 2024-04-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Some elements are missing in the information flows for private to support interworking with GSM-R. This CR adds the missing elements. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 10.4.1.2: Adding: - functional alias of the called party - Requested priority - Transfer indicator - Forwarding indicator - Remotely initiated call request indicator - Additional application specific data (as used by GSM-R) 10.4.1.3: - Adding Additional application specific data (as used by GSM-R) 10.4.1.5: - Additional application specific data (as used by GSM-R) |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Information flows for private call do not support interworking with GSM-R |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 2, 10.4.1.2, 10.4.1.3, 10.4.1.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | x | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

* * * First Change * * * *

# 2	References

The following documents contain provisions which, through reference in this text, constitute provisions of the present document.

-	References are either specific (identified by date of publication, edition number, version number, etc.) or non-specific.

-	For a specific reference, subsequent revisions do not apply.

-	For a non-specific reference, the latest version applies. In the case of a reference to a 3GPP document (including a GSM document), a non-specific reference implicitly refers to the latest version of that document in the same Release as the present document.

[1]	3GPP TR 21.905: "Vocabulary for 3GPP Specifications".

[2]	3GPP TS 22.280: "Mission Critical Services Common Requirements (MCCoRe); Stage 1".

[3]	3GPP TS 22.179: "Mission Critical Push to Talk (MCPTT); Stage 1".

[4]	3GPP TS 22.282: "Mission Critical Data services".

[5]	3GPP TS 23.280: "Common functional architecture to support mission critical services; Stage 2".

[6]	3GPP TS 23.282: "Functional architecture and information flows to support Mission Critical Data (MCData); Stage 2".

[7]	3GPP TS 23.379: "Functional architecture and information flows to support Mission Critical Push To Talk (MCPTT); Stage 2".

[8]	3GPP TS 33.180: "Security of the mission critical service"

[9]	TIA-603-D: "Land Mobile FM or PM Communications Equipment Measurement and Performance Standards".

[10]	ETSI TS 103 389: "Rail Telecommunications (RT); Global System for Mobile communications (GSM); Usage of Session Initiation Protocol (SIP) on the Network Switching Subsystem (NSS) to Fixed Terminal Subsystem (FTS) interface for GSM Operation on Railways".

[11]	ETSI TS 102 610: "Railways Telecommunications (RT); Global System for Mobile communications (GSM); Usage of the User-to-User Information Element for GSM Operation on Railways".

* * * Next Change * * * *

#### 10.4.1.2	IWF private call request

Table 10.4.1.2-1 describes the information flow IWF private call request from the MCPTT server to the IWF and from the IWF to the MCPTT server.

Table 10.4.1.2-1: IWF private call request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| Functional alias | O | The functional alias associated with the MCPTT ID of the calling party. |
| MCPTT ID (see NOTE 1) | O | The MCPTT ID of the called party |
| Functional alias (see NOTE 1) | O | The functional alias of the called party |
| Use floor control indication | M | This element indicates whether floor control will be used for the private call. |
| SDP offer | M | Media parameters of MCPTT client. |
| Encryption Algorithm | O | Encryption algorithm to use for the call. The field can also indicate whether the encryption algorithm choice is determined from information in the media stream. |
| Encryption mode | M | Whether E2EE will be used. |
| Requested commencement mode | O | An indication of the commencement mode to be used. |
| Implicit floor request (see NOTE 2) | O | An indication that the user is also requesting the floor. |
| Location | O | Location of the calling party |
| Requested priority | O | Application priority level requested for this call. |
| Transfer indicator | O | Indicates that the MCPTT private call request is a result of a call transfer (true/false). |
| Forwarding indicator | O | Indicates that the MCPTT private call request is a result of a call forwarding (true/false) |
| Remotely initiated call request indicator | O | Indicates that the MCPTT private call request is a result of receiving of a remotely initiated call request and may be included only for remotely initiated call. |
| Additional application specific data (see NOTE 3) | O | Some LMR systems use addional information at the application layer. |
| NOTE 1: At least one identity shall be present. If both elements are present, the MCPTT ID is used to route the call. NOTE 2: This element shall be included only when the originating client requests the floor. NOTE 3: This element can be present if the LMR system uses it (like GSM-R). GSM-R uses for example UUI as defined in [10] ETSI TS 103 389 and [11] ETSI TS 102 610. |  |  |

* * * Next Change * * * *

#### 10.4.1.3	IWF private call response

Table 10.4.1.3-1 describes the information flow IWF private call response from the MCPTT server to the IWF and from the IWF to the MCPTT server.

Table 10.4.1.3-1: IWF private call response information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ID | O | The MCPTT ID of the called party |
| Acceptance confirmation | O | An indication whether the user has positively accepted the call. |
| SDP answer | M | Media parameters selected |
| Result | M | Result of the IWF private call request: success or failure |
| Encryption Algorithm(s) response | O | A list of one or more alternative encryption algorithm(s) to use for the call. |
| Use floor control indication response | O | This element indicates whether the floor control indication in the request is acceptable. |
| Implicit floor request response | O | This element indicates whether the indication that the user is also requesting the floor in the request is acceptable. |
| Additional application specific data (see NOTE) | O | Some LMR systems use addional information at the application layer. |
| NOTE: This element can be present if the LMR system uses it (like GSM-R). GSM-R uses for example UUI as defined in [10] ETSI TS 103 389 and [11] ETSI TS 102 610. |  |  |

* * * Next Change * * * *

#### 10.4.1.5	IWF call end request

Table 10.4.1.5-1 describes the information flow IWF call end request from the MCPTT server to the IWF and from the IWF to the MCPTT server.

Table 10.4.1.5-1: IWF call end request information elements

| Information Element | Status | Description |
| --- | --- | --- |
| MCPTT ID | M | The MCPTT ID of the calling party |
| MCPTT ID | M | The MCPTT ID of the called party |
| Additional application specific data (see NOTE) | O | Some LMR systems use addional information at the application layer. |
| NOTE: This element can be present if the LMR system uses it (like GSM-R). GSM-R uses for example UUI as defined in [10] ETSI TS 103 389 and [11] ETSI TS 102 610. |  |  |

* * * Next Change * * * *